3GPP TSG-SA WG6 Meeting #70	S6-255256

Dallas, USA, 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on converaged solution to KI#1

Spec:	3GPP TR 23.700-52 0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR propose a converage solution for KI#1.

2. Reason for Change

Solution #2, #3, #4 have some common ideas, i.e., reusing the NRM-UU interface. The only different part is how to handle the interactions with NRM server.

Based on the agreed S6-254785/CR 0165, both the stardarded interactions(Solution#4) or the internal interactions (Solution#2, Solution#3) should be supported.

Currently in most procedures, the SEALDD could directly interact with 3GPP network for QoS. The in-direct interaction via the NRM is only captured in the step descriptions as a alternative. This solution is proposed to follow the same drafting rule.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.4.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x | X |  |  |  |
| Sol #2 |  |  |  |  |

6.x	Solution #x: Converged broadcast/multicast data delivery solution

6.x.1	Solution description

6.x.1.1	General

This solution is addressing key issue #1. It is the converged solution based solution #2, #3,  #4.

The following principles are captured in this solution:

-	using the NRM-UU between the server side and the client side.

-	providing a single service for both broadcast and multicast.

-	Support the SEALDD to interacti with the 3GPP network directly or indirectly via NRM.

6.x.1.2	Broadcast/Multicast Data delivery service

6.x.1.2.1	Broadcast/Multicast data delivery service create

Figure 6.x.1.2.1-1 illustrates the high-level procedure of creating a broadcast\multicast data delivery service for the VAL server DL data delivery.Pre-conditions:

Figure 6.x.1.2.1-1: Broadcast/multicast delivery service create

- The VAL server invokes the Sdd_BroadcastMulticastDelivery create service to distribute its DL content to its recipients within the service area or a certain group. The data delivery requirements e.g., service area, start and end time of the delivery, QoS requirements are included, broadcast or multicast indication. The endpoint of a VAL server which ingresses the DL data may be also provided.

NOTE 1：	This VAL server ingresses the DL data can be a different VAL server than the one in step 1.

2.	The SEALDD server determines to create a broadcast or multicast flow. The SEALDD server allocates the SEALDD flow ID for the broadcast/multicast, the source multicast address and port used for DL data delivery to the recipients.

3.	The SEALDD server triggers the initiation of broadcast/multicast network resource in two alternative ways:

a.	The SEALDD server requests the eMBMS bearer as described in 3GPP TS 23.468 [23.468] and 3GPP TS 23.247 [23.247], and/or create the broadcast MBS session as described in 3GPP TS 23.247 [23.247] for this data delivery service and obtains the TMGI(s) from 3GPP network.

b.	The SEALDD server may interact with NRM server to request the eMBMS bearer or broadcast MBS session if the NRM server is available.

NOTE 2:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [23.434] or can be implementation specific when the NRM function is implemented as part of SEALDD function.

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD also allocates a data delivery service ID which is used to update and delete the created data delivery service. The SEALDD server returns the Sdd_BroadcastMulticastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server provision the service information including the SEALDD server address, data delivery service ID to the VAL client.

6. The VAL client sends the service request to the SEALDD client. The SEALDD server address, data delivery service ID are included.

7.	The SEALDD client sends the connection request (data delivery service ID) to the SEALDD server.

8.	The SEALDD server determines and returns broadcast/multicast flow ID used for the DL data delivery based on the data delivery service ID.

9.	The NRM server sends the MBS service announcement with TMGI of the MBS session and or eMBMS bearer to the NRM client via the NRM-UU as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [23.434].

NOTE 3:	The step 9 may be sent after step 4.

10.	The NRM client stores the announced information.

For multicast MBS session, step 11 is performed and for broadcast step 12 is performed.

11.	The UE joins the announced multicast MBS session (11a). And the NRM client sends the UE session join notification to the NRM server.

12.	The UE starts to lisening the broadcast channel as inticated by the TMGI to receive the DL data as described in 3GPP TS 23.247 [23.247].

13.	The NRM client sends the MBS listening status report to the NRM server via the NRM-UU as as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [23.434]. This message may be sent multiple time due the MBS reception quality changes.

14.	The VAL server starts to send DL data to the allocated ingress address. The SEALDD server encapsulates the received DL data within the broadcast/multicast flow packet with the source multicast address and further delivers the encapsulated packet(s) via the established eMBMS bearer and/or the MBS session.

NOTE 3:	The VAL server can start to send DL data to the SEALDD server after step 5. And the SEALDD server can start to send DL data to the network even before step 7. Then the UE can immediately receive the data after step 9.

6.x.1.2.2	Broadcast/multicast Data delivery service update

Figure 6.x.1.2.2-1 illustrates the high-level procedure of updating a broadcast/multicast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast/multicast delivery service and obtained the its data delivery service ID as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.2-1: Broadcast/multicast delivery service update

1.	The VAL server invokes the Sdd_broadcastMulticast data delivery update service. The data delivery service ID and updated parameters, e.g., service area or QoS requirements are included.

2.	The SEALDD server triggers the update of the 5MBS session in two alternatives:

a.	The SEALDD server requests the eMBMS bearer update and/or the broadcast/multicast MBS session update directly as described in 3GPP TS 23.468 [23.468] and 3GPP TS 23.247 [23.247]. If the service areas are updated and new broadcast/multicast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server.

b.	The SEALDD server may interact with NRM server to update the eMBMS bearer or broadcast/multicast MBS session. Updated information may be sent to the NRM client by the NRM server.

NOTE 1:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [23.434] or can be implementation specific when the NRM function is implemented as part of SEALDD function.

3.	The SEALDD server returns the Sdd_BroadcastMulticastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated eMBMS bearer and/or the MBS session.

6.x.1.2.3	Data delivery service delete

Figure 6.x.1.2.3-1 illustrates the high-level procedure of deleting a data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast/multicast data delivery service and obtained the its data delivery service ID as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.3-1: Broadcast/multicast delivery service delete

1.	The VAL server no longer requires the broadcast/multicast delivery service. It invokes the Sdd_BroadcastMulticast data delivery delete service. The data delivery service ID is included.

2.	The SEALDD server triggers the deletion of broadcast/multicast network resource in two alternative ways:

a.	The SEALDD server performs the eMBMS bearer delete and/or the MBS session release accordingly and interacts with the SEALDD client as described in 3GPP TS 23.468 [23.468] and 3GPP TS 23.247 [23.247].

b.	The SEALDD server may interact with NRM server to delete the eMBMS bearer or MBS session.

Disannouncement is sent to NRM client from the NRM server.

NOTE 1:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [23.434] or can be implementation specific when the NRM function is implemented as part of SEALDD function.

4.	The SEALDD server returns the Sdd_BroadcastMulticastDelivery delete response indicating the success of deletion.

6.x.2	Architecture Impacts

There are no architecture impacts.

6.x.3	Corresponding services

This solution introduces a new broadcast/multicast SEALDD data delivery service.

6.x.4	Solution evaluation

This solution provides a new data delivery service, i.e., broadcast/multicast data delivery service for enable the VAL server only sends one copy of data to the SEALDD server, and the SEALDD server send it to all the recipients  of this VAL service within the service area or a certain group. During the service area update, the VAL server is not bothered by creating new MBS sessions with additional ingress addresses in some cases.