3GPP TSG-SA WG6 Meeting #70	S6-255275

Dallas, USA 17th – 21st November 2025	(revision of S6-254xxx)

Source:	Ericsson

Title:	Enhancements to XR data transmission to enable foveated streaming

Spec:	3GPP TR 23.700-51 v1.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This paper provides a solution for Key Issue #1 and KI#3.

2. Reason for Change

The XR use cases require high-resolution and low-latency data delivery. To provide an improved user experience and avoid motion sickness, the use cases demand 4K, 8K video delivery, frame rate of 120Hz. This leads to large bandwidth and low latency requirements in the network. Also, it impacts the device side due to heavy processing and high battery usage, since the device is constrained with limited CPU, GPU or rendering capabilities, battery capacity. Moreover, the human eye can perceive the details only in a small region (the fovea, ~2° of visual angle) and transmitting the full-resolution frames for the entire view could lead to wastage of network resources and intensive workload on the device side. The proposed solution enables the consumers to request the foveated-based transmission to transmit high-quality frames for the user’s view based on eye gaze telemetry information and rest all in relatively low resolution to maintain the overall user experience without using a significant amount of network and device resources.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v1.1.0.

* * * First Change * * * *

##### 9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal ata transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, Foveated rendering transmission request, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, Eye gaze tracking capability indication, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. If the VAL server request for Foveated rendering transmission and VAL UE indicates support for Eye gaze tracking capability indication, the response includes protocol description and RTP header extension for XR pose to the SEALDD client for later use to send the XR pose information in RTP HE over SEALDD-UU traffic. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE : If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). The SEALDD client performs the packetization for RTP packet to add RTP header extension for XR pose information defined in clause 4.3 TS 26.522 and send to the SEALDD server. Based on the received XR pose information, SEALDD server determines and performs adaptive encoding(e.g., transcoding) for the regions of the Field of view for the foveated rendering of the XR traffic over SEALDD-UU. The SEALDD server also performs the packetization to add RTP header extension for the SEALDD client to include the rendered pose information. #### 9.12.3.1 SEALDD enabled multi-modal transmission request Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service. Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of specific UE or VAL user. List of requested flows; M; List of the requested multi-modal flows. >SEALDD-S Data transmission connection information for multi-modal XR; M; Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow). >QoS information; O; QoS information provided by VAL server. >Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. Foveated streaming transmission request; O; Indicates the request to use foveated rendering or streaming for XR data delivery. #### 9.12.3.3 SEALDD multi-modal transmission connection establishment request Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal transmission connection establishment. Table 9.12.3.3-1: SEALDD multi-modal transmission connection establishment request Information element; Status; Description. SEALDD client ID; M; Identity of the SEALDD client.. Identity; O; The VAL user ID of the VAL user or VAL UE ID. SEALDD-UU flows info; M; Information of the list of SEALDD-UU flows.. > SEALDD-UU flow ID; M; Identity of the SEALDD-UU flow.. > SEALDD traffic descriptor; M; SEALDD traffic descriptor for the flow of the SEALDD client side used to establish SEALDD multi-modal XR connection.. >> Addressing info; M; Indicates the addressing information (e.g. address, port, URL).. >> Transport layer protocol; M; Indicates the transport layer protocol.. VAL server ID; O; Identity of the VAL server.. VAL service ID; O; Identity of the VAL service.. Eye Gaze tracking capability indication; O; Indicates the support for eye gaze tracking capability for foveated streaming or rendering.

* * * End of Changes * * *