3GPP TSG-SA WG6 Meeting #70	S6-255278

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Samsung

Title:	Pseudo-CR on Updates to Solution #6

Spec:	3GPP TR 23.700-43 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	Narendranath Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR updates the Solution #6 to address the open issue (whether and how the analytics consumer’s preference to the usage of service API information) in Key Issue #2. Also, addresses the resolution of the EN (Editor's Note:	Performing analytics inside or outside of CCF (e.g. ADAES) is FFS and outside is based on data available with CCF (i.e. CCF being data source) is FFS).

2. Reason for Change

The key issue#2 captures new open issues to handle the analytics consumer’s preference to the usage of service API information. This pCR proposes update to Solution #6 with inclusion of preference information in the Query Service API Analytics request message. Also, the EN (Editor's Note:	Performing analytics inside or outside of CCF (e.g. ADAES) is FFS and outside is based on data available with CCF (i.e. CCF being data source) is FFS) is resolved with clarification that when AMF prefers to receive analytics based on service API information from multiple API providers, then the solution #6 is used where CCF has service API information from multiple API providers, else AMF may rely on other mechanisms.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 v0.2.0.

* * * First Change * * * *

### 6.7.1	Solution description

Figure 6.7.1-1 illustrates the procedure for support of API provider administrator in CAPIF.

Pre-conditions:

1.	Authorization details of the API analytics consumer e.g. AMF are available with the CAPIF core function.

Figure 6.7.1-1: Procedure for support of API provider administrator in CAPIF

1.	For querying service API analytics, the API analytics consumer e.g. AMF triggers Query service API analytics request to the CAPIF core function. Table 6.7.1-1 describes the information flow Query service API analytics request from the API analytics consumer to the CAPIF core function, including Preference Information.

Table 6.7.1-1: Query service API analytics request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | Identity information of the entity querying service API analytics request |
| Analytics Query information | M | List of query filters related to the required analytics, such as Usage Metrics, Error Tracking, User pattern, Security Monitoring, Performance Optimization, Compliance and Auditing |
| Preference Information | M | List of preferences related to the usage of set of API provider domain’s service API information (e.g., service API invocation logs). Each element of this list consists of preference (allowed or not allowed) for a given set of service APIs and API provider domains (e.g., API provider domain functions of other API providers, Application Functions, API provider name, etc) |
| >preference value | M | Preference issued: “Allowed”, “Not Allowed”. |
| >target identifiers | O | List of identifiers (e.g., API provider domain functions, API provider name, etc ) of the targets for which the “preference value” applies. When this information is not present or includes a specific value (e.g., “All” or “Any” or “Allow All”, etc), then the “preference value” is applicable for all API provider entities, e.g., all API provider domains, all API provider domain functions, all API provider names, etc. |
| >Service APIs Information List | O | List of service APIs details (service API information, service API invocation log details) for which the “preference value” (“Allowed” or “Not Allowed”) is applicable on the “target identifiers”. Each element of the list consists of: service API name, API provider name, service API category (e.g. V2X, IoT), AEF location, interface details (e.g. IP address, port number, URI), protocols, version numbers, service API operation(s) and resource(s), Network Slice Info, security methods, input parameter information, invocation result, time stamp information. When this information is not present, then it indicates that the “preference value” on “target identifiers” is applicable for all service APIs (e.g., service API invocation logs) belonging to the API provider entities / API provider domains in “target identifiers”. |

2.	Upon receiving the query service API analytics request, the CAPIF core function accesses and/or generates the necessary analytics information. If Preference Information received in step 1, then the CCF generates the requested analytics using the service API information (e.g., service API invocation logs) from multiple API provider domains, as indicated in the Preference Information.

3.	The CAPIF core function returns the API analytics information to the API analytics consumer in the query service API analytics response. Table 6.7.1-2 describes the information flow Query service API analytics response from the CAPIF core function to the API management function.

Table 6.7.1-2: Query service API analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of query service API analytics request |
| API invocation analytics information | O (see NOTE) | API analytics information such as Usage Metrics (Requests per Second, Total Requests, Response Times), Error Tracking (Error Rates, Error Types), User pattern (Most Used Endpoints, User Segmentation, User feedback), Security Monitoring (Unauthorized Access Attempts, Rate Limiting Violations, Anomaly detection), Performance Optimization (Latency Analysis, Throughput Analysis), Compliance and Auditing (Data Usage Reports, Audit Logs) |
| NOTE: Information element shall be present when result indicates success. |  |  |

* * * Next Changes * * * *

### 6.7.2	Architecture Impacts

Supporting API analytics functionality is required.

The API analytics function has the following capabilities:

1.	Requests per Second (RPS), Total Requests, Response Times

2.	Error Rates, Error Types

3.	Most Used Endpoints, User Segmentation, User feedback

4.	Unauthorized Access Attempts, Rate Limiting Violations, Anomaly detection

5.	Latency Analysis, Throughput Analysis

6.	Data Usage Reports, Audit Logs

If the analytics functionality is internal to the CAPIF core function, the CAPIF-5 reference point as specified in 3GPP TS 23.222, between the AMF and the CAPIF core function is used for AMF as an API analytics consumer fetching API analytics from the CCF.

When the API provider is fine with the usage of service API information (e.g., service API invocation logs) belonging to multiple API provider domains, then the AMF includes the preference information in Query Service API Analytics request message. Else, the AMF may generate the API analytics based on service API information (e.g., service API invocation logs) available locally or may fetch the service API information (e.g., service API invocation logs) from CCF using Auditing service API invocation procedure as specified in clause 8.22 of 3GPP TS 23.222.

* * * End of Changes * * * *