3GPP TSG-SA WG6 Meeting #70	S6-255279

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Samsung

Title:	Pseudo-CR on New solution to Key Issue #3

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Narendranath Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR proposes new solution to Key Issue #3, addressing open issue (“Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes”).

2. Reason for Change

This pCR proposes new solution to addresses key issue # 3, open issue #6 (“Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes”). The trigger or reason for AIMLE context transfer is included in AIMLE context transfer request message. This information can be used by the target AIMLE server to determine the future AIMLE context transfers. The proposed solution is an enhancement to the existing AIMLE context transfer prodedure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |
| Sol #8 |  |  |  | X |  |  |  |  |
| Sol #9 |  |  |  |  |  | X |  |  |
| Sol #10 |  |  | X |  |  |  |  |  |
| Sol #11 |  |  | X |  |  |  |  |  |
| Sol #12 |  |  |  | X |  |  |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  | X |  |
| Sol #16 |  |  | X |  |  |  |  |  |
| Sol #17 | X |  |  |  |  |  |  |  |
| Sol #18 |  |  | X |  |  |  |  |  |
| Sol #19 |  |  | X |  |  |  |  |  |
| Sol #20 |  | X |  |  |  |  |  |  |
| Sol #21 |  |  |  |  |  | X |  |  |
| Sol #22 |  |  |  |  |  |  | X |  |
| Sol #23 |  |  |  |  |  |  |  | X |
| Sol #24 |  |  |  |  |  | X |  |  |
| Sol #25 |  |  |  |  | X |  |  |  |
| Sol #26 |  |  |  |  |  | X |  |  |
| Sol #X |  |  | X |  |  |  |  |  |

* * * Next Change * * * *

## 7.x 	Solution #x: Trigger for AIMLE context transfer

### 7.x.1 	Solution Description

#### 7.x.1.1 	General

This solution addresses key issue # 3, open issue #6 (“Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes”). This solution proposes to include the trigger or reason for AIMLE context transfer information in AIMLE context transfer request message. This information can be used by the target AIMLE server to determine the future AIMLE context transfers.The proposed solution is an enhancement to the existing AIMLE context transfer prodedure.

#### 7.x.1.2 	Procedure

This solution updates the AIMLE context transfer procedure specified in clause 8.24, 3GPP TS 23.482 [2] with trigger information. The updates to procedure is shown below (in bold).

## 8.24 AIMLE context transfer ### 8.24.1 General This clause describes AIMLE context transfer procedure between AIMLE servers (over AIML-E reference point). ### 8.24.2 Procedure Pre-conditions: 1. Each edge AIMLE server manages the AIMLE clients within its service area to perform AI/ML operations. 2. A UE associated with an AIMLE client moves from a source service area (managed by a source edge AIMLE server) to a target service area (managed by a target edge AIMLE server). The transition triggers application context relocation (ACR) procedure between the two edge AIMLE servers (as source EAS and target EAS, respectively) as specified in 3GPP TS 23.558 [12]. Figure 8.24.2-1. AIMLE context transfer 1. The source edge AIMLE server sends an AIMLE context transfer request to the target AIMLE server as described in Table 8.24.3.1-1. The request includes AIMLE context information which is generated based on the responses/notifications received from the transitioned AIMLE client (e.g. step 7 of clause 8.12.2 or step 3 of clause 8.20.1). The AIMLE context information is used by the target edge AIMLE server to determine whether the information (e.g. AI/ML operation output/result) received from the AIMLE client should be transferred to the source edge AIMLE server (or another edge AIMLE server that has been associated with the AIMLE client). For example, the target AIMLE server can forward the AIMLE service results received from the AIMLE client to the source AIMLE server if the results are only applicable to the source service area or the AIMLE client is part of a split operation pipeline formed in the source service area. If the AIMLE context transfer request message includes trigger or reason for context transfer, then the target edge AIMLE server may use the trigger or reason information in determining the future AIMLE context transfers. 2. The target edge AIMLE server sends an AIMLE context transfer response to the source edge AIMLE server as described in Table 8.24.3.2-1.

#### 7.x.1.3	Information Flows

The Source Edge AIMLE server includes the trigger information in the AIMLE context transfer request as shown below (updates in bold)

#### 8.24.3.1 AIMLE context transfer request Table 8.24.3.1-1 shows the AIMLE context transfer request that is sent by a source AIMLE server to a target AIMLE server. Table 8.24.3.1-1: AIMLE context transfer request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor (e.g., AIMLE server).. AIMLE Context Information; M; The AIMLE context information as described in Table 8.24.3.1-2.. Trigger for context transfer; O; Trigger or the reason for the AIMLE context transfer (e.g., mobility of UE associated with the AIMLE client, availability status of source AIMLE server, load conditions on source AIMLE server, change in ML task requirement, etc).. Table 8.24.3.1-2: AIMLE context information Information element; Status; Description. AIMLE client ID; M; The identifier of the AIMLE client associated with the context.. Current managing AIMLE server; O; The identifier of the AIMLE server that is currently managing the AIMLE client, i.e. the AIMLE server associated with the service area that the AIMLE client is currently in.. Previous managing AIMLE server; O; List of identifiers of AIMLE servers that have been associated with the AIMLE client. The list is populated by adding the identifier of the source edge AIMLE server whenever the UE transitioned from a source edge area to a target edge area.. AIMLE service status; O; Status of the AIML operations (task) at the AIMLE client, e.g. “active”, “paused”, “completed”, percentage of completion.. AIMLE service results; O; Results of the AIML operations (task) performed by the AIMLE client.. AIMLE service applicability; O; Applicability information of the AIML operations performed by the AIMLE client, e.g. the operation results are applicable within a certain edge service area, the operations are applicable within a certain split operation pipeline.. >ML context information; O; Context information related to the ML operation that the AIMLE client is participating in or performing.. >> VAL service Information; O; Information related to the VAL service for which the AIMLE task is performed (e.g., the VAL service identifier for the AIMLE HFL training operation).. >> ML task; O; Type of ML task (model training, model testing, model inference, model transfer, model offload, model split, intermediate AI/ML operation/task) to be continued at the target AIMLE server.. >> ML task information; O; Information related to the ML task mentioned in “ML task” information element. The Model Training task Information may include training objective to be achieved, HFL training information, VFL training information, data set information for training, status of training operation at AIMLE client (e.g. “active”, “paused”, “completed”), training results, etc. The Model Inference task information may include Inference results, inference job id, etc. The Model split task information may include split operation profile as specified in table 8.14.3.3-2.. >> ML model information; M; Model information related to the ML task. This information may include, the model identifier, Information to fetch ML model information, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides, Model parameters from ML training, etc).

### 7.x.2 	Architecture Impacts

There are no new reference points and functional entities proposed in this solution. This solution has no architecture impacts.

### 7.x.3	 Corresponding APIs

This solution updates the existing AIMLE context transfer API as specified in clause 9.2.16 of 3GPP TS 23.482 [2].

### 7.x.4	 Solution Evaluation

This solution addresses the open issue #6 in Key issue#3. This solution enables the Target Edge AIMLE server receiving the AIMLE context information with enhanced information on the trigger for transferring the AIMLE context. This trigger information may enable the Target Edge AIMLE to use for determinaing further AIMLE context transfers.

* * * Next Change * * * *

### 10.1.1	Summary of new AIMLE services

This clause provides a summary of the new AIMLE and ML repository capabilities based on the Solutions.

Table 10.1.1-1 is listing solutions proposing new capabilities related to AIMLE:

Table 10.1.1-1: Solutions proposing new or enhanced AIMLE capabilities

| Solution | Proposed Capabilities | New or Enhancement |
| --- | --- | --- |
| #1 | ML model inference | New |
| #2 | AIMLE data management assistance | Enhancement |
| #3 | ML Model Maintenance | New |
| #4 | ML Model Evaluation | New |
| #6 | ML model Perf monitoring | Enhancement |
| #7 | AIMLE client discovery in cross-PLMN scenarios | Enhancement |
| #8 | Multi-client split operation pipeline | Enhancement / New |
| #9 | AIMLE server registration in hierarchical AIMLE deployment | New |
| #10 | Support of AIMLE services for roaming UEs | Enhancement |
| #12 | ML model split learning using relays | New and Enhancement |
| #13 | Edge ML model training support | New |
| #14 | AIMLE Client Discovery | Enhancement |
| #15 | Sample alignment for VFL | Enhancement |
| #16 | AIMLE service migration support in UE roaming scenarios | New |
| #17 | ML model collaborative inference | New |
| #18 | Support for federated AI services | New |
| #19 | Cross-PLMN/Domain AIMLE client discovery, selection, monitoring | New |
| #20 | ML Model Performance Evaluation with information collection from ML model consumer | New |
| #21 | Split operation node registration involving multiple AIMLE clients | Enhancement |
| #22 | Sample Alignment Enablement for VAL Servers in Vertical Federated Learning | New |
| #X | AIMLE context transfer with trigger information | Enhancement |

Table 10.1.1-2 is listing the solutions proposing new ML repository capabilities:

Table 10.1.1-2: Solutions proposing new or enhanced ML repository capabilities

| Solution | Proposed Capabilities | New or Enhanced |
| --- | --- | --- |
| #11 | FL member registration in multi-operator scenarios | Enhanced |
| #12 | ML model split learning using relays | Enhancement |
| #18 | Support for federated AI services | Enhancement |

* * * End of Changes * * * *