3GPP TSG-SA WG6 Meeting #70	S6-255287

Dallas, USA 17th – 21st November 2025	(revision of S6-255xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for Editor’s Notes in KI#5

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for the Editor’s Notes in KI#5 for TR 23.700-43.

2. Reason for Change

KI#5 describes one existing gap in current deployments, about how to avoid in a controlled way that a service API call is served when e.g. the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network.

In the definition of the Open Issues for KI#5 the following Editor’s Notes remain:

Editor's Note:	Whether and how handle local access to service APIs in the VPLMN is FFS.

Editor's Note:	Core network roaming aspects in this key issue needs to be updated with the application roaming aspects.

For the first Editor’s Note, it is proposed to transform it into a NOTE to indicate that aspects of local access to service APIs in the VPLMN are not included in this version of the specification, since in most of the roaming solutions are based on Home Routed scenarios instead of LBO.

For the second Editor’s Note, it is also proposed to turn it into a NOTE to indicate that aspects of application roaming are not included in this version of the specification. The solution in this release can be focused in generally available applications and in established industry procedures for accessing an application while roaming in a visited country, i.e., based on application distributed deployment, with regionally deployed servers and centralized servers, and regional server selection based e.g. on Anycast and the location nearest to the sender.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0)

* * * First Change * * * *

### 4.5.2	Open issues

To support the control of service API invocations when the UE is roaming, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations, based on:

-	whether roaming agreements and policies between CSPs needed for exposure; and

-	whether application preferences related to roaming of the UE.

NOTE 1:	This KI ensures to keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc.).

NOTE 2:	Aspects of local access to service APIs in the VPLMN are not included in this present document. This issue focuses on Home Routed Scenarios.

NOTE 3:	Aspects of application roaming are not included in this present document.

* * * End of Change * * * *

<Proposed change in revision marks>