3GPP TSG-SA WG6 Meeting #70	S6-255292

Dallas, USA 17th – 21st November 2025	(revision of S6-255xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#3 on Certificate Unavailability

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#3 for TR 23.700-43.

2. Reason for Change

As described in KI#3, a pre-requisite to onboarding an API invoker to CAPIF, the API invoker needs to have previously obtained onboarding credentials. As result of API invoker onboarding, the API invoker receives digital credentials for subsequent accesses to the CAPIF entities. API providers are also assigned signed certificates when registering to the CCF. If the certificate becomes invalid or is lost, the API Invoker and API Provider can no longer manage their resources.

This solution proposes enhancements to the CAPIF framework to enable that the CAPIF Administrator can become aware of an invalid certificate, so that operation, if required, can continue and such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.

.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |
| Sol #X |  |  | X |  |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #X: CAPIF Administrator awareness of certificates validity

### 6.x.1	General

This solution relates to KI #3.

As described in KI#3, a pre-requisite to onboarding an API invoker to CAPIF, the API invoker needs to have previously obtained onboarding credentials. As result of API invoker onboarding, the API invoker receives digital credentials for subsequent accesses to the CAPIF entities. API providers are also assigned signed certificates when registering to the CCF. If the certificate becomes invalid or is lost, the API Invoker and API Provider can no longer manage their resources.

This solution proposes enhancements to the CAPIF framework to enable that the CCF become aware of an invalid certificate, so that operation, if required, can continue and obsolete information is removed from the CCF.

### 6.x.2	Architecture Impacts

This solution does not have any architectural impacts.

### 6.x.3	Solution description

#### 6.x.3.0	General

##### 6.x.3.0.1	Introduction

3GPP TS 33.122 [x] specifies two different mechanisms for delivering certificates to the API invoker:

1.	During API invoker onboarding, the CAPIF Core Function delivers a certificate and delivers it to the API invoker in the onboarding response (figure 6.1-1, step 4).

2.	During API invoker, the CAPIF Core Function receives an API Invoker certificate during the Onboarding request and doesn’t need to create a new certificate and return it in the Onboarding response. This alternative is covered by NOTE 2 of clause 6.1.

In the second case, for the API Invoker certificate, as well as for the AEF(s), APF(s) and AMF(s) certificates, it can be considered that the renewal of certificates is done in collaboration with the certificate management solution bound to the certificate issuer whose realization is out of 3GPP and hence out of CAPIF. How the CAPIF Administrator becomes aware of an invalid certificate and the required removal of the entity associated with the certificate is out of the scope of CAPIF specifications.

For the first case, the CAPIF framework may require enhancements to support some specific aspects of e.g., certificate renewal. The CAPIF impacts in this case would be out of SA6 scope and fall under SA3 responsibility. Under SA6 remit, as in the previous case, it can be considered that the CAPIF Administrator becomes aware of the need to remove an entity associated with an invalid certificate.

#### 6.x.3.0.2	Removing CAPIF entities without a valid certificate

This solution assumes that the CAPIF administrator can leverage in an existing certificate management solution that is able to inform about the entities affected by invalid certificates, so that the CAPIF administrator is able to take further actions on the information kept for the affected API Invoker(s) (e.g. offboard API invoker), APF(s), AMF(s) and AEF(s) (deregistering them from the CCF).

#### 6.x.3.1	Information flows

Editor's note:	The impact on the information flows due to the definition of CAPIF certificate renewal procedures depends on SA3 progress.

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

* * * End of Change * * * *

<Proposed change in revision marks>