3GPP TSG-SA WG6 Meeting #70	S6-255293

Dallas, USA, 17th – 21th November 2025	(revision of S6-255xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0181 | rev | - | Current version: | 19.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to misalignment between Packetization IE and related procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S6-253286 updated the “protocol description” sent from the SEALDD server to the SEALDD client and included: However, clause 9.12.2.1.2 specifies: This misalignment needs to be corrected, and the description of the Packetization indication needs to be updated to describe “The Packetization indication indicates whether packetization is needed or not, e.g. RTP packetization”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the Packetization Indication IE to indicate whether packetization is needed or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The packetization of the streamed multimodal flow is not indicated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.12.3.4	SEALDD multi-modal transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the multi-modal transmission connection establishment.

Table 9.12.3.4-1: SEALDD multi-modal transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow info | M | Information of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| > Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal service connection. |
| > SEALDD-UU flows list | M | Indicates the list of SEALDD-UU flows. |
| >> SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| >> SEALDD traffic descriptors | M | SEALDD traffic descriptors for the flows of the SEALDD server side used to establish SEALDD multi-modal connection. |
| >>> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >>> Transport layer protocol | M | Indicates the transport layer protocol. |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension for PDU set), packetization indication, payload type and format. |
| >Header extension information | O | The header extension information includes the RTP header extension type and RTP header extension identity. |
| >Packetization indication | O | Packetization indication indicates whether the packetization is needed or not for the Multi-modal SEALDD flow. |
| >Payload type | O | Indicates the type of the payload (e.g., RTP, SRTP). Header extension information is only applicable when payload type indicates RTP. |
| >Payload format | M | Indicates the format of the payload (e.g., H.264, H.265). |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * End of Change * * * *