3GPP TSG-SA WG6 Meeting #70	S6-255432

Dallas, United States 17th – 21st November 2025	(revision of S6-255192)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0097 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding reference point for spatial map localization for objects and Spatial anchors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Consider a scenario – where an object is on anotehr moving object, like while creating spatial map of a city – a person inside a moving train. To localize a person or to add anchor on a moving traing, it is required to provide reference location compared to the base / moving object. Without reference point – localization of an object on another moving object is not possible. SA6 agreed to have spatial anchors for mobile object witin spatial map. Further, the localization of the object within spatial map currently provides 3D posision. In order to display the spatial anchor or object wihtin spatial map, a reference point (or point of origin) is required and its refenfence can be used to display the spatial anchor or object. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a reference point for the spatial map and spatial anchor (when added within spatial map). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The localization and tracting of the object will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.3.1, 8.3.4.3.1, 9.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE 1) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE 1) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format) or object ID. |
| >>> Spatial map ID | O | Identifier of the spatial map |
| >>>> Spatial map layer ID | O | Identifier of the spatial map layer if Spatial map ID is present |
| >>>> Reference point | O | Reference point from where the spatial anchor is placed in the spatial map |
| >>>> Localization (NOTE 2) | O | 3D position within spatial map and pose of the VAL UE |
| >>>> Object ID (NOTE 2) | O | Identifier of the object on which the spatial anchor will be created |
| > Application specific Spatial anchor information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| > Associated name | O | Name of the product/service associated with the spatial anchor |
| > Time period | O | Indicates time period or duration or time windows – comprising a start time and an expiry time. A spatial anchor is active during this time period. After the expiry time, the state of a spatial anchor becomes inactive. The time period may include one or more values (e.g. repeated over time/day). |
| > Expiry handling indication | O | Indicates whether the spatial anchor has to be deleted or its state to be updated as inactive on expiry by the SAn server. Absence of this IE shall be considered that SAn server deletes the spatial anchor on expiry. |
| > Spatial anchor resource identifier | M | The identifier to associate the resource information with the created spatial anchors. |
| > Spatial anchor group ID | O | Identifier of spatial anchor group that the spatial anchor associates with |
| NOTE 1: One of the IEs shall be present. NOTE 2: If Spatial map ID is present, either one of the IEs shall be present. |  |  |

* * * Next Change * * * *

##### 8.3.4.3.1	Spatial anchor discovery request

Table 8.3.4.3.1-1 describes information elements for the spatial anchor discovery request from the SAn client or VAL server to the SAn server.

Table 8.3.4.3.1-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Location information | O | Indicates the location information of the VAL client when the requestor is a SAn client. |
| Discovery filters | O | The filters used for determining the spatial anchors. |
| > List of spatial anchor ID | O | The list of spatial anchor identifiers to discover. |
| > Area of interest | O | The area of interest (e.g., topological area and/or geographical area). |
| > Service ID | O | The identifier of a VAL service. |
| > Associated name | O | Name of the product/service associated with the spatial anchor |
| > Application specific Spatial anchor information | O | A container to contain application specific information. |
| > Reference point | O | Reference point from where the object is to discover |

* * * Next Changes * * * *

#### 9.4.3.1	Spatial localization service request

Table 9.4.3.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.4.3.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL User or VAL UE, VAL server) |
| Security credentials | M | The security credentials of the requestor. |
| Spatial Map list |  | List of spatial maps to be used for spatial localization |
| > Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target identities | O | List of identity of the users or objects to localize |
| List of UE-object association information | O | List of UE-object association information to identify associations between UEs and objects in a spatial map. |
| Reference point | O | Reference point from where the object is localized within spatial map |

* * * End of Changes * * * *