3GPP TSG-SA WG6 Meeting #70	S6-255516

Dallas, United States 17th – 21st November 2025	(revision of S6-255223)

Source:	China Mobile

Title:	Pseudo-CR on Solution of supporting virtual number in MMTel

Spec:	3GPP TR 23.700-57 v0.4.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of Open issue#1 of KI#5 on Support of virtual number.

This pCR proposes this new solution for TS23.700-57.

The capabilities used in this solution may be specified by SA2 in FS_NG_RTC_Ph3 (i.e. SP-250855) study under WT#2 as:

WT#2: Study on IMS and DC capability exposure enhancements

-	WT#2.1: How to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities;

In support of IMS capability exposure for eMMTel services (voice/video) as depicted above, study how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS, (e.g. subscriber registration event, or event of all UEs calling a particular customer support number and that have DC capabilities for customization of customer support to UE capabilities);

2. Reason for Change

This pCR provides the solution of Open issue#1 of KI#5 on Support of virtual number.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.4.0.

* * * First Change * * * *

6.X	Solution #X: Supporting virtual number in MMTel

Editor's note:	the solution has dependency on SA2 progress on IMS capability exposure framework to expose IMS capability for eMMTel services.