3GPP TSG-SA WG6 Meeting #70	S6-255584

Dallas, USA, 17th – 21th November 2025	(revision of S6-2532219)

Source:	Lenovo, Ericsson, InterDigital

Title:	Pseudo-CR on overall evaluation and conclusion of the Key Issue #2

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This pCR proposes the overall evaluation and conclusion of the Key Issue #2 of the solutions included on TR 23.700-83 v1.1.0.

2. Reason for Change

To include the evaluations and conclusion of the solutions that addresses the Key Issue #2 of TR 23.700-83 v1.1.0.

3. Conclusions

This pCR proposes the Key Issue #2 evaluation and conclusion of the solutions described on TR 23.700-83 v1.1.0.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

## 10.3	Key issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

The open issues studied in the Key Issue #2 are as follows:

1.	How to define ML model performance and ML model correctness in the context of the AIML enablement layer.

2.	Whether and how to evaluate ML model performance and correctness considering application specific ML models.

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

4.	Whether and how to trigger corrective actions on ML model performance and correctness degradation detection (e.g., model maintenance).

There is a total of six solutions that address different aspects of this key issue: solutions #2, #3, #4, #5, #6, and #20. Each of these solutions covers different factors of the Key Issue.

Solution #2 (Enhance AIMLE Data Management Assistance for Data Drift Detection) provides support for data drift detection capability, and it has been used by other solutions to solve the key issue #2. For instance, solution #3 (New AIMLE Service on ML Model Maintenance) can use data drift as one of the factors to be considered in determining ML model evaluation. Furthermore, in solution #5 (Solution on monitoring ML-analytics correctness), data drift can be one of the causes for mismatch detection in the ADAE analytics performance of the analytics task monitored by the discovered ADAEC(s) and/or edge ADAES. In solution #6 (Enhancement to performance monitoring), data drift is considered one of the expected models degradation types. Solution #4 (Management of evaluation information for model performance and correctness evaluation) provides the mechanism to store and fetch evaluation information to ML repository and can use ML model-specific evaluation for monitoring solutions related to data drift detection.

Solution #3 (New AIMLE Service on ML Model Maintenance) and solution #20 (ML Model Performance Evaluation with information collection from ML model consumer) both address the ML model maintenance task but with different approaches. Upon request from the AIMLE service consumer, solution #3 returns the ML model maintenance information by retrieving the ML model evaluation information using the provided ML Evaluation profile ID. In cases where the ML performance is not as expected, the AIMLE server determines that data drift detection should be used as one of the factors to consider for ML model evaluation (as mentioned in the previous paragraph). Additonally, Solution #20 deals with ML model evaluation performance, with the main difference being that this information is collected from the consumer (e.g., AIMLE client).

Solution #4 (Management of evaluation information for ML model performance and correctness evaluation) can be used to store ML evaluation information and can provide this functionality to all solutions that deal with checking ML performance.

Solution #5 (Solution on monitoring ML- analytics correctness) provides ADAE analytics performance monitoring capabilities to the AIMLE server. Differently from the previous solutions, solution #5 gets the performance monitoring data from ADAE clients and edge ADAES.

Solution #6 (Enhancements to performance monitoring) also focuses on performance monitoring but includes models running in a federated fashion.

Regarding the mapping of the above solutions to open issues, the solution doesn’t provide alternative, but complementary capabilities, in particular:

- Solution #2 addresses Open Issue 2

- Solution #3 addresses Open Issue 4

- Solution #4 addresses Open Issue 1, 3

- Solution #5 addresses Open Issue 3, 4

- Solution #6 addresses Open Issue 1, 3

- Solution #20 addresses Open Issue 2,3.

In each solution that addresses Key Issue #2, no AIMLE architectural impact was identified.

* * * Next of Change * * * *

## 11.2	Conclusions of key issue #2

Following solution considerations will be considered for the normative work for KI #2:

- Solution #2 can be considered for normative work to assist ML model maintenance and ML model performance monitoring.

- Solution #3 can be considered for normative work to support a new capability for ML model maintenance.

- Solution #4 can be considered for normative work to support storing and fetching evaluation info to the ML repository wih the support of AIMLE server.

- Solution #5 can be considered for normative work to support monitoring ML-enabled ADAE analytics correctness.

- Solution #6 can be considered for normative work to support enhancements to ML model performance monitoring capability.

- Solution #20 can be considered for normative work to support ML Model Performance Evaluation with information collection from ML model consumer.

* * * End of Change * * * *