3GPP TSG-SA WG6 Meeting #71	S6-260015

Goa, India 9th – 13rd February 2026	(revision of S6-26xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0263 | rev | - | Current version: | 20.2.0 |  |
| For HELP on usig this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Server-Server message (MCVideo) procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedures in clause 7.19.3.2 Ad hoc group call involving multiple MC systems describing already the information flows from server to server but not clearly. Describing the actual step more in detail in the figures and its belonging descriptions clarify the actual situation. Further Notes in clause 7.19.3.2 become obsolete and can be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying MCVideo server to server steps and figures in AHGC procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong figures may cause misunderstandings to the downstream groups and the readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 7.19.3.2	Ad hoc group call involving multiple MC systems

##### 7.19.3.2.1	Ad hoc group call setup – Participants list determined by the MCVideo server

Figure 7.19.3.2.1-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user wherein the list of participants is determined by the MCVideo server based on the criteria received from the MCVideo client and determined MCVideo users are from multiple MCVideo systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system share their users' information to the primary MC system.

2.	The authorized MCVideo user/dispatcher belongs to the primary MC system.

3.	The MCVideo server 1 of the primary MC system is where the authorized MCVideo user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group belong to MCVideo server 2 of the partner MC systems.

5.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group have been preconfigured in MCVideo client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

Figure 7.19.3.2.1-1: Ad hoc group call setup involving multiple MCVideo systems

1-3.	Same as described in subclause 7.19.3.1.3.

4.	The MCVideo server 1 determines the list of participants from the primary MC system and determines the partner MC system to be involved in the ad hoc group call based on the information present in the information element Criteria for determining the participants. This information element carries the criteria, indicator identifying pre-defined criteria, or a combination of both.

NOTE:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how the MCVideo server determines the list of participants are left to implementation.

5.	If the MCVideo server 1 needs to involve the partner system based on the agreement and based on the criteria for determining the participants list, it sends the ad hoc group call get userlist request to the MCVideo server 2. Depending on the criteria provided and based on local policy, the MCVideo server may modify the content of the criteria received in step 1 to determine the list of participants. This request carries the call resulting criteria to be used by the partner MC system and is equal or derived from the criteria received in the step 1.

6.	MCVideo server 2 evaluates the criteria and determines the participants satisfying the criteria (i.e. MCVideo client 3 and MCVideo client 4) and sends the response containing the list of MCVideo users satisfying the criteria. The partner MCVideo server may apply local policies if any while determining the participants satisfying the criteria.

7.	The MCVideo server 1 compiles the list of participants to be invited for the ad hoc group call including the participants from both primary and partner MC system.

8a-8b.	The MCVideo server 1 sends the ad hoc group call request towards the MCVideo client 3 and MCVideo client 4 via MCVideo server 2. While sending the ad hoc group call request, the MCVideo server shall remove the information elements that are not required to be conveyed to the target MCVideo clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCVideo server 1 considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

9.	The MCVideo server 1 sends the ad hoc group call requests towards the MCVideo client 2. While sending the ad hoc group call request, the MCVideo server shall remove the information elements that are not required to be conveyed to the target MCVideo clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCVideo server 1 considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

10a-10c.	The receiving MCVideo clients are notified about the incoming ad hoc group call.

11.	The MCVideo client 2 accept the ad hoc group call request and send ad hoc group call response to the MCVideo server 1.

12.	The MCVideo client 3 accepts the ad hoc group call request, and sends ad hoc group call response to the MCVideo server 1 via MCVideo server 2.

13.	The MCVideo client 4 accepts the ad hoc group call request, and sends ad hoc group call response to the MCVideo server 1 via MCVideo server 2.

14.	The MCVideo server 1 sends the ad hoc group call response to MCVideo client 1 through the signalling path to inform about successful call establishment. The ad hoc group call response contains the criteria used by the MCVideo server to determine the list of participants to be invited.

15.	The MCVideo server 1 may notify the initiating MCVideo user of all MCVideo users who acknowledged the ad hoc group call request and joined the ad hoc group call. The MCVideo server 1 more than once during the call may send this notification to the initiating MCVideo user whenever an MCVideo user joins or leaves the MCVideo ad hoc group call. If the authorized users (not shown in figure) are configured to receive the participants information of ad hoc group call, the MCVideo server provides ad hoc group call notify about all MCVideo users who acknowledged the ad hoc group call request and joined the ad hoc group call, and when MCVideo users join or leave the MCVideo ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCVideo server to determine the list of participants to be invited.

16.	The MCVideo client 1, MCVideo client 2, MCVideo client 3 and MCVideo client 4 establish media plane and transmission control resources.

The MCVideo server at the primary MC system and the MCVideo at the partner MC system continuously checks whether other MCVideo users in the primary MC system and MCVideo users in the partner MC system, respectively, meet the criteria in order to add them to the ongoing ad hoc group communication, or whether any of the participating MCVideo users in the primary MC system and MCVideo users in the partner MC system, respectively, no longer meet the criteria in order to remove them from the ongoing ad hoc group communication, according to clause 7.17.3.2.4.

* * * Next Change * * *

##### 7.19.3.2.2	Modification of ad hoc group call participants by the MCVideo server

Figure 7.19.3.2.2-1 below illustrates the MCVideo server modifying the ad hoc group call participants procedure involving multiple MCVideo systems.

Pre-conditions:

1.	The MCVideo user at MCVideo client 1 is authorized to initiate ad hoc group call.

2.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems determined the participants for the ad hoc group call based on the criteria specified by the MCVideo client 1 while initiating the ad hoc group call.

3.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The MCVideo server 1 of the primary MCVideo system detects that the MCVideo client 3 meets the criteria and MCVideo client 2 stops to meet the criteria specified by the MCVideo client 1.

5.	The MCVideo server 2 of the partner MCVideo system detects that the MCVideo client 5 meets the criteria and MCVideo client 4 stops to meet the criteria specified by the MCVideo client 1.

Figure 7.19.3.2.2-1: Modification of ad hoc group call participants by the MCVideo server

1.	The ad hoc group call is established and on-going with the participants MCVideo client 1, MCVideo client 2, and MCVideo client 4. The participants list is determined by both primary and partner systems MCVideo server based on the criteria specified by the MCVideo client 1 while initiating the call.

2.	The MCVideo server 1 detects that MCVideo client 3 now meets criteria and successfully added to call.

3.	The MCVideo server 1 detects that MCVideo client 2 does not meet criteria any more and successfully removed from a call.

4.	The MCVideo server 2 detects that the MCVideo client 5 meets the criteria specified by the MCVideo client 1.

5.	The MCVideo server 2 sends the notification to MCVideo server 1 of the primary MCVideo system to add the MCVideo user at MCVideo client 5 to on-going ad hoc group call.

6.	The MCVideo server 1 sends the ad hoc group call request to the MCVideo client 5 via MCVideo server 2.

7.	The MCVideo client 5 notifies the user about the incoming ad hoc group call.

8.	The MCVideo client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCVideo server 1 via MCVideo server 2.

9.	The on-going ad hoc group call is updated by adding MCVideo client 5 which meets the criteria specified by the MCVideo client 1.

10.	The MCVideo server 2 detects that the MCVideo client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

11.	The MCVideo server 2 sends the notification to MCVideo server 1 of the primary MCVideo system to remove the MCVideo user at MCVideo client 4 from on-going ad hoc group call.

12.	The MCVideo server 1 sends the ad hoc group call leave request to the MCVideo client 4 and removes it from the on-going ad hoc group call via MCVideo server 2.

13.	The MCVideo client 4 notifies the user of the ad hoc group call leave request.

14.	The MCVideo client 4 sends the ad hoc group call leave response to the MCVideo server 1 via MCVideo server 2.

15.	The on-going ad hoc group call is updated by removing MCVideo client 4, which no more meets the criteria specified by the MCVideo client 1.

* * * Next Change * * *

##### 7.19.3.2.3	Release ad hoc group call and stop determining the ad hoc group call participants by partner MCVideo system – Participants list determined by the MCVideo server

Figure 7.19.3.2.3-1 below illustrates the release of ad hoc group call and stopping of MCVideo server at the partner MC system from determining the ad hoc group call participants’ procedure once involving multiple MCVideo systems. This procedure, in particular describes about how the partner MC system is notified about ad hoc group call release to cease the determining of the participants by the partner MC system.

Pre-conditions:

1.	The MCVideo user at MCVideo client 1 is authorized to initiate ad hoc group call.

2.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems determined the participants for the ad hoc group call based on the criteria specified by the MCVideo client 1 while initiating the ad hoc group call.

3.	The MCVideo server 1 of the primary and MCVideo server 2 of the partner MCVideo systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

Figure 7.19.3.2.3-1: MCVideo server releases an ad hoc group call and stops the determination of ad hoc group call participants involving multiple MCVideo systems

1.	The ad hoc group call is established and on-going with the participants MCVideo client 1, MCVideo client 2, MCVideo client 3, and MCVideo client 4. The participants list is determined by both primary and partner systems MCVideo server based on the criteria specified by the MCVideo client 1 while initiating the call.

2.	The MCVideo server 1 detects that the ad hoc group call which is ongoing is to be released e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

3.	The MCVideo server 1 sends the notification to MCVideo server 2 of the partner MCVideo system about the ad hoc group call release to stop determining the participants list by MCVideo server 2 of the partner MCVideo system.

4a-4d.	The MCVideo server 1 sends ad hoc group call release request to all the participants of the ad hoc group call. MCVideo client 3/MCVideo client 4 which belong to the partner MCVideo system, receive the ad hoc group call release request via the partner MCVideo server 2 from MCVideo server 1 of the primary MCVideo system.

5a-5d.	The MCVideo clients notifies the user about the release of the ad hoc group call.

6a-6d.	All the participants of the ad hoc group call receives the ad hoc group call release request and sends the ad hoc group call release response to the MCVideo server 1 of the primary MCVideo server. MCVideo client 3/MCVideo client 4 which belong to the partner MCVideo system, send the ad hoc group call release response via the partner MCVideo server 2 to the MCVideo server 1 of the primary MCVideo system.

* * * Next Change * * *

##### 7.19.3.2.4	Procedure for ad hoc group call setup – Participants list provided by the Initiator

Figure 7.19.3.2.4-1 illustrates the procedure for ad hoc group call setup procedure initiated by an authorized user wherein either a list of participants or an ad hoc group ID from an ad hoc group emergency alert is provided by the authorised user and the determined MCVideo users are from multiple MCVideo systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system shares their users' information to the primary MC system.

2.	The authorized MCVideo user/dispatcher belongs to the primary MC system.

3.	The MCVideo server of the primary MC system is where the authorized MCVideo user/dispatcher controls the ad hoc group.

4.	Some users of the ad hoc group belong to partner MC systems.

5.	The preconfigured group identity and preconfigured group configuration to be used for an ad hoc group have been preconfigured in the MCVideo client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

6.	If an ad hoc group ID from an ad hoc group emergency alert is used to invite the ad hoc group members, MCVideo client is aware of the ad hoc group ID.

Figure 7.19.3.2.4-1: Ad hoc group call setup

1.	The MCVideo client of the authorized user initiates an ad hoc group call with multiple users from primary and partner MC systems. An ad hoc group call request message with the information of the MCVideo IDs or an ad hoc group ID if the ad hoc group call follows an ad hoc group emergency alert is routed to the MCVideo server of the primary MC system.

2.	If the ad hoc group call is supported, the MCVideo server of the primary MC system verifies whether the user at MCVideo client is authorized to initiate an ad hoc group call. If not authorized, the MCVideo server of the primary MC system rejects the ad hoc group call request as specified in the step 3.

If the information received in the request in step 1 does not contain an ad hoc group ID from an ad hoc group emergency alert, the MCVideo server of the primary MC system forms the ad hoc group by using received information. The MCVideo server further determines the preconfigured group to be used for the configuration and assigns an MCVideo group ID for the newly formed ad hoc group.

If no MCVideo ad hoc group ID was included in the ad hoc group call request of step 1, or if the provided MCVideo ad hoc group ID is not accepted by the MCVideo server, the MCVideo server assigns a MCVideo group ID for the newly formed ad hoc group

If the the MC service users' information received in step 1 does not contain an MCVideo ad hoc group ID from an ad hoc group emergency alert, the MCVideo server of the primary MC system forms the ad hoc group by using MCVideo users' information received in step 1, determines the preconfigured group to be used for the configuration of the ad hoc and assigns a MCVideo group ID for the newly formed ad hoc group.

The MCVideo server identifies the appropriate MCVideo server responsible for the MCVideo users of the ad hoc group. The MCVideo server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

NOTE 1:	The ad hoc group information including the MCVideo group ID and the list of users is held in dynamic data in the MCVideo server.

3.	The MCVideo server shall send the ad hoc group call request return message to the MCVideo client containing the below:

i.	The MCVideo ad hoc group ID either generated by the MCVideo server or provided by the MCVideo client if the ad hoc group ID is from an ad hoc group emergency alert (only included when the ad hoc group call is authorized);

ii.	The group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not.

If the ad hoc group call request is not authorized, the MCVideo client shall not proceed with the rest of the steps.

4.	The MCVideo server of the primary MC system sends the ad hoc group call request to the affiliated group members of the ad hoc group belonging to the primary MC system.

5.	The MCVideo clients receive in the ad hoc group call request the information of the MCVideo group ID for the ad hoc group and further notify their corresponding MCVideo user. The affiliated group members of the ad hoc group of the primary MC system may accept or reject the call and respond with the group call response.

6.	The primary MCVideo server further initiates an ad hoc group call request message to the MCVideo users of the partner MC system. The ad hoc group call request message is routed to the MCVideo users via the MCVideo server of the partner MC system.

7.	The MCVideo clients receive in the ad hoc group call request the information of the MCVideo group ID for the ad hoc group and further notify their corresponding MCVideo user. The MCVideo users upon receipt of the invitation may accept or reject the call, and respond with the ad hoc group call response. The ad hoc group call response message is routed to the MCVideo server of the primary MC system via the MCVideo server of the partner MC system.

8.	The MCVideo server of the primary MC system provides an ad hoc group call response message to the MCVideo client of the authorized MCVideo user upon receiving response to the corresponding ad hoc group call request in step 1. The ad hoc group call response will consist of the success or failure result and/or detailed reason information in case of failure.

NOTE 2:	The ad hoc group call response message is triggered depending on the conditions to proceed with the call.

9.	Upon successful ad hoc group call setup, a group call is established amongst the multiple group members from primary and partner MC systems. The media plane and transmission control resources are established.

* * * Next Change * * *

##### 7.19.3.2.5	Procedure for ad hoc group call release by MCVideo server – Participants list provided by the Initiator

This procedure focuses on the case where an MCVideo server initiates the termination of an ongoing MCVideo ad hoc group call for all the participants of that group call, since at least one of the termination conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

Procedures in figure 7.19.3.2.5-1 are the signalling control plane procedures for the MCVideo server initiating termination of an ongoing MCVideo ad hoc group call.

Pre-condition:

-	The MCVideo client 1 and MCVideo client 2 belong to primary MC system. The MCVideo client 3 belongs to partner MC system.

Figure 7.19.3.2.5-1: Ad hoc group call release

1.	The MCVideo users on MCVideo client 1, client 2 and client 3 are already part of the ongoing ad hoc group call (e.g. as a result of ad hoc group call setup as specified in clause 7.19.3.2.4).

2.	The MCVideo server would like to release the ad hoc group call which is ongoing e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

3.	The MCVideo server identifies the participants of the ongoing ad hoc group call and generates ad hoc group call release request to release ongoing session.

4.	The MCVideo server sends an ad hoc group call release request via SIP core towards each participant of the ongoing group call. If the participants belong to partner MC system, then ad hoc group call release request is routed to the MCVideo clients of the partner MC system via the partner MCVideo server.

5.	The MCVideo users are notified about the release of the ad hoc group call.

6.	The MCVideo client(s) receiving ad hoc group call release request, send acknowledgement towards the MCVideo server by sending a ad hoc group call release response. If the participants belong to partner MC system, then ad hoc group call release response is routed to the primary MCVideo server via the partner MCVideo server.

7.	The MCVideo client 1, client 2 and client 3 have successfully released the transmission control and media plane resources associated with the ad hoc group call that is terminated. The MCVideo servers remove the ad hoc group information from the dynamic data held in the MCVideo servers and the ad hoc group ceases to exist, otherwise if the ad hoc group call follows an ad hoc group emergency alert the MCVideo servers keep the dynamic data and the ad hoc group continues to exist.

* * * Next Change * * *

##### 7.19.3.2.6	Modification of ad hoc group call participants by an authorized user

Figure 7.19.3.2.6-1 below illustrates the MCVideo user modifying the ad hoc group call participants procedure involving multiple MCVideo systems.

Pre-conditions:

1.	An MCVideo ad hoc group call is already in progress and the participants list provided by the originating MCVideo user while initiating the MCVideo ad hoc group call.

2.	The participants of the MCVideo ad hoc group call belongs to the multiple MCVideo system.

3.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 3 needs to be invited to on-going ad hoc group call and MCVideo client 2 needs to be removed from the on-going ad hoc group call from the primary MCVideo system.

4.	The MCVideo user at MCVideo client 1 determines that the MCVideo client 5 needs to be invited to on-going ad hoc group call and MCVideo client 4 needs to be removed from the on-going ad hoc group call from the partner MCVideo system.

Figure 7.19.3.2.6-1: Modification of ad hoc group call participants by an authorized user

1.	The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server 1 to remove both MCVideo client 2 and MCVideo client 4 from the on-going ad hoc group call and to add both MCVideo client 3 and MCVideo client 5 into on-going ad hoc group call.

2.	The MCVideo server 1 verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCVideo server 1 sends modify ad hoc group call participants response to the MCVideo client 1.

4.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 3 from the primary MC system be invited to an on-going ad hoc group call and successfully added to the call according to the procedure defined in clause 7.19.3.1.4.

5.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 2 from the primary MC system be removed from the on-going ad hoc group call and successfully removed from call according to the procedure defined in clause 7.19.3.1.4.

6.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 5 from the partner MC system be invited to an on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call request to the MCVideo client 5 via MCVideo server 2.

7.	The MCVideo client 5 notifies the user about the incoming ad hoc group call.

8.	The MCVideo client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCVideo server 1 via MCVideo server 2.

9.	The on-going ad hoc group call is updated by adding MCVideo client 5 which is based on the modify participant list provided by the MCVideo client 1.

10.	The MCVideo server 1 determines that MCVideo client 1 has requested that MCVideo client 4 from the partner MC system be removed from the on-going ad hoc group call. The MCVideo server 1 sends the ad hoc group call leave request to the MCVideo client 4 via MCVideo server 2.

11.	The MCVideo client 4 notifies the user of the ad hoc group call leave request.

12.	The MCVideo client 4 sends the ad hoc group call leave response to the MCVideo server 1 via MCVideo server 2.

13.	The on-going ad hoc group call is updated by removing MCVideo client 4, which is based on the modify participant list provided by the MCVideo client 1.

14.	The MCVideo server 1 may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

15.	The MCVideo server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group call.

* * * Next Change * * *

##### 7.19.3.2.7	Modification of ad hoc group call criteria by an authorized user

Figure 7.19.3.2.7-1 illustrates the modification of ad hoc group call criteria procedure by the initiator of the ad hoc group call between multiple MC systems.

Preconditions:

-	The MCVideo user at the MCVideo client 1 is authorized to modify the criteria.

-	The MCVideo server A and MCVideo server B are aware of the criteria related to the ongoing ad hoc group call.

-	MC system A and MC system B are interconnected.

Figure 7.19.3.2.7-1: Modifying the criteria for determining the participants during an ongoing ad hoc group call between multiple MC systems

1.	An ad hoc group call has been established based on criteria sent by an authorized MCVideo client upon initiating the ad hoc group call.

2.	The MCVideo user at the MCVideo client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group call participants.

3a.	MCVideo client 1 sends a modify ad hoc group call criteria request to MCVideo server A.

3b.	MCVideo server A determines that the modify ad hoc group call criteria request received in step 3a has impact on users in MCVideo server B. Depending on the criteria provided and based on local policy, the MCVideo server may modify the content of the criteria received in step 3a to determine the list of participants.

3c.	MCVideo server A sends a modify ad hoc group call criteria request to MCVideo server B containing the call resulting criteria to be used by MCVideo server B.

NOTE 1:	If MCVideo server A identifies that the criteria require to add participants from another MC system then MCVideo server A sends an ad hoc group call request to the MCVideo server in that MC system.

4a/b.	The MCVideo server A receives the ad hoc group call criteria modify response and forwards the response to MCVideo client 1 in MC system A.

5.	MCVideo server B detects that MCVideo client 3 meets the criteria for the ongoing ad hoc group call initiated at MC system A.

5a.	MCVideo server B sends an ad hoc group call add user notification message towards MCVideo server A.

5b.	MCVideo server A sends an ad hoc group call request towards MCVideo client 3 via MCVideo server B.

5c.	The MCVideo user 3 is notified of entering an ongoing ad hoc group call.

5d.	MCVideo client 3 sends an ad hoc group call response towards MCVideo server A via MCVideo server B.

5e.	MCVideo server B sends an ad hoc group call remove user notification message towards MCVideo server A.

5f.	MCVideo server A sends an ad hoc group call leave request towards MCVideo client 4 via MCVideo server B.

5g.	The MCVideo user 4 is notified of leaving an ongoing ad hoc group call.

5h.	MCVideo client 4 sends an ad hoc group call leave response towards MCVideo server A via MCVideo server B.

6.	The MCVideo server A may notify the authorised MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the authorised MCVideo user by the MCVideo server A more than once during the call when MCVideo users join or leave the ad hoc group call. All ad hoc group call notify messages contain the call resulting criteria used by the MCVideo server to determine the list of participants to be invited.

7.	The MCVideo server A adds MCVideo client 3 to the ad hoc group call and removes MCVideo client 4 from the ad hoc group call.

The MCVideo servers continuously check whether other MCVideo clients meet or if participating MCVideo clients no longer meet the criteria for the ad hoc group call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of the ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

* * * End of change * * *