3GPP TSG-SA WG6 Meeting #71	S6-260056

Goa, India 9th – 13th February 2026	(revision of S6-260xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0515 | rev |  | Current version: | 19.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure using the group host MCPTT server in an AHGC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing procedure using the group host MCPTT server in an AHGC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new procedure showing an ad hoc group call using a group host MCPTT server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 guidance for stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.19.3.x	Ad hoc group call using group host MCPTT server

Figure 10.19.3.x-1 below illustrates the ad hoc group call setup using group host MCPTT server procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user at MCPTT client 1 wants to invite MCPTT users at MCPTT client 2 and MCPTT client 3 for the ad hoc group call and is aware of the MCPTT IDs of the participants or to invite the ad hoc group member if the ad hoc group call follows an ad hoc group emergency alert, the MCPTT client 1 is aware of the ad hoc group ID.

2.	Number of participants being invited for the ad hoc group call is within the limit.

3.	End-to-End encryption is supported for this ad hoc group call.

4.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group call have been preconfigured in MCPTT client 1 and other participants of the ad hoc group call have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

NOTE 1:	Selection of MCPTT IDs of the participants can be manual or from the user profile configuration data or by any other means. This is left for the implementation.

Figure 10.19.3.x-1: Ad hoc group call using Group host MCPTT server

1.	User at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing the list of participants or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server. Encryption supported information element is set to true since end-to-end encryption is supported. An SDP offer containing the MCPTT client 1 media parameters is included. If there is a floor request to transmit, then the ad hoc group call request contains an indication of an implicit floor request. If the MCPTT user at MCPTT client 1 has activated a functional alias, then the ad hoc group call request contains that functional alias. If the ad hoc group call request contains an implicit floor request it may also include location information.

If the MCPTT user at MCPTT client 1 initiates an MCPTT emergency ad hoc group call or the MCPTT emergency state is already set for the MCPTT client 1 (due to a previously triggered MCPTT emergency alert):

i.	the ad hoc group call request shall contain an emergency indicator;

ii.	if the MCPTT emergency state is not set already, MCPTT client 1 sets its MCPTT emergency state. The MCPTT emergency state of MCPTT client 1 is retained until explicitly cancelled by the MCPTT user at MCPTT client 1.

2.	If the ad hoc group call is supported, the MCPTT server verifies whether the MCPTT user at MCPTT client 1 is authorized to initiate an MCPTT ad hoc group call. If not authorized, the MCPTT server rejects the ad hoc group call request as specified in step 3. The MCPTT server accepts the ad hoc group call request if the ad hoc group call is authorized. If authorised, it validates whether the number of invited participants is within the configured limit before proceeding with the call setup.

If the ad hoc group call request contains a list of functional alias(es) instead of a list of MCPTT ID(s) as participants being invited to the ad hoc group call, the MCPTT server shall check whether the MCPTT user at MCPTT client 1 is allowed to use the functional alias of MCPTT users at MCPTT client 2 and 3 to setup an ad hoc group call and whether MCPTT client 2 and 3 are allowed to receive an ad hoc group call from MCPTT client 1 using the functional alias. The MCPTT server shall resolve the functional alias(es) to the corresponding MCPTT ID(s) for which each functional alias is active.

If functional alias is present, the MCPTT server checks whether the provided functional alias allowed to be used and has been activated for the user.

If location information was included in the ad hoc group call request, the MCPTT server checks the privacy policy of the MCPTT user at the MCPTT client 1 to decide if the location information of MCPTT client 1 can be provided to other users on the call (refer to Annex A.3 "Authorisation to provide location information to other MCPTT users on a call when talking").

If an emergency indicator is present in the received ad hoc group call request, the MCPTT ad hoc group is considered to be in the in-progress emergency state until this ad hoc group call is terminated.

If an imminent peril indicator is present in the received ad hoc group call request, the MCPTT ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group call is terminated.

If the ad hoc group call request received in step 1 does not contain an MCPTT ad hoc group ID from an ad hoc group emergency alert, the MCPTT server forms the ad hoc group by using information received in step 1, determines the preconfigured group to be used for the configuration of the ad hoc group call. If no MCPTT ad hoc group ID was included in the ad hoc group call request in step 1, or if the provided MCPTT ad hoc group ID is not accepted by the MCPTT server, the MCPTT server assigns a MCPTT ad hoc group ID for the newly formed MCPTT ad hoc group call.

The MCPTT server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group call.

3.	The MCPTT server shall send the ad hoc group call request return message to MCPTT client 1 containing the below:

i.	The MCPTT ad hoc group ID either generated by the MCPTT server or provided by the MCPTT client 1 if the MCPTT ad hoc group ID is from an ad hoc group emergency alert (only included when the ad hoc group call is authorized);

ii.	The MCPTT group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not.

If the ad hoc group call request is not authorized, the MCPTT server and MCPTT client 1 shall not proceed with the rest of the steps.

4.	As the MCPTT ad hoc group ID identifies a different MCPTT server hosts this ad hoc group call, the MCPTT server modified ad hoc group call request message accordingly and sends it to the group host MCPTT server.

5a-5b.	The group host MCPTT server sends the ad hoc group call requests towards the MCPTT clients of the invited users based on step 1 and 2. While sending the ad hoc group call requests, the group host MCPTT server shall remove the information elements that are not required to be conveyed to the target MCPTT clients (e.g. MCPTT ID list of the users who are required to acknowledge)

6a-6b.	The receiving MCPTT clients notify their corresponding MCPTT users about the incoming MCPTT ad hoc group call.

7a-7b.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the group host MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by its hosting MCPTT server.

NOTE 2:	For emergency ad hoc group call the receiving MCPTT client shall always accept the request.

8.	The group host MCPTT server sends the ad hoc group call response to the MCPTT server.

9. 	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

NOTE 3:	Steps 6 to step 9 can start to occur before all of step 5 is completed since the group host MCPTT server do not require to wait for the previous ad hoc group call request to complete before sending the ad hoc group request to another participant.

10.	If the initiating MCPTT user requires the acknowledgement from the invited MCPTT users, and the required MCPTT users do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), then the MCPTT server may proceed with or abandon the call and then notify the initiating MCPTT user that the acknowledgements did not include all required members according to ad hoc group call policy from the user profile configuration. The MCPTT server may notify the initiating MCPTT user of all MCPTT users who did not acknowledge the ad hoc group call request within the configured time. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when invited MCPTT users join or leave the ad hoc group call.

11.	MCPTT client 1, MCPTT client 2 and MCPTT client 3 establish media plane and floor control resources.

NOTE 4:	Step 11 can occur any time following step 9 if the conditions to proceed with the call are met.