3GPP TSG-SA  Meeting #75	SP-170079

# Dubrovnik, Croatia, 8th Mar 2017 - 10th Mar 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0315 | rev | 1 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of emergency procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | BlackBerry UK Limited, Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | SEW2 |  |  |  |  |  | Date: |  |  | 2017-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Annex J.2 requires that the UE is able to store between 0 and 10 emergency numbers received from the network. Like requirements for other access can be found in stage 3: TS 24.008 and TS 24.301. TS 23.401 does not have a simular requirement. TS 23.167 does not have simular requirements outside annex J. The requirement is out of scope in a document describing the stage 2 service description for emergency services. In addition, 23.167 continues to limit support for emergency services over WLAN to S2b. Changes since rev0: - Updated coverpage - Removed dependency - Updated CR template - Added cosigner |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex E limits support for emergency services over WLAN to S2b. Remove stage 3 requirement from stage 2 TS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency services in TS 23.167 continues to be limited to S2b, according to Annex E. Stage 3 requirement found in Annex J of TS 23.167. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex E, J.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

Annex E (Informative):
Emergency support in different IP-CANs

Support for emergency services in the IP Multimedia Core Network can be provided by certain IP-CANs according to table E.1, which shows the level of possible emergency support in this version of the specification. The UE may be able to use also other IP-CANs that are not included in table E.1 to access emergency services in the IP Multimedia Core Network, but the support of such access technologies is out of scope of this specification.

Table E.1: Support for IMS Emergency Services by Different IP-CANs

| IP-CAN | Normal Access | Emergency support | Insufficient Security Credentials |
| --- | --- | --- | --- |
| GPRS (UTRAN) | X | X | X |
| Fixed Broadband | X | X | X |
| cdma2000 HRPD/PDS | X | X | X |
| cdma2000 HRPD/EPC | X | X | X |
| EPS (UTRAN and E-UTRAN) | X | X | X |
| WLAN access to EPS | X (NOTE 1) | X | - |
| NOTE 1: Emergency Sessions use a normal access to EPC only in case of emergency sessions not detected by the UE and nevertheless served by the IMS network. |  |  |  |

The term "normal access" in table E.1 means that the IP-CAN is not made aware of the UE intent to establish an emergency services session and therefore provides no special emergency support e.g. call priority.

The term "emergency support" in the table means that the IP-CAN is made aware of the UE intent to establish an emergency services session, or at least of the priority nature of the access, so that the access network is able to handle the emergency call with appropriate priority.

The term "insufficient security credentials" in the table means that the IP CAN is able to allow a UE with no UICC card or a UICC card with insufficient credentials to obtain IP bearer access for emergency services in the IP Multimedia Core Network.

***** Next Change *****

# J.2	UE specific behaviour

For the specific case where the UE has selected to make an emergency call over WLAN access to EPC (hence the UE has detected that the target URI or dialled digits correspond to an emergency sessions), the UE shall use the following procedures:

-	Procedure for determining the list of emergency numbers:

-	In addition to the emergency numbers/types stored either on the USIM or on the user equipment specified in TS 22.101 [8] clause 10.1.1, the UE shall use the stored Local Emergency Numbers List received from the 3GPP network to detect that the number dialled is an emergency number.

-	If a UE has a SIM/USIM and has received a list of emergency numbers/types from the last registered PLMN via mobility management procedures as described in TS 24.301 [33] and TS 24.008 [13] when connected to the PLMN using UTRAN or E-UTRAN radio access network, the UE shall use this list to determine if the dialled digits correspond to an emergency call as long as the UE has determined that the list is valid (i.e. UE has determined that it is in the same country when the list of number was received from PLMN).

-	The UE may also determine that the dialled digits/types correspond to an emergency call based on the list of Emergency Call Numbers retrieved via a previous query to a DNS in the same country or via Access Network Query Protocol (ANQP) procedures defined in IEEE Std 802.11-2012 [47] from the associated WLAN AP.

NOTE 1:	When using S2b, it is assumed that the DNS used for ePDG selection can provide the list of emergency numbers/types for the country.

NOTE 2:	When using S2a, ANQP procedures are considered as trustable. However, when using S2b, ANQP procedures are not trustable and should be used with care.

-	The list of downloaded emergency numbers shall be deleted at country change.

-	When the UE has selected WLAN to issue an emergency session over EPC:

-	The UE shall establish an emergency PDN connection over WLAN and shall perform an IMS emergency registration before sending an IMS emergency session request.

-	The UE shall include any available location information in the IMS emergency session request. This is further detailed in TS 23.228 [1].

-	For the media supported during IMS emergency sessions, media codec and format support is specified in TS 26.114 [34].

-	For the support of DRVCC on emergency call on WLAN, see TS 23.237 [32].

A UE shall not establish an emergency PDN connection over WLAN if the UE initiated a normal (i.e. non-emergency) session and is subsequently informed by the network that the ongoing session is accepted by the network even though it is an emergency session.