SA WG2 Meeting #S2-136	 S2-1912433

# 18 - 24 Nov, 2019, Reno, Nevada, US									   (revision of S2-1911048 )

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1721 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UDM subscription to Nsmf_EventExposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is described in subclause 4.15.3.2.3 of TS 23.502, if the requested event (e.g. "Availability after DDN Failure") requires SMF assistance, the UDM selects the SMF based on the Slice information, the DNN and/or the AF Service ID. However, AF-Service-Identifier is not part of the PDU Session data that are registered by SMF in the UDM, and therefore it’s not possible for the UDM to identify the SMF for event exposure subscription based on AF-Service-Identifier. The UDM selecting the SMF based on the Slice information and DNN means that the NEF need to include S-NSSAI and DNN in the subscription to UDM exposure event. How the NEF can determine the combination of S-NSSAI and DNN depends on what info is provided by the AF as follows: Option-1 NEF determines the combination of S-NSSAI and DNN based on mapping from AF-Service-Identifier; Option-2 NEF determines the combination of S-NSSAI and DNN based on mapping from AF-Identity. Mapping from AF-Service-Identifier to DNN and S-NSSAI combination is specified in the AF influencing traffc routing, see below from TS 23.502: 4.3.6 Application Function influence on traffic routing 4.3.6.1 General … If the AF interacts with PCF via the NEF, the NEF performs the following mappings where needed: - Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration. - Map the AF-Service-Identifier into a list of DNAI(s) and Routing Profile ID(s) determined by local configuration. And mapping from AF Identity to DNN and S-NSSAI combination is specified in the NIDD procedure in clause 5.31.5 of TS 23.501 v16.2.0. Considering that AF-Identity is always available from the AF while the AF-Service-Identifier may not, it’s proposed that the NEF use AF-Identity as input to determine DNN and S-NSSAI. Please also note that AF-Identity is already specified for monitoring event according to stage 3 (i.e. TS 29.522 which refers to 29.122 for the event configuration) while AF-Service-Identifier is currently specified only for traffic influencing, which means that no impact on stage 3 if AF-Identity is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “AF service ID” from the descripton of UDM subscritpion to Nsmf_EventExposure. Clarify that NEF maps the AF-Identity into the DNN and S-NSSAI combination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM subscritpion to Nsmf_EventExposure does not work as it’s not possible for the UDM to identify the SMF for event exposure subscription based on AF-Service-Identifier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.3, 5.2.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Using AF-Identity does not have stage 3 impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

##### 4.15.3.2.3	NEF service operations information flow

The procedure is used by the AF to subscribe to notifications and to explicitly cancel a previous subscription. Cancelling is done by sending Nnef_EventExposure_Unsubscribe request identifying the subscription to cancel with Subscription Correlation ID. The notification steps 6 to 8 are not applicable in cancellation case. If AMF or SMF receive Monitoring event subscription for roaming UEs and I-NEF is deployed as defined in clause 6.2.5a of TS 23.501 [2], clause 4.15.3.2.3a applies after step 5.

Figure 4.15.3.2.3-1: Nnef_EventExposure_Subscribe, Unsubscribe and Notify operations

1.	The AF subscribes to one or several Event(s) (identified by Event ID) and provides the associated notification endpoint of the AF by sending Nnef_EventExposure_Subscribe request.

Event Reporting Information defines the type of reporting requested (e.g. one-time reporting, periodic reporting or event based reporting, for Monitoring Events). If the reporting event subscription is authorized by the NEF, the NEF records the association of the event trigger and the requester identity. The subscription may also include Maximum number of reports and/or Maximum duration of reporting IE.

The AF may also provide Traffic Descriptor (e.g. IP 3-tuples) information to support "Availability after DDN Failure" for multiple AFs case.

2.	[Conditional - depending on authorization in step 1] The NEF subscribes to received Event(s) (identified by Event ID) and provides the associated notification endpoint of the NEF to UDM by sending Nudm_EventExposure_Subscribe request. The NEF maps the AF-Identifier into DNN and S-NSSAI combination based on local configuration, and include DNN, S-NSSAI in the request.

If the reporting event subscription is authorized by the UDM, the UDM records the association of the event trigger and the requester identity. Otherwise, the UDM continues in step 4 indicating failure.

3a.	[Conditional] If the requested event (e.g. monitoring of Loss of Connectivity) requires AMF assistance, then the UDM sends the Namf_EventExposure_Subscribe to the AMF serving the requested user. The UDM sends the Namf_EventExposure_Subscribe request to the all serving AMF(s) (if subscription applies to a UE or a group of UE(s)), or all the AMF in the same PLMN as the UDM (if subscription applies to any UE).

As the UDM itself is not the Event Receiving NF, the UDM shall additionally provide the notification endpoint of itself besides the notification endpoint of NEF. Each notification endpoint is associated with the related (set of) Event ID(s). This is to assure the UDM can receive the notification of subscription change related event.

If the subscription applies to a group of UE(s), the UDM shall include the same notification endpoint of itself, i.e. Notification Target Address (+ Notification Correlation Id), in the subscriptions to all UE's serving AMF(s).

NOTE 1:	The same notification endpoint of UDM is to help the AMF identify whether the subscription for the requested group event is same or not when a new group member UE is registered.

3b.	[Conditional] AMF acknowledges the execution of Namf_EventExposure_Subscribe.

3c.	[Conditional] If the requested event (e.g. "Availability after DDN Failure" with traffic descriptor) requires SMF assistance, then for existing PDU session(s) or during the PDU session establishement procedure, the UDM selects the SMF based on the Slice information, the DNN and/or the AF Service ID and sends the Nsmf_EventExposure_Subscribe with the event type "Availability after DDN Failure" and Traffic Descriptor to the SMF(s) serving the requested user. As the UDM itself is not the Event Receiving NF, the UDM shall provide the AMF's notification endpoint information, i.e., Notification Target Address (+ Notification Correlation Id). Each notification endpoint is associated with the related (set of) Event ID(s).

NOTE 2:	In the home routed case, the UDM sends the subscription to the V-SMF via the H-SMF.

3d.	[Conditional] The SMF acknowledges the execution of Nsmf_EventExposure_Subscribe.

4.	[Conditional] UDM acknowledges the execution of Nudm_EventExposure_Subscribe.

If the subscription is applicable to a group of UE(s) and the Maximum number of reports is included in the Event Report information in step 1, the Number of UEs is included in the acknowledgement.

5.	NEF acknowledges the execution of Nnef_EventExposure_Subscribe to the requester that initiated the request.

6a - 6b.	[Conditional - depending on the Event] The UDM (depending on the Event) detects the event occurs and sends the event report, by means of Nudm_EventExposure_Notify message to the associated notification endpoint of the NEF along with the time stamp. NEF may store the information in the UDR along with the time stamp using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

6c - 6d.	[Conditional - depending on the Event] The AMF detects the event occurs and sends the event report, by means of Namf_EventExposure_Notify message to associated notification endpoint of the NEF along with the time stamp. NEF may store the information in the UDR along with the time stamp using either Nudr_DM_Create or Nudr_DM_Update service operation as appropriate.

If the AMF has a maximum number of reports stored for the UE or the individual member UE, the AMF shall decrease its value by one for the reported event.

For both step 6a and step 6b, when the maximum number of reports is reached and if the subscription is applied to a UE, The NEF unsubscribes the monitoring event(s) to the UDM and the UDM unsubscribes the monitoring event(s) to AMF serving for that UE.

For both step 6a and step 6b, when the maximum number of reports is reached for an individual group member UE, the NEF uses the Number of UEs received in step 4 to determine if reporting for the group is complete. If the NEF determines that reporting for the group is complete, the NEF unsubscribes the monitoring event(s) to the UDM and the UDM unsubscribes the monitoring event(s) to all AMF(s) serving the UEs belonging to that group.

When the Maximum duration of reporting expires in the NEF, the UDM and the AMF, then each of these nodes shall locally unsubscribe the monitoring event.

6e.	[Conditional - depending on the Event] When the SMF is informed that the UE is unreachable, the SMF interacts with the UPF to remove the buffered packets and requests the UPF to report the traffic information (e.g. Source IP address, Source port number) of the discarded packets. By comparing the traffic information reported by the UPF with the Traffic Descriptor(s) received in the event subscription(s), the SMF determines whether any traffic from an AF having a event subscription occurred and if so, the SMF sends the event report, by means of Nsmf_EventExposure_Notify message (including NEF reference ID), to the AMF indicated as notification endpoint. If the UE is not reachable after the AMF received the notification from the SMF, the AMF shall set a Notify-on-available-after-DDN-failure flag corresponding to the NEF Reference ID.

6f.	[Conditional] The AMF detects the UE is reachable and sends the event report based on the Notify-on-available-after-DDN-failure flag received in step 6e, by means of Namf_EventExposure_Notify message, only to the NEF indicated as notification endpoint. In this way, only the AF(s) for which DL traffic transmission failed are notified.

7.	[Conditional - depending on the Event in steps 6a-6f] The NEF forwards to the AF the reporting event received by either Nudm_EventExposure_Notify and/or Namf_EventExposure_Notify.

8.	[Conditional - depending on the Event] The AMF detects the subscription change related event occurs, e.g. Subscription Correlation ID change due to AMF reallocation or addition of new Subscription Correlation ID due to a new group UE registered, it sends the event report, by means of Namf_EventExposure_Notify message to the associated notification endpoint of the UDM.

***** 2nd CHANGE *****

##### 5.2.3.5.2	Nudm_EventExposure_Subscribe service operation

Service operation name: Nudm_EventExposure_Subscribe

Description: The NF consumer subscribes to receive an event, or if the subscription is already defined in UDM, then the subscription is updated.

NF Consumers: NEF.

Inputs (required): Target of Event Reporting: UE(s) ID (SUPI or GPSI, Internal Group Identifier or External Group Identifier, or indication that any UE is targeted), Event filter containing the Event Id(s) (see clause 4.15.3.1) and Event Reporting Information defined in Table 4.15.1-1.

Inputs (optional): Expiry time, DNN, S-NSSAI.

Outputs (required): Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs (optional): First corresponding event report is included, if corresponding information is available (see clause 4.15.1), Number of UE if the External Group Identifier and Maximum Number of Reports are included in the inputs.

Number of UEs indicates the number of UEs within the group identified by the External Group Identifier. The NEF uses this value to determine whether the monitoring event has been reported for all group member UEs.

***** End of CHANGE *****