3GPP TSG-SA WG2 Meeting #136	S2-1912015

# Reno, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0023 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Application classifier at Rx UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In broadcast communications over PC5, the Rx UE determines the appropriate V2X service based on the destination Layer-2 ID and forwards the message to the determined V2X service. In unicast communications over PC5, a PC5 unicast link can support more than one V2X services/applications. Therefore, the Rx UE can not determine the appropriated V2X service based on the destination Layer-2 ID because the destination Layer-2 ID doesn’t indicate a certain V2X service/application. [In Rev2] In TS 23.287, it states: “the UE determines if there is any existing PC5 QoS Flow matching the service data or request, i.e. based on the PC5 QoS Rules for the existing PC5 QoS Flow(s).” “the UE maintains the mappings of PFIs to the PC5 QoS Context and PC5 QoS Rules per destination identified by Destination Layer-2 ID.” “A PC5 QoS Context includes PC5 QoS parameters (e.g. PQI and Range) and the V2X service type (e.g. PSID or ITS-AID). When the UE assigns a new PFI for V2X service, the UE stores it with the corresponding PC5 QoS Context and PC5 QoS Rules for the destination.” It implies that PFI can be a value to be used to identify the V2X service type. The Rx UE can determine the appropriate V2X service/application wherein to forward the received V2X messages based on the PFI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | clarify the usage of PFI at Rx UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurancy in unicast communications over PC5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmission of unhandled paper from SA2#135 Rev2: clarify the usage of PFI at Rx UE Rev3: clarify that PFI is used to identify V2X service |  |  |  |  |  |  |  |  |

*** 1st change ***

5.2.1.4	Unicast mode communication over PC5 reference point

Unicast mode of communication is only supported over NR based PC5 reference point. Figure 5.2.1.4-1 illustrates an example of PC5 unicast links.

Figure 5.2.1.4-1: Example of PC5 Unicast Links

The following principles apply when the V2X communication is carried over PC5 unicast link:

-	A PC5 unicast link between two UEs allows V2X communication between one or more pairs of peer V2X services in these UEs. All V2X services in the UE using the same PC5 unicast link use the same Application Layer ID.

NOTE 1:	An Application Layer ID may change in time as described in clauses 5.6.1.1 and 6.3.3.2, due to privacy. This does not cause a re-establishment of a PC5 unicast link.

-	One PC5 unicast link supports one or more V2X services (e.g. PSIDs or ITS-AIDs) if these V2X services are at least associated with the pair of peer Application Layer IDs for this PC5 unicast link. For example, as illustrated in Figure 5.2.1.4-1, UE A and UE B have two PC5 unicast links, one between peer Application Layer ID 1/UE A and Application Layer ID 2/UE B and one between peer Application Layer ID 3/UE A and Application Layer ID 4/UE B.

NOTE 2:	A source UE is not required to know whether different target Application Layer IDs over different PC5 unicast links belong to the same target UE.

-	A PC5 unicast link supports V2X communication using a single network layer protocol e.g. IP or non-IP.

-	A PC5 unicast link supports per-flow QoS model as specified in clause 5.4.1.

When the Application layer in the UE initiates data transfer for a V2X service which requires unicast mode of communication over PC5 reference point:

-	the UE shall reuse an existing PC5 unicast link if the pair of peer Application Layer IDs and the network layer protocol of this PC5 unicast link are identical to those required by the application layer in the UE for this V2X service, and modify the existing PC5 unicast link to add this V2X service as specified in clause 6.3.3.4; otherwise

-	the UE shall trigger the establishment of a new PC5 unicast link as specified in clause 6.3.3.1.

After successful PC5 unicast link establishment, UE A and UE B use the same pair of Layer-2 IDs for subsequent PC5-S signalling message exchange and V2X service data transmission as specified in clause 5.6.1.4. The V2X layer of the transmitting UE indicates to the AS layer whether a transmission is for a PC5-S signalling message (i.e. Direct Communication Request/Accept, Link Identifier Update Request/Response, Disconnect Request/Response, Link Modification Request/Accept) or V2X service data.

For every PC5 unicast link, a UE self-assigns a distinct PC5 Link Identifier that uniquely identifies the PC5 unicast link in the UE for the lifetime of the PC5 unicast link. Each PC5 unicast link is associated with a Unicast Link Profile which includes:

-	service type(s) (e.g. PSID or ITS-AID), Application Layer ID and Layer-2 ID of UE A; and

-	Application Layer ID and Layer-2 ID of UE B; and

-	network layer protocol used on the PC5 unicast link; and

-	for each V2X service, a set of PC5 QoS Flow Identifier(s) (PFI(s)). Each PFI is associated with QoS parameters (i.e. PQI and optionally Range).

For privacy reason, the Application Layer IDs and Layer-2 IDs may change as described in clauses 5.6.1.1 and 6.3.3.2 during the lifetime of the PC5 unicast link and, if so, shall be updated in the Unicast Link Profile accordingly. The UE uses PC5 Link Identifier to indicate the PC5 unicast link to V2X Application layer, therefore V2X Application layer identifies the corresponding PC5 unicast link even if there are more than one unicast link associated with one service type (e.g. the UE establishes multiple unicast links with multiple UEs for a same service type).

The Unicast Link Profile shall be updated accordingly after a Layer-2 link modification for an established PC5 unicast link as specified in clause 6.3.3.4.

V2X Service Info and QoS Info are carried in PC5-S signalling messages and exchanged between two UEs as specified in clause 6.3.3. Based on the exchanged information, PFI is used to identify V2X service. When the receiving UE receives V2X service data over the established PC5 unicast link, the receiving UE determines the appropriate V2X service based on the PFI to forward the received V2X service data to the upper layer.

*** End of the changes ***