3GPP TSG-SA5 Meeting #127	S5-196590

# Sophia-Antipolis, France, 14th Oct 2019 - 18th Oct 2019          revised S5-196308

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.659 | CR | 0038 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing stage 3 CORBA/IDL solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CORBA/IDL stage 3 solution is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to use CORBA/IDL solution for the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, A.2.2, A.2.2.x (new), A.2.2.y (new), A.2.2.z (new), A.2.2.z1 (new), A.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | y |  | O&M Specifications |  |  |  | TS 28.659 CR0035 (S5-196156CR Rel-15 28659 v1530 Add missing (E-UTRAN) cell and freq relation) |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named E-UTRAN Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning E-UTRAN resources. The E-UTRAN NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

# The present document specifies the Solution Sets for the E-UTRAN NRM IRP.

# This Solution Set specification is related to 3GPP TS 28.658 V16.2.X [4].

Next modification

## A.2.2	Information Object Class (IOC) mapping

### A.2.2.1	IOC ENBFunction

Mapping from NRM IOC ENBFunction attributes and associations to SS equivalent MOC ENBFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| eNBId | eNBId | unsignedLong |
| intraANRSwitch | intraANRSwitch | boolean |
| iRATANRSwitch | iRATANRSwitch | boolean |
| x2BlackList | x2BlackList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2WhiteList | x2WhiteList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2HOBlackList | x2HOBlackList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| x2IpAddressList | x2IpAddressList | genericEUTRANNRMAttributeTypes:: ipAddressListType |
| tceIDMappingInfoList | tceIDMappingInfoList | genericEUTRANNRMAttributeTypes:: TceIDMappingInfoListType |
| sharNetTceMappingInfoList | sharNetTceMappingInfoList | genericEUTRANNRMAttributeTypes:: SharNetTceMappingInfo |
| netListeningRSForRIBS | netListeningRSForRIBS | genericEUTRANNRMAttributeTypes:: NetListeningRSForRIBS |
| lWIPSeGWList | lWIPSeGWList | genericEUTRANNRMAttributeTypes:: lWIPSeGWListType |
| NOTE: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

### A.2.2.2	IOC EUtranGenericCell

Mapping from NRM IOC EUtranGenericCell attributes and associations to SS equivalent MOC EUtranGenericCell attributes

| IS Attributes | SS Attributes |  | SS Type |  |
| --- | --- | --- | --- | --- |
| cellLocalId | cellLocalId |  | unsignedShort |  |
| cellLocalIdList | cellLocalIdList | genericEUTRANNRMAttributeTypes:: cellLocalIdListType |  |
| cellSize | cellSize |  | genericEUTRANNRMAttributeTypes:: cellSizeEnumType |  |
| plmnIdList | plmnIdList Note: the first plmnId in the SS attribute plmnIdList is the primary PLMN id |  | genericEUTRANNRMAttributeTypes:: plmnIdListType |  |
| cellAccessInfoList | cellAccessInfoList |  | genericEUTRANNRMAttributeTypes:: cellAccessInfoListType |  |
| tac | tac |  | long |  |
| pci | pci |  | short |  |
| pciList | pciList |  | genericEUTRANNRMAttributeTypes:: pciListType |  |
| maximumTransmissionPower | maximumTransmissionPower |  | short |  |
| referenceSignalPower | referenceSignalPower |  | short |  |
| pb | pb |  | short |  |
| partOfSectorPower | partOfSectorPower |  | short |  |
| relatedTmaList | relatedTmaList |  | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |  |
| relatedAntennaList | relatedAntennaList |  | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |  |
| relatedSector | relatedSector |  | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |  |
| operationalState | operationalState |  | StateManagementIRPOptConstDefs:: OperationalStateTypeOpt |  |
| administrativeState | administrativeState |  | StateManagementIRPOptConstDefs:: AdministrativeStateTypeOpt |  |
| availabilityStatus | availabilityStatus |  | StateManagementIRPOptConstDefs:: AvailabilityStatusTypeOpt |  |
| cellResvInfo | cellResvInfo |  | genericEUTRANNRMAttributeTypes:: cellResvInfoType |  |
| nbIoTcellFlag | nbIoTcellFlag |  | genericEUTRANNRMAttributeTypes:: yesNoType |  |
| allowedAccessClasses | allowedAccessClasses |  | genericEUTRANNRMAttributeTypes:: allowedAccessEnumClassesType |  |
| isChangeForEnergySavingAllowed | isChangeForEnergySavingAllowed |  | GenericNetworkResourcesIRPSystem:: AttributeTypes::yesNoType |  |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |  |  |

### A.2.2.3	IOC ExternalEUtranGenericCell

Mapping from NRM IOC ExternalEUtranGenericCell attributes and associations to SS equivalent MOC ExternalEUtranGenericCell attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| pci | pci | short |
| plmnIdList | plmnIdList Note: the first plmnId in the SS attribute plmnIdList is the primary PLMN id | genericEUTRANNRMAttributeTypes:: plmnIdListType |
| cellLocalId | cellLocalId | unsignedShort |
| eNBId | eNBId | unsignedLong |

### A.2.2.4	IOC EUtranCellFDD

Mapping from NRM IOC EUtranCellFDD attributes and associations to SS equivalent MOC EUtranCellFDD attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| earfcnDl | earfcnDl | short |
| earfcnUl | earfcnUl | short |

### A.2.2.5	IOC ExternalEUtranCellFDD

Mapping from NRM IOC ExternalEUtranCellFDD attributes and associations to SS equivalent MOC ExternalEUtranCellFDD attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| earfcnDl | earfcnDl | short |
| earfcnUl | earfcnUl | short |

### A.2.2.6	IOC EUtranRelation

Mapping from NRM IOC EUtranRelation attributes and associations to SS equivalent MOC EUtranRelation attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| tCI | tCI | long |
| isRemoveAllowed | isRemoveAllowed | boolean |
| isHOAllowed | isHOAllowed | boolean |
| adjacentCell | adjacentCell | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| isICICInformationSendAllowed | isICICInformationSendAllowed | boolean |
| isLBAllowed | isLBAllowed | boolean |
| isESCoveredBy | isESCoveredBy | genericEUTRANNRMAttributeTypes::IsEsCoveredByEnumType |
| cellIndividualOffset | cellIndividualOffset | genericEUTRANNRMAttributeTypes::qOffsetEnumType |
| qOffset | qOffset | genericEUTRANNRMAttributeTypes::qOffsetEnumType |
| NOTE: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

### A.2.2.7	IOC Link_ENB_ENB

None.

### A.2.2.8	IOC Cdma2000Relation

Mapping from NRM IOC Cdma2000Relation attributes and associations to SS equivalent MOC Cdma2000Relation attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| adjacentSector | adjacentSector | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |

### A.2.2.9	IOC ExternalENBFunction

Mapping from NRM IOC ExternalENBFunction attributes and associations to SS equivalent MOC ExternalENBFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| eNBId | eNBId | unsignedLong |

### A.2.2.10	IOC EUtranCellTDD

Mapping from NRM IOC EUtranCellTDD attributes and associations to SS equivalent MOC EUtranCellTDD attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| earfcn | earfcn | short |
| sfAssignment | sfAssignment | short |
| specialSfPatterns | specialSfPatterns | short |

### A.2.2.11	IOC ExternalEUtranCellTDD

Mapping from NRM IOC ExternalEUtranCellTDD attributes and associations to SS equivalent MOC ExternalEUtranCellTDD attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| earfcn | earfcn | short |

### A.2.2.12	IOC MCEFunction

None.

### A.2.2.13	IOC MBSFNArea

Mapping from NRM IOC MBSFNAreaattributes and associations to SS equivalent MOC MBSFNAreaattributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| mbsfnAreaId | mbsfnAreaId | short |
| cellIdList | cellIdList | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |

### A.2.2.14	IOC RNFunction

Mapping from NRM IOC RNFunction attributes and associations to SS equivalent MOC RNFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| servingCell | servingCell | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReference |
| candidateDeNBCells | candidateDeNBCells | genericEUTRANNRMAttributeTypes:: EcgiListType |

### A.2.2.15	IOC DeNBCapability

Mapping from NRM IOC DeNBCapabilityattributes and associations to SS equivalent MOC  DeNBCapabilityattributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| servedRN | servedRN | GenericNetworkResourcesIRPSystem:: AttributeTypes::MOReferenceSet |
| maxNbrRNAllowed | maxNbrRNAllowed | unsignedShort |

### A.2.2.16	IOC ExternalRNFunction

None.

### A.2.2.17	IOC QciDscpMapping

Mapping from NRM IOC QciDscpMapping attributes and associations to SS equivalent MOC QciDscpMapping attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| qciDscpMappingList | qciDscpMappingList | genericEUTRANNRMAttributeTypes:: qciDscpMappingListType |

### A.2.2.18	IOC  CellOutageCompensationInformation

Mapping from NRM IOC CellOutageCompensationInformation attributes and associations to SS equivalent MOC CellOutageCompensationInformation attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| cOCStatus | cellOutageCompensationStatus | GenericSONPolicyNRMAttributeTypes:: cellOutageCompensationStatus |
| isCOCAllowed | isCOCAllowed | Boolean |

### A.2.2.19	IOC EUtranCellNMCentralizedSON

Mapping from NRM IOC EUtranCellNMCentralizedSON attributes and associations to SS equivalent MOC EUtranCellNMCentralizedSON attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | id | string |
| a1ThresholdRsrp | a1ThresholdRsrp | unsignedShort |
| a1ThresholdRsrq | a1ThresholdRsrq | unsignedShort |
| a2ThresholdRsrp | a2ThresholdRsrp | unsignedShort |
| a2ThresholdRsrq | a2ThresholdRsrq | unsignedShort |
| a3Offset | a3Offset | short |
| a4ThresholdRsrp | a4ThresholdRsrp | unsignedShort |
| a4ThresholdRsrq | a4ThresholdRsrq | unsignedShort |
| a5Threshold1Rsrp | a5Threshold1Rsrp | unsignedShort |
| a5Threshold1Rsrq | a5Threshold1Rsrq | unsignedShort |
| b1ThresholdUtraRscp | b1ThresholdUtraRscp | short |
| b1ThresholdUtraEcN0 | b1ThresholdUtraEcN0 | unsignedShort |
| b1ThresholdGeran | b1ThresholdGeran | unsignedShort |
| b1ThresholdCdma2000 | b1ThresholdCdma2000 | unsignedShort |
| b2Threshold1Rsrp | b2Threshold1Rsrp | unsignedShort |
| b2Threshold1Rsrq | b2Threshold1Rsrq | unsignedShort |
| b2Threshold2UtraRscp | b2Threshold2UtraRscp | short |
| b2Threshold2UtraEcN0 | b2Threshold2UtraEcN0 | unsignedShort |
| b2Threshold2Geran | b2Threshold2Geran | unsignedShort |
| b2Threshold2Cdma2000 | b2Threshold2Cdma2000 | unsignedShort |
| commonChannelPowerOffset | commonChannelPowerOffset | short |
| configurationIndex | configurationIndex | unsignedShort |
| contentionResolutionTimer | contentionResolutionTimer | genericEUTRANNRMAttributeTypes:: contentionResolutionTimerEnumType |
| hysteresisEutraA1 | hysteresisEutraA1 | unsignedShort |
| hysteresisEutraA2 | hysteresisEutraA2 | unsignedShort |
| hysteresisEutraA3 | hysteresisEutraA3 | unsignedShort |
| hysteresisEutraA4 | hysteresisEutraA4 | unsignedShort |
| hysteresisEutraA5 | hysteresisEutraA5 | unsignedShort |
| hysteresisIratB1 | hysteresisIratB1 | unsignedShort |
| hysteresisIratB2 | hysteresisIratB2 | unsignedShort |
| numberOfRaPreambles | numberOfRaPreambles | genericEUTRANNRMAttributeTypes:: numberOfRaPreamblesEnumType |
| preambleInitialReceivedTargetPower | preambleInitialReceivedTargetPower | genericEUTRANNRMAttributeTypes:: preambleInitialReceivedTargetPowerEnumType |
| preambleTransMax | preambleTransMax | genericEUTRANNRMAttributeTypes:: preambleTransMaxEnumType |
| pMax | pMax | short |
| powerRampingStep | powerRampingStep | genericEUTRANNRMAttributeTypes:: powerRampingStepEnumType |
| qHyst | qHyst | genericEUTRANNRMAttributeTypes:: qHystEnumType |
| qOffsetUtra | qOffsetUtra | unsignedShort |
| qOffsetGeran | qOffsetGeran | unsignedShort |
| qOffsetCdma2000 | qOffsetCdma2000 | unsignedShort |
| qQualMinUtra | qQualMinUtra | unsignedShort |
| qRxLevMinEUtraSib1 | qRxLevMinEUtraSib1 | short |
| qRxLevMinEUtraSib3 | qRxLevMinEUtraSib3 | short |
| qRxLevMinGeran | qRxLevMinGeran | unsignedShort |
| qRxLevMinUtra | qRxLevMinUtra | short |
| responseWindowSize | responseWindowSize | genericEUTRANNRMAttributeTypes:: responseWindowSizeEnumType |
| rootSequenceIndex | rootSequenceIndex | unsignedShort |
| sIntraSearch | sIntraSearch | unsignedShort |
| sizeOfRAPreamblesGroupA | sizeOfRAPreamblesGroupA | genericEUTRANNRMAttributeTypes:: sizeOfRAPreambleGroupAEnumType |
| timeToTriggerEutraA1 | timeToTriggerEutraA1 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA2 | timeToTriggerEutraA2 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA3 | timeToTriggerEutraA3 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA4 | timeToTriggerEutraA4 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerEutraA5 | timeToTriggerEutraA5 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerIratB1 | timeToTriggerIratB1 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| timeToTriggerIratB2 | timeToTriggerIratB2 | genericEUTRANNRMAttributeTypes:: timeToTriggerEUtraEnumType |
| tReselectionCdma2000 | tReselectionCdma2000 | unsignedShort |
| tReselectionEUtra | tReselectionEUtra | unsignedShort |
| tReselectionGeran | tReselectionGeran | unsignedShort |
| tReselectionUtra | tReselectionUtra | unsignedShort |
| tStoreUeContext | tStoreUeContext | unsignedShort |
| Note: For all conditional qualifiers, see attribute constraints in 28.658 [4] |  |  |

### A.2.2.20	IOC WTFunction

Mapping from NRM IOC WTFunction attributes and associations to SS equivalent MOC WTFunction attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| wLANInfoList | wLANInfoList | GenericNetworkResourcesIRPSystem:: AttributeTypes::WLANInfoListType |

### A.2.2.21	IOC EP_Xw

None.

### A.2.2.22	IOC WLANMobilitySet

Mapping from NRM IOC WLANMobility attributes and associations to SS equivalent MOC WLANMobilitySet attributes

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |

### A.2.2.x	IOC Link_MCE_ENB

### None.

### A.2.2.y	IOC Link_MCE_MME

None.

### A.2.2.z	IOC EUtranFreqRelation

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| cellIndividualOffset | cellIndividualOffset | genericEUTRANNRMAttributeTypes:: qOffsetEnumType |
| blackListEntry | blackListEntry | unsignedLong |
| blackListEntryIdleMode | blackListEntryIdleMode | unsignedLong |
| cellReselectionPriority | cellReselectionPriority | unsignedLong |
| cellReselectionSubPriority | cellReselectionSubPriority | unsignedShort |
| pMax | pMax | unsignedLong |
| qOffsetFreq | qOffsetFreq | unsignedLong |
| qQualMin | qQualMin | unsignedLong |
| qRxLevMin | qRxLevMin | unsignedLong |
| threshXHighP | threshXHighP | unsignedLong |
| threshXHighQ | threshXHighQ | unsignedLong |
| threshXLowP | threshXLowP | unsignedLong |
| threshXLowQ | threshXLowQ | unsignedLong |
| tReselectionEutra | tReselectionEutra | unsignedShort |
| tReselectionEutraSfHigh | tReselectionEutraSfHigh | unsignedShort |
| tReselectionEutraSfMedium | tReselectionEutraSfMedium | unsignedShort |
| eUtraFrequencyRef | eUtraFrequencyRef | string |

### A.2.2.z1	IOC EUtranFrequency

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| earfcnDL | earfcnDL | unsignedLong |
| multiBandInfoListEutra | multiBandInfoListEutra | unsignedLong |

Next modification

## A.3.2	IDL specification "EUtranNetworkResourcesNRMDefs.idl"

//File:EUtranNetworkResourcesNRMDefs.idl

#ifndef _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#define _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#include "EPCResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module EUtranNetworkResourcesNRMDefs

{

/*

* Definitions for MO class ENBFunction

*/

interface ENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ENBFunction";

// Attribute Names

//

const string intraANRSwitch= "intraANRSwitch";

const string iRATANRSwitch= "iRATANRSwitch";

const string eNBId = "eNBId";

const string x2BlackList= "x2BlackList";

const string x2WhiteList= "x2WhiteList";

const string x2HOBlackList= "x2HOBlackList";

const string x2IpAddressList= "x2IpAddressList";

const string tceIDMappingInfoList= "tceIDMappingInfoList";

const string sharNetTceMappingInfoList= "sharNetTceMappingInfoList";

const string netListeningRSForRIBS= "netListeningRSForRIBS";

const string lWIPSeGWList = "lWIPSeGWList";

};

/*

* Definitions for MO class RNFunction

*/

interface RNFunction: ENBFunction

{

const string CLASS = "RNFunction";

// Attribute Names

//

const string servingCell = "servingCell";

const string candidateDeNBCells = "candidateDeNBCells";

};

/*

* Definitions for MO class DeNBCapability

*/

interface DeNBCapability: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "DeNBCapability";

// Attribute Names

//

const string servedRN= "servedRN";

const string maxNbrRNAllowed= "maxNbrRNAllowed";

};

/*

* Definitions for MO class ExternalRNFunction

*/

interface ExternalRNFunction: ExternalENBFunction

{

const string CLASS = "ExternalRNFunction";

// Attribute Names

//

};

/*

/*

* Definitions for MO class EUtranGenericCell

*/

interface EUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranGenericCell";

// Attribute Names

//

const string cellLocalId = "cellLocalId";

const string cellLocalIdList = "cellLocalIdList";

const string cellSize = "cellSize";

const string plmnIdList = "plmnIdList";

const string cellAccessInfoList = "cellAccessInfoList";

const string tac = "tac";

const string pci = "pci";

const string pciList = "pciList";

const string operationalState = "operationalState";

const string administrativeState = "administrativeState";

const string availabilityStatus = "availabilityStatus";

const string maximumTransmissionPower = "maximumTransmissionPower";

const string referenceSignalPower = "referenceSignalPower";

const string pb = "pb";

const string partOfSectorPower = "partOfSectorPower";

const string relatedTmaList = "relatedTmaList";

const string relatedAntennaList = "relatedAntennaList";

const string relatedSector = "relatedSector";

const string allowedAccessClasses = "allowedAccessClasses";

const string isChangeForEnergySavingAllowed = "isChangeForEnergySavingAllowed";

const string cellResvInfo = "cellResvInfo";

const string nbIoTcellFlag = "nbIoTcellFlag";

};

/*

* Definitions for MO class ExternalEUtranGenericCell

*/

interface ExternalEUtranGenericCell: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalEUtranGenericCell";

// Attribute Names

//

const string pci= "pci";

const string plmnIdList = "plmnIdList";

const string cellLocalId = "cellLocalId";

const string eNBId = "eNBId";

};

/*

* Definitions for MO class EUtranCellFDD

*/

interface EUtranCellFDD: EUtranGenericCell

{

const string CLASS = "EUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class ExternalEUtranCellFDD

*/

interface ExternalEUtranCellFDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellFDD";

// Attribute Names

//

const string earfcnDl = "earfcnDl";

const string earfcnUl = "earfcnUl";

};

/*

* Definitions for MO class EUtranCellTDD

*/

interface EUtranCellTDD: EUtranGenericCell

{

const string CLASS = "EUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

const string sfAssignment = "sfAssignment";

const string specialSfPatterns = "specialSfPatterns";

};

/*

* Definitions for MO class ExternalEUtranCellTDD

*/

interface ExternalEUtranCellTDD: ExternalEUtranGenericCell

{

const string CLASS = "ExternalEUtranCellTDD";

// Attribute Names

//

const string earfcn = "earfcn";

};

/*

* Definitions for MO class EUtranRelation

*/

interface EUtranRelation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranRelation";

// Attribute Names

//

const string id = "id";

const string tCI = "tCI";

const string isRemoveAllowed = "isRemoveAllowed";

const string isHOAllowed = "isHOAllowed";

const string adjacentCell = "adjacentCell";

const string isICICInformationSendAllowed = "isICICInformationSendAllowed";

const string isLBAllowed = "isLBAllowed";

const string cellIndividualOffset = "cellIndividualOffset";

const string qOffset = "qOffset";

};

/*

* Definitions for MO class Link_ENB_ENB

*/

interface Link_ENB_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_ENB_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Cdma2000Relation

*/

interface Cdma2000Relation:GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "Cdma2000Relation";

// Attribute Names

//

const string id = "id";

const string adjacentSector = "adjacentSector";

};

/*

* Definitions for MO class ExternalENBFunction

*/

interface ExternalENBFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ExternalENBFunction";

// Attribute Names

//

const string eNBId = "eNBId";

};

/*

* Definitions for MO class MCEFunction

*/

interface MCEFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MCEFunction";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_ENB

*/

interface Link_MCE_ENB: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_ENB";

// Attribute Names

//

};

/*

* Definitions for MO class Link_MCE_MME

*/

interface Link_MCE_MME: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_MCE_MME";

// Attribute Names

//

};

/*

* Definitions for MO class MBSFNArea

*/

interface MBSFNArea: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "MBSFNArea";

// Attribute Names

//

const string id = "id";

const string mbsfnAreaId= "mbsfnAreaId";

const string cellIdList= "cellIdList";

};

/*

* Definitions for MO class EnergySavingProperties

*/

interface EnergySavingProperties: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EnergySavingProperties";

// Attribute Names

//

const string id = "id";

const string energySavingState= "energySavingState";

const string energySavingControl= "energySavingControl";

};

/*

* Definitions for MO class CellOutageCompensationInformation

*/

interface CellOutageCompensationInformation: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "CellOutageCompensationInformation";

// Attribute Names

//

const string id = "id";

const string cellOutageCompensationStatus = "cellOutageCompensationStatus";

const string isCOCAllowed =

"isCOCAllowed";

};

/*

* Definitions for MO class EUtranCellNMCentralizedSON

*/

interface EUtranCellNMCentralizedSON: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EUtranCellNMCentralizedSON";

// Attribute Names

//

const string id = "id";

const string a1ThresholdRsrp = "a1ThresholdRsrp";

const string a1ThresholdRsrq = "a1ThresholdRsrq";

const string a2ThresholdRsrp = "a2ThresholdRsrp";

const string a2ThresholdRsrq = "a2ThresholdRsrq";

const string a3Offset = "a3Offset";

const string a4ThresholdRsrp = "a4ThresholdRsrp";

const string a4ThresholdRsrq = "a4ThresholdRsrq";

const string a5Threshold1Rsrp = "a5Threshold1Rsrp";

const string a5Threshold1Rsrq = "a5Threshold1Rsrq";

const string b1ThresholdUtraRscp = "b1ThresholdUtraRscp";

const string b1ThresholdUtraEcN0 = "b1ThresholdUtraEcN0";

const string b1ThresholdGeran = "b1ThresholdGeran";

const string b1ThresholdCdma2000 = "b1ThresholdCdma2000";

const string b2Threshold1Rsrp = "b2Threshold1Rsrp";

const string b2Threshold1Rsrq = "b2Threshold1Rsrq";

const string b2Threshold2UtraRscp = "b2Threshold2UtraRscp";

const string b2Threshold2UtraEcN0 = "b2Threshold2UtraEcN0";

const string b2Threshold2Geran = "b2Threshold2Geran";

const string b2Threshold2Cdma2000 = "b2Threshold2Cdma2000";

const string commonChannelPowerOffset = "commonChannelPowerOffset";

const string configurationIndex = "configurationIndex";

const string contentionResolutionTimer = "contentionResolutionTimer";

const string hysteresisEutraA1 = "hysteresisEutraA1";

const string hysteresisEutraA2 = "hysteresisEutraA2";

const string hysteresisEutraA3 = "hysteresisEutraA3";

const string hysteresisEutraA4 = "hysteresisEutraA4";

const string hysteresisEutraA5 = "hysteresisEutraA5";

const string hysteresisIratB1 = "hysteresisIratB1";

const string hysteresisIratB2 = "hysteresisIratB2";

const string numberOfRaPreambles = "numberOfRaPreambles";

const string preambleInitialReceivedTargetPower = "preambleInitialReceivedTargetPower";

const string preambleTransMax = "preambleTransMax";

const string pMax = "pMax";

const string powerRampingStep = "powerRampingStep";

const string qHyst = "qHyst";

const string qOffsetUtra = "qOffsetUtra";

const string qOffsetGeran = " qOffsetGeran";

const string qOffsetCdma2000 = "qOffsetCdma2000";

const string qQualMinUtra = "qQualMinUtra";

const string qRxLevMinEUtraSib1 = "qRxLevMinEUtraSib1";

const string qRxLevMinEUtraSib3 = "qRxLevMinEUtraSib3";

const string qRxLevMinGeran = "qRxLevMinGeran";

const string qRxLevMinUtra = "qRxLevMinUtra";

const string responseWindowSize = "responseWindowSize";

const string rootSequenceIndex = "rootSequenceIndex";

const string sIntraSearch = "sIntraSearch";

const string sizeOfRAPreamblesGroupA = "sizeOfRAPreamblesGroupA";

const string timeToTriggerEutraA1 = "timeToTriggerEutraA1";

const string timeToTriggerEutraA2 = "timeToTriggerEutraA2";

const string timeToTriggerEutraA3 = "timeToTriggerEutraA3";

const string timeToTriggerEutraA4 = "timeToTriggerEutraA4";

const string timeToTriggerEutraA5 = "timeToTriggerEutraA5";

const string timeToTriggerIratB1 = "timeToTriggerIratB1";

const string timeToTriggerIratB2 = "timeToTriggerIratB2";

const string tReselectionCdma2000 = "tReselectionCdma2000";

const string tReselectionEUtra = "tReselectionEUtra";

const string tReselectionGeran = "tReselectionGeran";

const string tReselectionUtra = "tReselectionUtra";

const string tStoreUeContext = "tStoreUeContext";

};

/*

* Definitions for MO class EUtranFreqRelation

*/

interface EUtranFreqRelation: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranFreqRelation";

// Attribute Names

//

const string cellIndividualOffset = "cellIndividualOffset";

const string blackListEntry = "blackListEntry";

const string blackListEntryIdleMode = "blackListEntryIdleMode";

const string cellReselectionPriority = "cellReselectionPriority";

const string cellReselectionSubPriority = "cellReselectionSubPriority";

const string pMax = "pMax";

const string qOffsetFreq = "qOffsetFreq";

const string qQualMin = "qQualMin";

const string qRxLevMin = "qRxLevMin";

const string threshXHighP = "threshXHighP";

const string threshXHighQ = "threshXHighQ";

const string threshXLowP = "threshXLowP";

const string threshXLowQ = "threshXLowQ";

const string tReselectionEutra = "tReselectionEutra";

const string tReselectionEutraSfHigh = "tReselectionEutraSfHigh";

const string tReselectionEutraSfMedium = "tReselectionEutraSfMedium";

const string eUtraFrequencyRef = "eUtraFrequencyRef";

};

/*

* Definitions for MO class EUtranFrequency

*/

interface EUtranFrequency: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EUtranFrequency";

// Attribute Names

//

const string earfcnDL = "earfcnDL";

const string multiBandInfoListEutra = "multiBandInfoListEutra";

};

/*

* Definitions for MO class WTFunction

*/

interface WTFunction: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "WTFunction";

// Attribute Names

//

const string id = "id";

const string wLANInfoList = "wLANInfoList";

};

/*

* Definitions for MO class EP_Xw

*/

interface EP_Xw: GenericNetworkResourcesNRMDefs::Top

{

const string CLASS = "EP_Xw";

};

/*

* Definitions for MO class WLANMobilitySet

*/

interface WLANMobilitySet: GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "WLANMobilitySet";

// Attribute Names

//

};

/*

* Definitions for MO class MemberWLAN

*/

interface MemberWLAN

{

const string CLASS = "MemberWLAN";

// Attribute Names

//

const string wLANId = "memberWlanList";

const string wLANGeoLocation = "wLANGeoLocation";

const string isLWASupported = "isLWASupported";

const string isLWIPSupported = "isLWIPSupported";

};

module genericEUTRANNRMAttributeTypes

{

typedef sequence<string> cellLocalIdListType;

enum CellSizeEnumType

{

verysmall,

small,

medium,

large

};

enum AllowedAccessClassesValues

{

EmergencyCall,

ForPLMNUse,

SecurityServices,

PublicUtilities,

EmergencyServices,

PLMNStaff

};

typedef sequence < AllowedAccessClassesValues,6> AllowedAccessClasses

struct PlmnIdType

{

// A bounded string of 3 characters representing 3 digits.

string mcc;

// A bounded string of 2 or 3 characters representing 2 or 3 digits.

string mnc;

};

const short PLMNID_LIST_LENGTH = 6;

typedef sequence<PlmnIdType > plmnIdListType;

struct CellAccessInfoType

{

PlmnIdType plmnId;

long tac;

unsigned short cellId;

string managementServiceExposed;

};

typedef sequence <CellAcessInfoType, 5> cellAcessInfoListType;

const short NO_OF_PCIS = 504;

typedef sequence<short,NO_OF_PCIS> pciListType;

typedef sequence<string> ipAddressListType;

enum CellResvInfoType

{

reservedCell,

nonReservedCell

};

struct QciDscpMappingType

{

short qci;

short dscp;

};

typedef sequence<QciDscpMappingType> QciDscpMappingListType;

struct EcgiType

{

// A bounded string of 3 characters representing 3 digits.

string mcc;

// A bounded string of 2 or 3 characters representing 2 or 3 digits.

string mnc;

unsignedlong eci

};

typedef sequence <EcgiType> EcgiListType;

enum isEsCoveredByEnumType

{

no,

partial,

yes

};

enum yesNoType

{

no,

yes

};

struct TceIDMappingInfo

{

short tceID;

string tceIPAddr;

};

typedef sequence<TceIDMappingInfo> TceIDMappingInfoListType;

struct SharNetTceMappingInfo

{

PlmnIdType plmnId;

short tceID;

string tceIPAddr;

};

typedef sequence<SharNetTceMappingInfo> SharNetTceMappingInfoListType;

enum CellOutageCompensationState

{

cOCActivating,

cOCActive,

cOCDeactivating,

cOCDeactive

};

enum qOffsetEnumType

{

dB-24,

dB-22,

dB-20,

dB-18,

dB-16,

dB-14,

dB-12,

dB-10,

dB-8,

dB-6,

dB-5,

dB-4,

dB-3,

dB-2,

dB-1,

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum contentionResolutionTimerEnumType

{

sf8,

sf16,

sf24,

sf32,

sf40,

sf48,

sf56,

sf64

};

enum numberOfRaPreamblesEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

n64

};

enum preambleInitialReceivedTargetPowerEnumType

{

dBm-120,

dBm-118,

dBm-116,

dBm-114,

dBm-112,

dBm-110,

dBm-108,

dBm-106,

dBm-104,

dBm-102,

dBm-100,

dBm-98,

dBm-96,

dBm-94,

dBm-92,

dBm-90

};

enum preambleTransMaxEnumType

{

n3,

n4,

n5,

n6,

n7,

n8,

n10,

n20,

n50,

n100,

n200

};

enum powerRampingStepEnumType

{

dB0,

dB2,

dB4,

dB6

};

enum qHystEnumType

{

dB0,

dB1,

dB2,

dB3,

dB4,

dB5,

dB6,

dB8,

dB10,

dB12,

dB14,

dB16,

dB18,

dB20,

dB22,

dB24

};

enum responseWindowSizeEnumType

{

sf2,

sf3,

sf4,

sf5,

sf6,

sf7,

sf8,

sf10

};

enum sizeOfRAPreambleGroupAEnumType

{

n4,

n8,

n12,

n16,

n20,

n24,

n28,

n32,

n36,

n40,

n44,

n48,

n52,

n56,

n60,

};

enum timeToTriggerEUtraEnumType

{

ms0,

ms40,

ms64,

ms80,

ms100,

ms128,

ms160,

ms256,

ms320,

ms480,

ms512,

ms640,

ms1024,

ms1280,

ms2560,

ms5120

};

typedef sequence<string> DnList;

struct CellOutageCompensationStatus

{

CellOutageCompensationState cellOutageCompensationState;

DnList errorList;

};

struct ReferenceSignalPattern

{

yesNoType prs_supported;

};

enum ReferenceSignalPeriodicity

{

ms1280,

ms2560,

ms5120,

ms10240

};

const short MAX_CONFIGURATION_NO = 4;

struct NetListeningRefSignalType

{

ReferenceSignalPattern referenceSignalPattern;

short num_of_crs_ports;

ReferenceSignalPeriodicity referenceSignalPeriodicity;

unsigned short referenceSignalOffset;

};

typedef sequence<NetListeningRefSignalType, MAX_CONFIGURATION_NO> NetListeningRSForRIBS;

struct WLANInfoType

{

string WLANId;

enum WLANOperationalState

{

Disabled, Enabled

};

string EnbWithLWARelation;

};

typedef sequence<WLANInfoType> WLANInfoListType;

struct GeoLocationType

{

short latitude;

short longitude

};

struct lWIPSeGWInfoType

{

string LWIPSeGWId;

ipAddressListType LWIPSeGWIpAddressList;

};

typedef sequence<lWIPSeGWInfoType> lWIPSeGWListType;

};

#endif // _EUTRANNETWORKRESOURCESNRMDEFS_IDL_

End of modification