3GPP TSG-<SA5> Meeting # 127	S5-196859

# Sophia Antipolis, France,  14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.511 | CR | 0026 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E-UTRAN ANR Specification level requirements for NR support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2019-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Management of E-UTRAN lacks support for Neighbor Cell Relations to NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add specification level requirements to allow management of E-UTRAN to support Neighbor Cell Relations from E-UTRAN to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management of E-UTRAN will not be able to observe or configure Neighbor Cell Relations from E-UTRAN to NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | n | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.1	Business level requirements

REQ-ANR-CON-001	NCRs shall be set up and optimized with no or minimal human intervention.

REQ-ANR-CON-002	For E-UTRAN, initial status of the newly created NCR by ANR function shall be such that HO is allowed, X2 connection setup is allowed, and the NCR is allowed to be removed by ANR function in eNB.

REQ-ANR-CON-003	E-UTRAN ANR supports management of NCRs from E-UTRAN to E-UTRAN, from E-UTRAN to UTRAN, from E-UTRAN to CDMA2000, from E-UTRAN to GERAN and from E-UTRAN to NG-RAN.

REQ-ANR-CON-004	For UTRAN, initial status of the newly created NCR by ANR function shall be such that HO is allowed and the NCR is allowed to be removed by ANR function in RNC.

REQ-ANR-CON-005	UTRAN ANR supports management of NCRs from UTRAN to UTRAN, from UTRAN to E-UTRAN and from UTRAN to GERAN.

Next change

#### 5.2.5.x	ANR function management for NG-RAN

The business level requirements in section 5.1 are decomposed into the following specification level requirements, applicable for NG-RAN:

REQ-ANR -FUN-x1 The IRPAgent shall support a capability allowing the IRPManager to request that HO is allowed from source E-UTRAN cell to target NG-RAN cell.

REQ-ANR-FUN-x2 The IRPAgent shall support a capability allowing the IRPManager to request that HO is prohibited from source E-UTRAN cell to target NG-RAN cell.

REQ-ANR-FUN-x3	The IRPAgent shall support a capability allowing the IRPManager to request that HO is allowed from source E-UTRAN cell to target NG-RAN cell and that no other entity than an IRPAgent can remove that request. This is termed as HO white-listing.

REQ-ANR-FUN-x4	The IRPAgent shall support a capability allowing the IRPManager to request that HO is prohibited from source E-UTRAN cell to target NG-RAN cell and that no other entity than an IRPAgent can remove that request. This is termed as HO black-listing.

End of changes