TSG SA Meeting #SP-86  	SP-191234

# 11 - 13 December 2019, Sitges, Spain	(revision of S2-1912045)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1848 | rev | 9 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Inter PLMN UP functionality in the architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia?, Nokia Shanghai Bell?], Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-12-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S3-193096/S2-1908713 “LS to SA2 on UP gateway function” Rev8 (in yellow highlight color) At the S3#97 meeting, SA3 has agreed that the IPUPS is the transparent proxy as described in the agreed CR(S3-194443). It is proposed to align that conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe inter PLMN User Plane security at PLMN borders |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing description of inter PLMN User Plane security at PLMN borders |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X; 6.2.3 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Annex X (informative):
Inter PLMN User Plane security at the PLMN border

Inter PLMN User Plane security at the PLMN border corresponds to following features:

- For N9 traffic received from outside of the PLMN: forward valid GTP-U traffic and discard the remaining, non-valid GTP-U traffic.

Such features are further specified in TS 33.501 [29].

Operators can deploy UPF(s) supporting inter PLMN User Plane security function, i.e. IPUPS, at the border of their network. Alternatively, for this purpose, Operators can deploy dedicated standalone capabilities that have access to the N4 signalling exchanged by a UPF and thus can enforce security on inter PLMN N9 traffic that this UPF receives. How these capabilities are inserted over N4 and N9 is out of the scope of this specification but has no impact on 5GC signalling.

The interaction between the SMF and UPF including the inter PLMN User Plane security function is shown as below:

vSMF

UPF

IPUPS

IPUPS

UPF

UPF    +

IPUPS

hSMF

vSMF

hSMF

UPF     +    IPUPS

N9

N9

N4

N4

N4

N4

Collocated UPF+IPUPS

Standalone IPUPS

:

Figure x-1: Interaction between the SMF and UPF including the inter PLMN User Plane security function

NEXT CHANGE (2)

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	TSN Translator functionality to hold and forward user plane packets for de-jittering when 5G System is integrated as a bridge with the TSN network.

-	High latency communication, see clause 5.31.8.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

- Traffic filtering used e.g. for inter PLMN N9 interface.

END OF CHANGES