SA WG2 Meeting #101	S2-140193

# 20 - 24 January 2014, Taipei, Taiwan	(revision of S2-14xxxx)

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.402 | CR | 1234 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | WLANSP rules selection provided from two VPLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | WLAN_NS |  |  |  |  |  |  | Date: |  | 2014-01-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | “VPLMNs with preferred WLAN Selection Rules” is used by the roaming UE to determine whether or not to perfer the WLANSP rules provided by the VPLMN, however, the UE may simultaneously connect to two VPLMNs, in this case which WLANSP rules provided by the VPLMN should be selected is not specified. Consider the following example, a roaming UE is registered to VPLMN1 via 3GPP access and VPLMN1 is included in the “VPLMNs with preferred WLAN Selection Rules”, so the roaming UE uses WLANSP rules from VPLMN1 to select the most preferred WLAN and perfoms 3GPP-based access authentication, however, the PSPL is not set to "prefer 3GPP RPLMN" such that the UE selects another VPLMN (e.g. VPLMN2) which is included in the PSPL and receives WLANSP rules from VPLMN2. In this case, UE have two WLANSP rules, one from VPLMN1 and the other from VPLMN2, which one shall be used? |  |  |  |  |  |  |  |  |
| Summary of change: |  | Each VPLMN in the “VPLMNs with preferred WLAN Selection Rules” is assigned a priority to facilitate the UE to select the WLANSP rules in case the UE is connected to two VPLMNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WLANSP rules selection is not clear if the UE is connected to two VPLMNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

4.8.2.1.7	VPLMNs with preferred WLAN Selection Rules

The VPLMNs with preferred WLAN Selection Rules is a prioritized list of PLMNs that is used by the UE when roaming. When the UE is roaming to one of the PLMNs in the list, the UE is configured to prefer the WLANSP rules provided by this PLMN over the WLANSP rules provided by the HPLMN. When the UE is roaming to any other PLMN, the UE is configured to prefer the WLANSP rules provided by the HPLMN. In case the UE is simultaneously connected to two VPLMNs both via 3GPP access and WLAN access, the WLANSP rules provided by the VPLMN with the highest priority is selected.

How the UE uses this list of PLMNs for WLAN selection is specified in more detail in clause 4.8.2a.1.

END OF CHANGES