SA WG2 Meeting #106	S2-144141/4418

17 - 21 November 2014, San Francisco, California, USA	(revision of S2-14xxxx)

Source:	Samsung, ZTE

Title:	Correlation of IMS session between native IMS client vs. WIC

Document for:	Approval

Agenda Item:	7.12

Work Item / Release:	eWebRTCi / Rel-13

Abstract of the contribution: This paper discusses the issue with IMS session correlation between WIC and native IMS client from a common device.

1. Introduction

In Rel-7 when DR-VCC was introduced, domain selection was included to guide the UE whether voice call should be started with WiFi or CS RAT when UE has access to both RATs. Then in Rel-8 when CSFB was introduced, domain selection is further updated to provision the UE to which domain (CS or PS) a voice/sms service should be used. Overall domain selection requirement is described in TS 23.221.

WIC allows a UE to access IMS using WebRTC. UE may also have native IMS client accessing the IMS via Gm. It is possible that a UE having both the WIC and native IMS client are activated by the user in which two parallel IMS registrations become active (one via W2 and the other via Gm). For a terminating IMS service, T-ADS will have to determine which IMS access to try first if there is a choice of providing to the S-CSCF the eP-CSCF or P-CSCF contact address.

Similarly for the UE, it should also use the same IMS Access that T-ADS has selected as primary IMS access for terminating service delivery. Having multiple IMS sessions handled by the same client (whether IMS or WIC) can simplify the interaction handling within the UE (e.g., swapping between HELD and ACTIVE voice session).

2. Proposal

To add the following to TR 23.706.

*** begin next texts ***

# 4	Assumptions and architectural requirements

Editor’s Note: This section contains Assumptions and Architectural requirements that are valid for all features studied as part of this TR

-	If a UE performs multiple IMS registrations due to the device having multiple IMS clients (e.g.,. having both normal IMS client and WIC, etc), then these IMS registrations are assumed to be uncorrelated (i.e, not tied to the same IMS subscription).

*** end ***