SA WG2 Meeting #107	S2-150382

26 - 30 January 2015, Sorrento, Italy	(revision of S2-15xxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.060 | CR | 1955 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintaining dedicated CN nodes during RAU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DECOR |  |  |  |  |  |  | Date: |  | 2015-01-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | During SA2#106 meeting in San Francisco SA2 completed the DÉCOR study in TR23.707 and agreed the conclussions for the normative spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to apply the conclusions from DECOR TR23.707, s5.2.2.1.3 for maintaining the UEs in dedicated networks to the normative spec TS23.060. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | no support for dedicated core networks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS TR ….CR… |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************** start 1st change *****************

#### 5.3.7.3	SGSN selection function

The SGSN selection function selects an available SGSN to serve a UE. The selection is based on network topology, i.e. the selected SGSN serves the UE's location and in case of overlapping SGSN service areas, the selection may prefer SGSNs with service areas that reduce the probability of changing the SGSN. Other criteria for SGSN selection may be load balancing between SGSNs. In networks that deploy dedicated MMEs/SGSNs, e.g. for UEs configured for low access priority, the possible target SGSN selected by source MME/SGSN is typically restricted to SGSNs with the same dedication.

If dedicated core networks are supported, the RAN node shall have configuration organizing the NRIs of neighbouring and local SGSN pools into sets. Each set contains NRIs that arw uniquely associated with the same dedicated core network. To maintain the MS in the same dedicated core network when the MS enters a new SGSN pool area, the RAN node shall select an SGSN from the configured set for that same dedicated core network by associating the NRIs or MMEGIs of neighbouring pools of a particular core network to the NRIs of the local SGSN pool of the same dedicated core network. If RAN node finds no SGSN in the new Pool area that is associated with the same dedicated core networks the RAN node shall perform normal SGSN selection as described in this subclause.

***************** end 1st change *****************