SA WG2 Meeting #S2-108	S2-150713

13 - 17 April 2015, San Jose Del Cabo, Mexico

3GPP TSG CT3 Meeting #80	C3-150450

# Sorrento, Italy; 2nd – 6th February 2015

Title:	Reply LS on End-to-end QoS handling of MTSI

Response to:	LS (C3-1502554/S4-150163) on TR 26.924 Study on Improved e2e QoS from SA4

Work Item:	QOSE2EMTSI-CT

Source:	CT3

To:	SA4

Cc:	CT1, CT4, SA2

Contact Person:

#### Name:	Susana Fernández

Tel. Number:	+34 646006865

E-mail Address:	Susana (dot) Fernandez(at) ericsson (dot) com

1. Overall Description:

CT3 would like to thank SA4 for obtaining an early opportunity to comment the requirements for End-to-end QoS handling of MTSI suggested in SA4´s LS.

CT3 thanks SA4 for the LS on TR 26.924 Study on improved end-to-end QoS handling. CT3 already commented on clause 5 of that document and thanks SA4 for taking those comments into consideration. CT3 has now analysed some of the provided use cases from a stage 3 PCC perspective and would like to provide additional comments.

Although no specific solutions are provided in the Technical Report yet, some suggestions are mentioned for some use cases. CT3 would like to comment on the identified PCC impacts for the following aspects:

- Bandwidth negotiation in a second Offer-Answer

This second negotiation is considered relevant for those cases when multiple codecs or specific conditions for a codec are being negotiated. CT3 would like to comment that this solution would work according to current PCC procedures and no (or small) changes in the PCC specifications are expected. It would however have impacts in extra-signalling in the network as bearer procedures may be initiated with each interaction with the PCC architecture. Latency in the IMS session establishment or modification should also be considered. Additionally this second interaction would only be efficient when PCRF uses bitrate specific data for bandwidth derivation as there will be no modification in the resource reservation if the maximum bitrate is derived based on the codec information.

- Maximum supported bandwidth negotiated between the UEs for each direction

There is already a derivation of the MBR parameter from the SDP b:AS bandwidth modifier defined in PCC specification. CT3 is aware that the b:AS bandwidth modifier can indicate a higher bandwidth than actually required for media in the direction towards the offerer. CT3 would welcome clarifications how the proposed new maximum supported bandwidth relates to the B:AS bandwidth modifier, and if it would provide improved information about the bandwidth required for media in the direction towards the SDP offerer. If so that would allow the Application Function (AF) and PCRF to derive the MBR properly for both uplink and downlink directions in both local and remote networks. If the assumption is correct, the PCRF would not require codec specific algorithms for the bandwidth derivation.

- Minimum supported bandwidth negotiated between the UEs for each direction

CT3 assumes that the minimum supported bandwidth is not relevant for the PCRF and thus not used over the Rx interface.

For the handling in a MGW performing transcoding, the information is not required, as the MGW will also need to understand codec specific information.

- Minimum desired bandwidth negotiated between the UEs for each direction

CT3 assumes that the requirement to make the network aware of the minimum desired bandwidth will allow the AF and PCRF to derive the GBR properly for both uplink and downlink directions in both local and remote networks. If the assumption is correct, the PCRF would not require codec specific algorithms for the bandwidth derivation.

CT3 supports the approach about adding SDP attributes that help the derivation of the bandwidth in the PCRF as it would avoid the definition of algorithms in the PCRF based on currently used SDP attributes that will not ensure a proper derivation of the bandwidth in all cases.

CT3 realizes that the work is still ongoing and will wait for further progress before further commenting in these and other topics. Further analysis of those cases will proceed in future meetings.

2. Actions:

To  SA4 group.

ACTION: 	CT3 would like to ask SA4 to take this feedback of CT3 into account.

3. Date of Next TSG-CT3 Meetings:

CT3#81	25 - 29 May 2015	Sanya, CN

CT3#82	17 - 21 Aug 2015	Vancouver, CA