SA WG2 Meeting #108	S2-151230

13 - 17 April 2015, San Jose Del Cabo, Mexico

Source:	Acer Incorporated, ITRI

Title:	Issue for eDRX

Document for:	Approval

Agenda Item:	6.15

Work Item / Release:	FS_eDRX / Rel-13

Abstract of the contribution: This document proposes to add an issue to the TR for FS_eDRX

Introduction

Extending DRX cycle will affect idle mode UE measurement, and as a result will affect UE cell reselection. Purpose of cell reselection is to keep UE under service of a suitable cell, but eDRX mechanism is for scenarios in which UE sleeps longer to save battery, for example MTC. The intent of this PCR to propose to study the scenarios of cell reselection and measurement rules for extended DRX.

Some impacts on idle mode measurement when applying extended DRX cycle are identified as given below

- Serving cell measurement & evaluation

  - Because UE shall filter RSRP and RSRQ measurements using at least 2 measurements (shall be spaced by at least DRX cycle/2), the impact of Extended DRX cycle is that will cause too long space.

  - If UE has evaluated in Nserv consecutive DRX cycles that serving cell does not fulfil cell selection criterion S, it shall measure all indicated neighbour cells. The impact of Extended DRX cycle is that will cause Nserv consecutive cycles too long.

- Measurements of intra-frequency E-UTRAN cells

  - Because UE shall filter RSRP and RSRQ measurements of each measured intra-frequency cell using at least 2 measurements (shall be spaced by at least Tmeasure,EUTRAN_Intra/2), the impact of Extended DRX cycle is that will cause too long space.

  - For an intra-frequency cell that has been already detected, but that has not been reselected to, UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion within Tevaluate,E-UTRAN_intra when Treselection = 0. If Treselection timer has a non-zero value and the intra-frequency cell is better ranked than the serving cell, UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains better ranked within this duration, then UE shall reselect that cell. In the case of extended DRX, the processing time (Tevaluate,E-UTRAN_intra, Treselection ) should be adjusted.

Proposal

It is proposed to add the following to the TR for eDRX.

START CHANGES

4.3	Issues for RAN/GERAN

Editor’s Note: This clause will list issues and questions which require feedback/guidance from GERAN/RAN.

Issue 1:

-	Given a maximum response time desired by UE/applications, how to derive the eDRX value taking RAN/GERAN aspects into account (e.g. possible paging misses due to lack of SFN or frame timing alignment between cells, etc.).

Issue 2:

-	Impact on idle mode UE measurement and cell reselection.

END CHANGES