SA WG2 Meeting #115	S2-162952

23 - 27 May 2016, Nanjing, P.R. China	(revision of S2-162692, S2-161809)

Source:	Deutsche Telekom AG, Qualcomm Incorporated

Title:	Proposal to update key issue 16 on 3GPP system aspects to integrate private networks

Document for:	Approval

Agenda Item:	6.10.0

Work Item / Release:	FS_NextGen / Rel-14

Abstract of the contribution: This contribution proposes to update key issue 16 on 3GPP system aspects to integrate private networks.

# Discussion

The NextGen Architecture should be albe to strech across public and private networks, rather than ending in the single device (e.g. CPE, Relay UE) behind the access. I.e. devices (Remote Devicess) within the private network should become “visible” to the core network.

In today’s environments the CPE belongs to the customer and is a part of the customer’s private network. So it automatically presents a demarcation point or a boundary between the private network and the public network. Similar situation exist with Remote Devices behind Relay UEs.

In future this will change to a model that is able to “integrate” the private networks into the public network and realize new use cases for Home Network, Enterprise networks, Industry e.g. managed WLAN, extended Smarthome, etc. “Integration” foresees a higher degree and well defined interworking between public and private networks, going beyond just being connected. Integration allows the operator to take influence on the behavior of the private network if so desired by the private network owner.

Several different cases exist that should be studied (see also figure below):

- Remote Devices (e.g.Remote UEs) connected via Relay UE to the Core Network where the link between Remote Devices and Relay UEs may based on Bluetooth, WLAN, Zigbee etc in addition 3GPP based protocols

- When a device is added to the private network, how and what would need to be advertised (registered) to the Core Network in order to make them addressable in a secure manner?

- Relay UE’s connected to the Core Network based on WLAN or Fixed access in addition to 3GPP access, .i.e. including CPEs.

- Different scenarios depending on the capabilities of the Remote Devices should be considered spanning from fully “capable" smart phones to very constraint devices such as sensors:

- Smart phones would have full capabilities, i.e. handover and session continuity from indirect to direct connection and vice versa would be possible

- Constraint Devices, e.g. like sensors might not even have the 3GPP capabilities for communication and security. In this case the Relay UE would take care of that.

- Mobility, including mobility from fixed to mobile and vice versa, of the whole private network, i.e. when Remote Devices would move together with the Relay UE.

- How the Relay UE may support Remote Devices to communicate amongst each other, e.g. via different protocols

Remote Device4

Remote Device2

Remote Device1

Remote UE3

RAN

Core Network

Fixed

Access

Relay UE1

Relay UE2

BT

WLAN

5G

WLAN

Relevant Smarter requirements are listed below:

## From  NEO TR:

## 5.6	Access

…….

3GPP/Non-3GPP Access Integration

The 5G system shall be able to provide 3GPP services to the UE using various 3GPP and non-3GPP access networks (e.g. WLAN, Fixed broadband access, Bluetooth, etc).

The 5G system shall be able to support:

- Inter-system mobility between 3GPP and non-3GPP access networks,

- Capability for the UE to select the 3GPP or non-3GPP access based on operator policy,

- Simultaneous connection to different accesses, & capability for the UE to access the 3GPP services provided by the 5G network using non-3GPP access e.g. FMSS…,

- Authentication to access 5 G network through a non-3GPP access using 3GPP credentials.

….

The 5G system shall enable the UE to simultaneously connect to the network via different accesses, both non-3GPP and 3GPP.

The 5G system shall be able to manage the addition or dropping of the various accesses dynamically during the session according to the quality of the individual connections.

For UEs simultaneously connected to the network via different access (both non-3GPP and 3GPP), the network shall support data transmissions that leverage these accesses.

The 5G system shall be able to support the interworking with fixed broadband network.

NOTE:	The specification of fixed broadband access network is outside the scope of 3GPP.Which evolution of fixed broadband access network architecture needs to be considered in stage 2.

….

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic from 3GPP RAT to non-3GPP RATs and vice versa, taking into account traffic load.

The 3GPP network shall be able to integrate fixed and wireless access management and provide an efficient provision of services over 3GPP and non-3GPP accesses.

[PR.5.1.3.5-002] The 3GPP system shall be able to support enhanced authentication and authorization mechanisms to support various types of connectivity (e.g., subscribed, connectivity), with or without the presence of operator credentials in the device.

The 3GPP system owned by a 3rd party (e.g., factory owner) shall support network access using  identities, credentials, and authentication that is provided and managed by that 3rd party.

The 3GPP system shall support an authentication process that can handle alternative authentication methods with different types of credentials to allow for different deployment scenarios such as industrial factory automation.

From mIOT TR:

### 5.2.3	Potential requirements

Requirements below are for all kinds of connectivity aspects, including devices which support both direct and indirect 3GPP connection mode and devices which support only indirect 3GPP connection mode, using 3GPP RAT or non-3GPP RAT, in a licensed band or an unlicensed band:

-	[PR.5.2.3-001] The 3GPP system shall support a device to access to the 3GPP network in direct 3GPP connection mode.

…

-	[PR.5.2.3-005] Devices in direct device communication using 3GPP RAT shall have an established trusted relationship with each other.

-	[PR.5.2.3-006] Devices in direct device communication using non-3GPP RAT may use 3GPP credentials to establish a trusted relationship with each other.

Editor’s Note: The definition of trusted relationship need to be further studied.

….

-	[PR.5.2.3-008] A device which accesses the 3GPP network in direct 3GPP connection mode or in indirect 3GPP connection mode using a 3GPP RAT or a non-3GPP RAT, shall have a 3GPP subscription.

-	[PR.5.2.3-009] The 3GPP system shall support real time services (e.g. real time voice and/or real time video) for a device, whether the device is in indirect 3GPP connection mode or in direct 3GPP connection mode.

-	[PR.5.2.3-010] The 3GPP system shall support non-real time services for a device, whether the device is in indirect 3GPP connection mode or in direct 3GPP connection.

-….

-	[PR.5.2.3-012] The 3GPP system shall support service continuity for a device, when the device changes from a direct 3GPP connection to an indirect 3GPP connection and vice versa.

-	….

-	[PR.5.2.3-015] The 3GPP system shall identify, address and reach a device, when the device is in indirect 3GPP connection mode.

-	[PR.5.2.3-016] The 3GPP system shall support end to end QoS for a device, when the device is in indirect 3GPP connection mode.

-...-	[PR.5.2.3-018] The 3GPP system shall support selection of different connectivity aspects (e.g. direct device connection, direct/indirect 3GPP connection) to be used for a device.

# Proposal

It is proposed to update key issue 16 in order to capture what should be studied in addition.

* * * Start of change * * * *

## 5.16	Key Issue 16: 3GPP system aspects to support the connectivity of remote Devices via a relay UE

### 5.16.1	Description

This key issue will study:

-	Definition of a relay UE in the NextGen framework.

-	Definition of remote Devices in the Next Gen framework.

-	The effects of a relay UE on the mobility framework as well as session management and session continuity.

-	The support of remote Devices with different capabilities (e.g. no mobility management support, or no session management support, or using a non-3GPP access for the indirect 3GPP connection, etc.)

-	Which functions are further required/enhanced comparing to Rel-13 relay UE functionality.

-	If there is need for selection criteria based on operator policy between the direct connectivity to the network and the connectivity via a relay UE to provide efficient service.

-	Efficient session and/or service continuity mechanism between:

-	the direct connectivity to the network and the connectivity via a relay UE, and

-	the connectivity via a current relay UE and the connectivity via a new relay UE with minimizing the interrupt time for the service and signalling overhead.

-	How to enable flexible transport of traffic between the remote Devices and the core network, e.g. switch between direct and indirect mode or transfer traffic between private network behind the relay UE and the core network

-	The security and privacy aspects (e.g. how to establish the connectivity via a UE relay when there is no trusted relationship between the Remote Device and the relay UE), including authentication of remote Devices (with or without operator credentials);

-	Identification and management by the CN of Remote Devices

-	Control by the MNO of the connectivity via a relay UE.

-	Impact on QoS support for Remote Devices

Additional inputs from SA WG1 will be considered for possible enhancement to a relay UE.

* * * End of changes * * * *