SA WG2 Meeting #123	S2-177820

October 23 – 27, 2017, Ljubljana, Slovenia	(was S2-177325)

Source:	Cisco

Title:	23.502: Time zone change reporting as Event Notification by AMF

Document for:	Discussion

Agenda Item:	6.5.3

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Adds time zone change reporting as an event notification from AMF.

The list of events that AMF can report as events as captured in 23.502 does not include Time-zone change.

Time-zone change is an event that PCF, NEF, SMF or AF (eg. P-CSCF) can subscribe to.

## 4. Proposed changes to specification

************************* 23.502 FIRST CHANGE **********************

#### 5.2.2.3	Namf_EventExposure service

##### 5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an event.

Following UE mobility information event are considered:

-	Location changes (TAI, Cell ID, N3IWF node, UE local IP address and optionally UDP source port number);

-	Time zone changes (UE Time zone)

-	AN type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status along with optional list of sessions to be activated;

-	Mobility statistics and

-	UE indication of switching off SMS over NAS service.

As well as monitoring events, as described in clause 5.4.2.

The following service operations are defined for the Namf_EventExposure service.

************************* 23.502 END CHANGE **********************