SA WG2 Meeting #123	S2-177676

23 - 27 October 2017, Ljubljana, Slovenia 	(was S2-177304)

Source:	China Mobile, CATT

Title:	OI#27 TS 23.502 EPS bearer ID allocation procedure

Document for:	Discussion / Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes the procedure of allocating EPS bearer ID to a QoS flow(s).

- Discussion

As discussed in last SA2 meeting, the procedure for EPS bearer ID allocation should be introduced as a separated clause. Therefore, a new sub clause is proposed to capture the agreement achieved in the last meeting on EPS bearer ID allocation.

2.	Proposal

It is propose to include the following changes into the TS 23.502.

* * * * First Change * * * *

4.11.1	N26 based Interworking Procedures

4.11.1.1	General

N26 interface is used to provide seamless session continuity for single registration mode.

4.11.1.x	Procedures for EPS bear ID allocation

Following procedures are updated to allocate EPS bearer ID(s) towards EPS bearer(s) mapped from QoS flow(s):

-	Non-roaming and Roaming with Local Breakout (clause 4.3.2.2.1)

-	Home-routed Roaming (clause 4.3.2.2.2)

-	UE or network requested PDU session modification (non-roaming and roaming with local breakout) (clause 4.3.3.2)

-	UE or network requested PDU session modification (home-routed roaming) (clause 4.3.3.3)

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2. 	If the SMF determines that EPS bearer ID(s) needs to be assigned to the QoS flow(s) in the PDU session, SMF invokes Namf_Communication _EBIAssignment (PDU session ID, ARP list). If the SMF receives any EBI(s) from the AMF, it needs to include the received EBI(s) into the mapped EPS QoS parameters to be sent to the UE.

3.	The relevant steps of the procedure as specified in the figure above are executed.

* * * * Next Change * * * *

5.2.2	AMF Services

5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

# Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Request/Response | Peer AMF |
|  | RegistrationCompleteNotify | Subscribe/Notify | Peer AMF |
|  | N1MessageNotify | Subscribe/Notify | SMF, SMSF, PCF, NEF |
|  | N1MessageSubscribe |  | SMF, SMSF, PCF, NEF |
|  | N1MessageUnSubscribe |  | SMF, SMSF, PCF, NEF |
|  | N1N2MessageTransfer | Request/Response | SMF, SMSF, PCF, NEF |
|  | N2InfoSubscribe | Subscribe/Notify | SMF |
|  | N2InfoUnSubscribe |  | SMF |
|  | N2InfoNotify |  | SMF |
|  | EBIAssignment | Request/Response | SMF |
| Namf_EventExposure | Subscribe for one UE, group of UE(s) or any UE. | Subscribe/Notify | NEF, SMF, PCF, UDM |
|  | UnSubscribe for one UE, group of UE(s) or any UE. | Subscribe/Notify | NEF, SMF, PCF, UDM |
|  | Notify | Subscribe/Notify | NEF, SMF, PCF, UDM |
| Namf_MT | EnableUEReachability | Request/Response | NEF, SMF, PCF, UDM |

Editor's note:	Whether additional NF services can be identified or be decomposed from existing NF services are FFS.

* * * * Next Change * * * *

5.2.2.2.2.11	Namf_Communication _EBIAssignment service operation

Service operation name: Namf_Communication _EBIAssignment.

Description: The consumer NF uses this service operation to request a bunch of EPS bearer IDs for a PDU session, and optionally put a list of EBIs back.

# Known NF Consumers: SMF

Inputs, Required: SUPI, PDU Session ID

# Input, Optional: ARP list

# Outputs, Required: Cause.

# Outputs, Optional: a list of <ARP, EBI> pair.

The consumer NF invokes the Namf_Communication _EBIAssignment service operation when it determines that one or more EPS bearer IDs are required for EPS QoS mapping for a PDU session. The AMF responds the consumer NF with a cause which indicates whether the assignment is successful or not. If the assignment is successful, the AMF provides a list of <ARP, EBI> pair to the consumer NF.

* * * * Next Change * * * *

5.2.8.2.6	Nsmf_PDUSession_Update SM Context service operation

Service operation name: Nsmf_PDUSession_Update SM Context.

Description: It allows to update the SM Context of a certain PDU Session due to new/updated N1/N2 SM information.

# Known NF Consumers: AMF.

Input, Required: SUPI, PDU Session ID.

Input, Optional: N1 SM container, N2 SM information, Operation type (e.g. activate, deactivate), UE location information, AN type.

# Output, Required: Result Indication.

Output, Optional: Cause, EBI list, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE.

See step 1a, 1e, 3a, 7 of clause 4.3.3.2 and step 1a, 1e, 4a, 8, 8a of clause 4.3.3.3 for an example usage of this service operation.

If the consumer NF is AMF and the SMF determines that some EBIs are not needed, the SMF will put the EBIs back.

5.2.8.2.7	Nsmf_PDUSession_Release SM Context service operation

Service operation name: Nsmf_PDUSession_Release SM Context.

Description: It allows to release the SM Context of a certain PDU Session (e.g. UE is being deregistered).

# Known NF Consumers: AMF.

Input, Required: SUPI, PDU Session ID.

Input, Optional: UE location information, AN type.

# Output, Required: Result Indication.

# Output, Optional: Cause.

See step 1a, 3a of clause 4.3.4.2 and step 1a, 6a of clause 4.3.4.3 for an example usage of this service.

If the consumer NF is AMF and the PDU session indicated by the PDU session ID has been assigned some EBIs, the AMF needs to re-collect all the assigned EBIs.

* * * * End of Changes * * * *