SA WG2 Meeting #127bis	S2-185586

28 May – 1 Jun, 2018, Newport Beach, CA, USA	(revision of S2-185407)

Source:	Apple (UK) Limited

Title:	23.793: Update of Solution 5

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes URSP policy extensions for MPTCP PDU session type and other minor changes for Solution 5.

# 1.	Discussion

In Rel-15, the UE is responsible for establishing new PDU session(s) based on URSP provided by the network (PCF) and/or preconfigured in the UE as described in TS 23.503 [7] clause 6.6.2. As such, URSP provides SDF level steering capability for UE to establish new PDU session(s) over the preferred access network based on traffic descriptor and rule precedence.

Solution 5 extends URSP as described in TS 23.503 [7] section 6.6.2 by adding:

-	The PDU Session Type field in the Route Selection Descriptor to specify the type of the PDU session (PDU or MPTCP PDU) to be established for the matching application;

- 	If the PDU session type is MPTCP PDU:

- 	The Allowed MPTCP Service Type(s) field specifies the MPTCP Service Type(s) that are allowed for the MPTCP PDU session:

-	Handover MPTCP Service Type: An MPTCP service that provides seamless handover between non-3GPP and 3GPP access in order to preserve the connection. If the rule specifies Handover MPTCP Service Type, Interactive and Aggregate Service Types shall not be present;

-	Interactive MPTCP Service Type: A service whereby Multipath TCP attempts to use the lowest-latency interface. If the rule specifies Interactive MPTCP Service Type, Handover MPTCP Service Type may be present but Aggregate Service Type shall not be present; or

-	Aggregate MPTCP Service Type: An MPTCP service that aggregates the capacities of the access networks in an attempt to increase throughput and minimize latency. If the rule specifies Aggregate MPTCP Service Type, Handover and Interactive MPTCP Service Types may be present as well;

-	In case of WLAN access, the Recommended SSID(s) for MPTCP PDU Session field specifies the list of SSID(s) that are recommended on the WLAN access for the MPTCP PDU Session;

-	A new descriptor called Validity Descriptor to specify validity conditions, i.e. conditions indicating when the rule is valid (such conditions may include e.g. a time duration, a location area, etc.);

# 2.	Proposal

It is proposed to modify Solution 5 in TR 23.793 as follows:

***** First Change *****

## 6.5	Solution 5: Multipath TCP (MPTCP) proxy architecture

### 6.5.1	General

This solution addresses Key Issues #1 and #4.

Multipath TCP (MPTCP) defined in [8] is a set of extensions to regular TCP to provide a Multipath TCP service, which enables a transport connection to operate across multiple paths simultaneously.

MPTCP defines how to setup multiple paths ("subflows"), manage these subflows, reassemble of data and terminate sessions.

The simultaneous use of multiple disjoint (or partially disjoint) paths across the network increases the resilience of the connectivity by providing multiple paths, protects UEs from the failure of one path, and it also increases the efficiency of the resource usage, and thus increases the network capacity available to UEs.

### 6.5.2	MPTCP proxy

When communicating with traditional TCP servers, MPTCP capable devices have to fall back to traditional TCP and cannot fully utilize their MPTCP capability. In order to support the use of MPTCP session between an MPTCP host and a TCP host, or to help to aggregate MPTCP subflows, an MPTCP proxy needs to be introduced.

Figure 6.5.2-1: Illustration of an MPTCP proxy

The MPTCP proxy makes it possible to achieve the benefits of MPTCP for access aggregation and route selection without requiring MPTCP support in all TCP servers.

### 6.5.3	Addition of MPTCP proxy to 5GC

In order to introduce the MPTCP proxy to 5GC and allow 3GPP UEs to make use of the MPTCP capability, the UPF shall incorporate the MPTCP proxy functionality for the PDU session.

Figure 6.5.3-1: Addition of MPTCP proxy to 5GC

## 6.5.4	MPTCP PDU session establishment procedure

## This solution supports the Multi-Access PDU session establishment defined in section 6.2. Each of the PDU sessions may have its own set of UPFs, but both child PDU sessions share a common PDU session anchor (PSA).

## This procedure is based on the procedure defined in section 6.2.2.2 with the following exceptions:

## 1. The UE requests the network to include an MPTCP proxy on the user plane;

## 2. The MPTCP proxy address is transported back to the UE;

## 3. The child PDU sessions do not share a common IP address but they are allocated separate IP addresses.

Figure 6.5.4-1-1: UE-requested MPTCP PDU Session Establishment for non-roaming and roaming with local breakout

Editor's note: 	It is FFS whether instead of the above PDU session setup procedure the UE could setup two independent PDU sessions for MPTCP procedures as defined in section 6.2.2.1. In this case it is also FFS how the UE receives the address of the MPTCP proxy.

1.	The UE initiates the "UE Requested PDU Session Establishment" procedure as specified in TS 23.502 [6] clause 4.3.2.2.1 on 3GPP access with an additional preference to establish an MPTCP PDU Session. The UE can also first initiate the PDU Session Establishment procedure on untrusted non-3GPP access.

2.	If the network supports MPTCP procedures, the AMF selects an SMF that supports MPTCP procedures.

3.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request to SMF.

4.	The SMF registers with the UDM.

5.	The SMF sends an Nsmf_PDUSession_CreateSMContext Response to AMF.

6.	Optional Secondary authorization/authentication.

7a.	If dynamic PCC is deployed, the SMF performs PCF selection.

7b.	The SMF may perform a Session Management Policy Establishment.

8.	The SMF selects an UPF that incorporates an MPTCP proxy functionality. As an alternative, the UPF can select a standalone MPTCP proxy.

9.	SMF may perform a Session Management Policy Modification procedure.

10-17.		The user plane of the PDU session over 3GPP access is established and the UE creates the first MPTCP subflow over the 3GPP access. The first MPTCP subflow can also be created over the non-3GPP access once the PDU Session is established over the untrusted non-3GPP access.

-	In step 11, the SMF indicates to AMF the MPTCP proxy address.

-	In step 13, the UE receives a PDU Session Establishment Accept message containing the MPTCP proxy address (if the network supports MPTCP procedures), thus, the UE determines that the MPTCP PDU session over 3GPP access is established.

19-27.		The user plane of the PDU session over untrusted non-3GPP access is established and the UE creates an additional MPTCP subflow over the untrusted non-3GPP access.

NOTE:	The steps 10-17 and 19-27 can be executed in parallel.

## 6.5.5	ATSSS rules for MPTCP PDU Session

In Rel-15, the UE is responsible for establishing new PDU session(s) based on URSP rules provided by the network (PCF) and/or preconfigured in the UE as described in TS 23.503 [7] clause 6.6.2. As such, URSP rules provide SDF level steering capability for UE to establish new PDU session(s) over the preferred access network based on traffic descriptor and rule precedence.

## The ATSSS rules defined by this solution are extension of the URSP rules described in TS 23.503 [7] section 6.6.2. The following new fields are created:

## - 	The MPTCP PDU Session field in the Route Selection Descriptor specifies whether the type of the PDU session is an MPTCP PDU session to be established for the matching application;

- 	In case of an MPTCP PDU session:

- 	The Allowed MPTCP Service Type(s) field specifies the MPTCP Service Type(s) that are allowed for the MPTCP PDU session:

-	Handover MPTCP Service Type: An MPTCP service that provides seamless handover between non-3GPP and 3GPP access in order to preserve the connection. If the rule specifies Handover MPTCP Service Type, Interactive and Aggregate Service Types shall not be present;

-	Interactive MPTCP Service Type: A service whereby Multipath TCP attempts to use the lowest-latency interface. If the rule specifies Interactive MPTCP Service Type, Handover MPTCP Service Type may be present but Aggregate Service Type shall not be present; or

-	Aggregate MPTCP Service Type: An MPTCP service that aggregates the capacities of the access networks in an attempt to increase throughput and minimize latency. If the rule specifies Aggregate MPTCP Service Type, Handover and Interactive MPTCP Service Types may be present as well;

## -	In case of WLAN access, if the UE has not yet selected a WLAN access network, the Preferred SSID(s) for MPTCP PDU Session field specifies a prioritized list of SSID(s) that are preferred on the WLAN access for the MPTCP PDU Session;

The PCF may select the ATSSS rules applicable for each UE based on local configuration and operator policies taking into consideration e.g. UE location or time of the day.

## The PCF may provide the ATSSS rules at the initial Registration procedure, or when the operator policies indicate that the conditions for updating the UE are met, i.e. at change of UE location or at mobility with change the AMF, or at any time, as defined in TS 23.502 [3]. Operator defined policies in the PCF may depend on input data such as UE location, time of day, information provided by other NFs, etc.

Table 6.5.5-1: UE Route Selection Policy (from TS 23.503 [7] table 6.6.2.1-1)

| Information name | Description | Category | PCF permitted to modify in a URSP | Scope |
| --- | --- | --- | --- | --- |
| URSP rules | 1 or more URSP rules as specified in table 6.6.2.1-2 | Mandatory | Yes | UE context |

Table 6.5.5-2: UE Route Selection Policy Rule (from TS 23.503 [7] table 6.6.2.1-2)

| Information name |  | Description |  | Category |  | PCF permitted to modify in a UE context |  | Scope |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rule Precedence |  | Determines the order the URSP rule is enforced in the UE. |  | Mandatory (NOTE 1) |  | Yes |  | UE context |
| Traffic descriptor |  | This part defines the traffic descriptors for the policy |  |  |  |  |  |  |
| Application identifiers |  | Application identifier(s) |  | Optional |  |  |  |  |
| IP descriptors |  | IP 3 tuple(s) (destination IP address or IPv6 network prefix, destination port number, protocol ID of the protocol above IP) |  | Optional |  | Yes |  | UE context |
| Non-IP descriptors |  | Descriptor(s) for non-IP traffic |  | Optional |  | Yes |  | UE context |
| DNN |  | This is the DNN information provided by the application | Optional | Yes | UE context |
| List of Route Selection Descriptors |  | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.2.1-3 |  | Mandatory |  |  |  |  |
| NOTE 1: Rules in a URSP shall have different precedence values. |  |  |  |  |  |  |  |  |

Table 6.5.5-3: : Route Selection Descriptor (from TS 23.503 [7] table 6.6.2.1-3)

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value PDU Session Type | Optional | Yes | UE context |
| Non-seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 3) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| MPTCP PDU Session | Indicates whether the type of the PDU session is MPTCP PDU session type or not. | Mandatory (NOTE 4) | No | UE context |
| Allowed MPTCP Service Type(s) | Either a single value or a list of values of Allowed MPTCP Service Type(s). Possible values: Handover, Interactive, Aggregation. | Optional (NOTE 5) | Yes | UE context |
| Preferred SSID(s) for MPTCP PDU Session | In case of WLAN access, if the UE has not yet selected a WLAN access network, it indicates a prioritized list of SSID(s) that are preferred for MPTCP PDU Session Type. | Optional (NOTE 5) | Yes | UE context |
| NOTE 1: Every Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the route selection component shall be present. NOTE 3: If this indication is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. NOTE 4: For the MPTCP PDU Session, the Non-seamless Offload indication and the Access Type preference components shall not be included in the Route Selection Descriptor. NOTE 5: This component shall only be present for the MPTCP PDU Session. |  |  |  |  |

***** End of Changes *****