SA WG2 Meeting #129	S2-1810209

October 15-19, 2018, Dongguan, P. R. China	(revision of S2-18xxxx)

Source:	Ericsson

Title:	Key Issue 2 Conclusion

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract of the contribution: Interim conclusion for 5G CIoT Key Issue 2

# Discussion

In SA2#128BIS an interim conclusion for Key Issue 2 was made in S2-188995. The current conclusion reads:

At least one of the solutions 5, 7, 19, 29, 36, 41 will be selected for normative work.

NOTE:	The specific solution from the ones listed above will be selected in a subsequent meeting. It is not precluded that some aspects of the solutions listed above may be merged. Existing solutions that enhance the solutions above are also not precluded. Support of RDS will be decided as part of the conclusion for key issue 8.

The figure below gives an overview of the above solutions, plotted based on a rough estimate of where the optimizations are most effective onto an axis with inactivity time i.e. the frequency of small data communication. Solution 7 has two alternatives for buffering of DL data which have quite different characteristics and has therefore been plotted twice in the figure (with CN buffering and without i.e. with RAN buffering).

It is noted that the key issue 2 solutions are all user plane optimizations and just as in in EPS, user plane optimizations can also be applied for infrequent small data communication.

Inactivity time

1 sec

7. RRC Inactive

”frequent”

”infrequent”

19. Suspend/Resume

7. RRC Inactive w/ CN buffering

1 min

1 hour

1 day

1 month

5. SD Fast Path

29. ConnectionLess

41. Combining RRC-Inactive & sol 19

36. Inactive resume & deact.

Figure: Key issue 2 solutions on a frequency/inactivity time axis

The companies that support this proposal believe solution 19, and solution 35 (not shown in figure above) should be a base for normative standardization. Solution 19 (5G UP Optimization) is very similar to the 4G User Plane CIoT EPS Optimization. Solution 35 (Small data delivery function) is for operators that deploy an optional NEF based API for small data transmissions.

Motivation: It is a feature alignment of NB-IoT/LTE-M in both 5GC and EPC.

*** First change ***

## 8.2	Key Issue 2: Support for frequent small data transmission

Solution 19 and solution 35 should be a base for normative standardization.

*** End of changes ***