3GPP TSG-SA WG2 Meeting #130	S2-1900079

21 – 25 January 2019, Kochi, India

Source:	KDDI, TOYOTA ITC

Title:	7.4 NWADF-assisted Background Data Transfer

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This paper proposes to add the new featurture related to NWDAF-assited Background Data Transfer into TS 23.288.

- Discussion

As discussed and studied in TR 23.791, Solution 18 and 28 are selected as the concluded solution for NWDAF-Assisted Future Background Data Transfer. This PCR proposes to add the new feature about NWDAF-Assisted Future Background Data Transfer into TS 23.288 as an implemention.

Here, we propose that the same information as NWDAF input and output is used for the features based on Solution 18 and 28, respectively.

- Proposal

The following changes are proposed in TS 23.288.

* * * Start of Change (all text is new)  * * *

7.4	NWDAF-Assisted Future Background Data Transfer

Editor’s note: Describes requirements, procedures and related network functions for NWDAF-Assisted future background data transfer.

7.4.1	General Procedure Description

The NWDAF shall provide the analytics related to the network condition in order to help the PCF in deciding the transfer policy for the background data transfer. The PCF can decide the transfer policy based on the latest network condition.

The NWDAF shall also provide the analytics related the network condition in order to help the PCF in deciding whether the notification that the background data traffic is discarded should be sent to the particular AF.  When the PCF sends this, the AF may try to negotiate with the MNO regarding the transfer policy again. The network condition may be changed, after the transfer policy is agreed between MNO and ASP. There are the cases where the background data traffic may get discarded for example when the network becomes congested, then other traffic is prioritized rather than the background data traffic.

7.4.2	Information for the support of NWDAF-Assisted Future Background Data Transfer

The UE level information related to background data transfer is defined in Table 7.4.2-1 in order that the NWDAF performs/provides the big data analytics.

There are 2 variants of the information related to the UE location:

- AF knowledge of UE area: in this variant the AF provides the target area where Future Background Data Transfer is expected. This variant focuses on the fact that the AF has a better knowledge than the network on where the UE(s) should be at the expected time of the background data transfer.

- NWDAF prediction of the UE area: in this variant the AF does not provide the target area where Future Background Data Transfer is expected but the NWDAF has to determine (guess/predict) it based on:

‐	the knowledge of the list of UE corresponding to the AF transfer. To avoid AF providing huge lists of UE(s) it is assumed that all these UE are configured to belong to a group and the AF only refers to the Group ID.

‐	predictions on UE mobility: NWDAF derives the UE Moving Trajectory by collecting the UE level information such as location information from AMF.

The information related to the UE location is not needed in case of AF knowledge of UE area.

Table 7.4.2-1: NWDAF input data

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID or Internal group Id | C |  | Present if NWDAF requests location information |
| Location info |  |  |  |
| >Timestamp | C | AMF | The age of the information |
| >Location Info | C | AMF | The location info for the UE e.g. Cell ID or TA ID |
| Network performance info |  |  | It is collected from OAM |
| Area information | C | OAM | Present if NWDAF requests network performance information |
| Load information | C |  | Statistics on load in the area. |

Table 7.4.2-2 and 7.4.2-3 show the event id and the analytics information output by the NWDAF based on the input information described in Table 7.4.2-1.

Table 7.4.2-2: Event ID to provide analytics for background data transfer

| Event ID | Event Filter | Description |
| --- | --- | --- |
| Network Condition list | List of UE identities, Time, Date, Network area | Provides the network condition list in the area where the ASP will send background data traffic. |

Table 7.4.2-3: Content of Network Condition List

| IE/Group Name | Presence | Range | Semantics description |
| --- | --- | --- | --- |
| Network Condition list |  | 1 |  |
| > Network Condition IEs |  | 1 to M |  |
| >>list of UE IDs | C |  |  |
| >> Expected Moving Trajectory | C | 1 to N | UE´s expected geographical movement (e.g. as described in TS 23.502 [3] clause 14.5.6.3). |
| >>Network Performance Info | C |  | Performance information on the NF where the UE is expected to be according to the expected moving trajectory |

When the PCF notifies the ASP  that the background data traffic is discarded,  the PCF sends the event id and content described in Tables 7.4.2-4 and 7.4.2-5. The AF may subscribe to BDT policy warning in the PCF.

Table 7.4.2-4: BDT policy warning to renegotiate the BDT policy

| Event ID | Event Filter | Description |
| --- | --- | --- |
| BDT policy warning | ASP identifier, Time, Date, Network area information | PCF requests the ASP to renegotiate the BDT policy, due to potential delays in the transmission of background data. The ASP may trigger a procedure to negotiate the BDT policy with the MNO. |

Table 7.4.2-5: Content of List of BDT policy warning

| IE/Group Name | Presence | Description |
| --- | --- | --- |
| List of BDT policy warning |  |  |
| > IEs |  |  |
| >> ASP identifier | M | Identifier to identify the ASP |
| >>BDT policy reference ID | M | Reference ID of agreed BDT policy |
| >>Time | O | It indicates the time when the background data of the UEs of this ASP may be delayed due to network conditions |
| >>Date | O | It indicates the date when background data of the UEs of this ASP may be delayed due to network conditions |
| >>Network area | O | indicates the network area where background data of the UEs of this ASP may be delayed due to network conditions |
| >>UE ID | O | Indicates the list of UEs whose background data of the UEs of this ASP may be delayed due to network conditions |

7.4.3	Procedure to support NWDAF-Assisted Future Background Data Transfer

The PCF may subscribe to the NWDAF or periodically requests a network condition list for an ASP based on AF request. Through the negotiations for future background data transfer in clause 4.16.7 TS 23.502 [x], the PCF determines one or more transfer policies, based on information provided by the AF and the available information from the NWDAF.

The AF in the ASP can subscribe the BDT warning notification to the PCF through the NEF. Then, the PCF subscribes to the NWDAF or periodically requests a network condition list for an ASP based on AF request.When the PCF judges that the background data traffic is discarded based on the available information from the NWDAF,  the PCF sends the BDT warning notification described in Tables 7.4.2-4 and 7.4.2-5 to the AF through the NEF.

7.4.4	Interaction with other NFs

Editor’s Note: Describes the interaction between the NWDAF and other NFs to assist future background data transfer.

NWDAF:

- Collects the data described in Table 7.4.2-1 from the other NFs.

- Provides the analysis related to the network condition list described in 7.4.2-3.

PCF:

- Utilize the information from the NWDAF to decide the transfer policy for the background data transfer

- Supports the event ID to notify the network conditions for the ASP and have the logic to judge whether the NW conditions impact the UEs of the ASP and then a notification should be transferred to the particular AF to renegotiate a background data transfer.

AF

- Subscribe the BDT warning notification to PCF

- Support the request to renegotiate a new BDT policy due to network conditions.

* * * End of change * * *