3GPP TSG-SA WG2 Meeting #130	S2-1900955

21st – 25th, January, 2019, Kochi, India	(Revision of S2-1900406)

Source:	Samsung Electronics

Title:	MICO mode parameter optimization for mIoT terminals

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA/ Rel-16

Abstract of the contribution: This paper proposes the text for clause 7.6 of TS 23.288, based on the conclusions in TR 23.791.

1. Introduction

Based on the conclusions in the SA2 #129bis meeting and Solution 29 of TR 23.791, the contribution proposes a text for the eNA feature on the support of battery lifetime enhancement of mIoT devices. The proposed text is imported from the Solution 29 of TR 23.791, with adaptation to the normative specification and proper editorial corrections.

2. Proposal

It is proposed to adopt the following text changes in TS 23.288.

**** Start of 1st Changes ****

7.x	MICO Mode Parameter Optimization for mIoT Terminals

7.x.1	General

To enhance the mIoT terminal's battery lifetime, the proper configuration of MICO mode related parameters are required. The NWDAF may provide analytics information to the AMF to assist the AMF to decide configuration parameters for MICO mode such as 1) Allow/disallow MICO mode, 2) periodic registration timer, 3)Active Time and Connected Time.

Editor’s Note:	The exact parameters for MICO mode configuration may be further updated based on CIoT conclusion.

Depending on the decision algorithm in the AMF, the AMF may utilize the external parameters provisioned by AFs such as the behavioural pattern in TR 23.724 and the communication pattern in TS 23.682. The AMF may request the NWDAF to include recommended values for any missing external parameters in the analytics information.

7.x.2	Input Data

The information to be collected by the NWDAF for generation of analytic data related to MICO mode parameter optimization can be classified into three groups, 1) network data from NFs, 2) service data from AFs, and 3) management data from the OAM.

The network data to collect from NFs in the 5GC is defined in Table 6.29.1.2.1-1. The input data consist of data for identification of a specific target and data for extracting analytic estimation from the similar past situations or the similar group of targets.

Table 7.6.2.1-1: Network data collection for the analysis

| Information | Source | Description |
| --- | --- | --- |
| Location Info | AMF or SMF | The location information of the UE. |
| DNN | SMF or PCF | To identify the DNN |
| S-NSSAI | SMF or AMF | To identify the S-NSSAI for the PDU Session which contains the QoS flow |
| UE ID | AMF or SMF | To identify UE |
| Internal Group ID | SMF | To identify UE group if available |
| Traffic usage report | UPF or SMF | To extract traffic characteristics such as flow length, size, and inter packet arrival time. |
| Subscription data | UDR/UDM | To extract the specific UE related information. To extract UE's service contract, UE context, application data, UE specific policy data, and etc. |
| External provisioned parameters | UDR/UDM | To extract the provisioned external parameters such as network configuration parameters, provisioned communication and behavioural patterns. |
| NF resource status | OAM/NRF | The status of assigned resources such as CPU and memory. |
| NF load | OAM/NRF | The load of specific NFs. |
| Service experience | AF | User or service provide feedback on service experience such as MOS. |

NOTE:	The correlation of analysed traffic information per DNN, per Slice, per service, per group of UE is dependent on usage of the information. Timestamp can be used to correlate the collected data from different sources.

7.x.3	Output Analytics

The AMF may consult NWDAF to obtain the analytics information for MICO mode parameter optimization. The NWDAF may respond to the AMF request with the analytic estimation of necessary parameter values and any missing external parameter values. The accuracy and confidence interval related to the analytic assistant information can be optionally contained together in the response. It provides that the confidence on the results of the NWDAF compared to the past observed data. The examples of parameters include behavioural pattern in TR 23.724 and communication pattern in TS 23.682.

Editor’s Note:	The exact analytics information for MICO mode parameter optimization is FFS.

**** End of 1st Changes ****