3GPP TSG- SA WG2 Meeting #130	S2-1900563

# January 21 - 25, 2019, Kochi, India

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0874 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing NF/NF service instance set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBA |  |  |  |  |  | Date: |  |  | 2019-01-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR 23.742 concludes the need for NF/NF Service set concept for 5GC NFs. This is to support better reliability and resiliency for 5G System |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposes to introduce NF/NF service set concept. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Improved reliability and resiliency support not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st  change * * * *

### 5.x.y	NF/NF service instance set for reliability

NF/NF Service instance Set is support for all NFs defined for 5GC in section 4. The NF Set shall be identified by NF Set ID, NF Service instance Set is identified by NF Service Instance Set ID. Following are the principles for NF/NF Service Instance Set:

1)	The NF/NF Service instances within a given NF/NF Service instance Set are expected to have access to the same data sets in a data storage function UDSF. Thus, in principle, any NF/NF Service Instance within a set should be able to process UE transactions as it has access to UE context. Following are the key principles for NF/NF Service Instance Sets:

-	A Set of instances of the same service type.

-	All NF/NF Service/Service instances in a Set can access the same data storage UDSF.

2)	When a NF/NF Service/Service Instance Set exposes multiple NF/NF Service instances towards a consumer, the consumer is allowed to reselect a different NF/NF Service Instance (within the same set) between transactions. Race conditions with multiple requests for the same UE is up to implementation to resolve, potentially using mechanisms like redirect between NF/NF Service Instances in the Set.

3)	Some restrictions that apply for assigning NFs/NF Services to a set:

-	Sets of  SMF/SMF Services managing UPFs need to be connected to the same UPF.

NOTE 1:	The UPFs are not grouped as SETs.

NOTE 2:	The Set within an operator network can be identified using SET ID and the actual mapping of instances to a given SET is up to deployment.

NOTE 3:	When a single Set expands across multiple locations within an operator network, the Service Zone ID can be used to refer to NF/NF Service instances within a Set in particular location. Service Zone ID corresponds to the "locality" information.

6)	The combination of SET ID and Service Zone ID can be used to indicate that the target NF/NF Service instance should be selected from the SET in particular Service Zone.

7)	If NF Service instances within the NF are made discoverable in the NRF, then it should be an option to exhibit and register NF Service instances of the NF Service instance Set in the NRF along with NF Service Instance Set ID (i.e. allowing load balancing of NF Service instance by the consumer or SFSF). Alternatively, the NF Service Instance Set can be made discoverable as a single NF service instance while registering in the NRF (i.e. in this case, load balancing of NF Service Instances of the NF Service Instance Set is internal to the NF).

* * * * End of Changes * * * *