3GPP TSG-SA WG2 Meeting #130 	S2-1901065

# 21 - 25 January 2019, Kochi, India											 (revision of S2-1900618)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0970 | rev | 1 | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of analytics for background data transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-01-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per agreement during FS_eNA, the PCF can use analytics in order to determine the transfer policy for the background data transfer. Based on this, the description in the procedure should be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Analytics from the NWDAF are added as possible input for PCF to determine the transfer policy for the background data transfer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not match with the conclusion of FS_eNA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START of CHANGE

4.16.7.2	Procedures for future background data transfer

Figure 4.16.7.2-1: Negotiation for future background data transfer

1.	The AF invokes the Nnef_BDTPNegotiation_Create service. The AF may include a Group ID or the target network area information (e.g. geo location). When the Group ID is provided, the NWDAF interacts with the NWDAF to derive the target network area information as defined in TS 23.288 [xx]. When neither Group ID nor target area information is provided, the target network area information is the whole operator network.

2.	Based on an AF request, the NEF invokes the Npcf_BDTPolicyControl_Create service with the H-PCF to authorize the creation of the policy regarding the background data transfer. The AF provides the Group ID or the target network area information to the H-PCF.

NOTE 1:	The NEF does not provide any information about the identity of the UEs potentially involved in the future background data transfer.

NOTE 2:	If the 3rd party application server is not able to provide any specific network area information and the request includes no Group ID, the AF request is for a whole operator network.

3	The H-PCF may request from the UDR the stored transfer policies for all the ASPs using Nudr_DM_Query (Policy Data, Backgroud Data Transfer) service operation.

NOTE 3:	In case only one PCF is deployed in the PLMN, the transfer policy can be locally stored and no interaction with UDR is required.

4.	The UDR provides all the stored transfer policies and corresponding network area information to the H-PCF.

5.	The H-PCF determines, based on information provided by the AF and other available information one or more transfer policies. The PCF may interact with the NWDAF and request analytics information on both, the number of UEs in the area and the network performance (e.g. load) in that area, as defined in TS 23.288 [xx].

NOTE 4:	The maximum aggregated bitrate is not enforced in the network.

6.	The H-PCF send the acknowledge message to the NEF with the acceptable transfer policies and a Background Data Transfer Reference ID.

NOTE 5:	The NEF forwards the received transfer policies to the AF and stores the Background Data Transfer Reference ID for future interaction with the PCF.

7.	The NEF sends a Nnef_BDTPNegotiation_Create response to the AF to provide one or more background data transfer policies to the AF. If the NEF received only one background transfer policy from the PCF, steps 8-11 are not executed and the flow proceeds to step 12. Otherwise, the flow proceeds to step 8.

8.	The AF invokes the Nnef_BDTPNegotiation_Update service to provide the NEF with the selected background data transfer policy.

9.	The NEF invokes the Npcf_BDTPolicyControl_Update service to provide the H-PCF with the selected background data transfer policy and the associated Background Data Transfer Reference ID.

10.	The H-PCF sends the acknowledge message to the NEF.

11.	The NEF sends the acknowledge message to the AF.

NOTE 6:	If the NEF receives only one transfer policy, the AF is not required to confirm.

12.	The H-PCF stores the reference ID together with the new transfer policy and the corresponding network area information in the UDR by invoking Nudr_DM_Update (BDT Reference id, Policy Data, Background Data Transfer, updated data). This step is not executed, when the PCF decides to locally store the transfer policy.

13.	The UDR sends a response to the H-PCF as its acknowledgement.

END OF CHANGES