3GPP TSG-SA2 Meeting #131	S2-1901639

# Tenerife, Spain, 25 Feb. – 1 Mar., 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0019 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conclusion proposal on KI#7 GRB recovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TR 23.725, there are 5 alternative solutions for KI#7 (solution#16, 20, 21, 22, 23). Solution#16 mentions that UE may be awared when the GBR cannot be guaranteed bu using RRC message. The target RAN node to UE signalling may also carry an RRC indication that is to be passed to the UE's NAS layer to inform the UE's NAS that the GBR QoS will not be immediately available. After the handover, when the GBR QoS is able to be restored, the RAN node sends an indication of the restoration of QoS to the UE, e.g. using new Rel-16 RRC signalling. However, in our understanding, it is not validated (i.e. the usecase for New RRC signalling to keep the UE's NAS aligned with the Notification status in the Core Network ). Also, it cannot do anything to help recovering the GBR flow. So we think this parameter in RRC message is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description in conclusion clause that there is no UE imapct |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No conclusion of FS_URLLC KI#7 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

8	Conclusions

8.x  Key Issue #7: Automatic GBR service recovery after handover

For this Key Issue#7, no UE impact will be introduced.

* * * * End of Change * * * *