3GPP TSG-SA2 Meeting #131	S2-1901902

# Santa Cruz - Tenerife, Spain, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0024 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conclusion proposal for Key Issue #7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Conclude on KI#7 Automatic GBR service recovery after handover |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes conclusion for KI#7: - Reusing the R15 Notification control parameter for KI#7. - The parameter is transferred from Source RAN to Target RAN. - If the GFBR can’t be guaranteed at Target RAN, the Target RAN still setup the QoS Flow. The Target RAN continues allocating the uplink/downlink resource to the QoS Flow. It’s up to AF decision whether to stop the traffic. - There’s no need to inform UE by RRC signalling that the GFBR can’t be guaranteed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No conclusion for KI#7. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

8.x	Key Issue #7: Automatic GBR service recovery after handover

The following principles are concluded:

- Reusing the R15 Notification control parameter for KI#7.

- The parameter is transferred from Source RAN to Target RAN.

- If the GFBR can’t be guaranteed at Target RAN, the Target RAN still setup the QoS Flow. The Target RAN continues allocating the uplink/downlink resource to the QoS Flow. It’s up to AF decision whether to stop the traffic.

- There’s no need to inform UE by RRC signalling that the GFBR can’t be guaranteed.

* * * End Of Changes * * *