3GPP TSG-SA2 Meeting #131	S2-1901905

# Santa Cruz - Tenerife, Spain, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0985 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-UE QoS for NB-IoT UEs using data over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support Inter-UE QoS for NB-IoT UEs using data over NAS, in EPS MME chooses one of the QoS profiles of the active EPS bearers and sends it to eNB for QoS differentiation. In 5GS, we prefer also sticking to this principle. In this paper, we propose to add "QoS Differentiation Information" in the CN assisted RAN parameters tuning, it includes the QoS profile selected by SMF to support Inter-UE QoS differentiation for NB-IoT UEs using Control Plane CIoT 5GS Optimisation. The “QoS Differentiation Information" is provided to AMF in the " SMF derived CN assisted RAN parameters tuning" which has been introduced in the last meeting (S2-1901307). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause 5.X.Y to support Inter-UE QoS differentiation for NB-IoT UEs using Control Plane CIoT 5GS Optimisation. Add "QoS Differentiation Information" in the CN assisted RAN parameters tuning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Inter-UE QoS differentiation for NB-IoT UEs can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y(new), 5.4.6.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new clause 5.X.Y should be implemented under the clause “5.X Support for Cellular IoT” of CR0748. |  |  |  |  |  |  |  |  |

BEGINNING OF CHANGES

5.X.Y	Inter-UE QoS for NB-IoT UEs using Control Plane CIoT 5GS Optimisation

To allow the NG-RAN to prioritise resource allocation between different NB-IoT UEs when some of the UEs are using the Control Plane CIoT 5GS Optimisation, the ng-eNB may request, based on configuration, the AMF to supply the ng-eNB with the QoS Differentiation Information for any NB-IoT UE that is using the Control Plane CIoT 5GS Optimisation.

In order to reduce signalling load on the AMF, the ng-eNB may be configured to request the QoS Differentiation Information from the AMF by using the UE's 5G-S-TMSI as identifier, e.g., when the ng-eNB's NB-IoT load exceeds certain threshold(s) or when the eNB needs to cache the QoS profile.

NEXT CHANGE

5.4.6.2	Core Network assisted RAN parameters tuning

Core Network assisted RAN parameters tuning aids the RAN to minimize the UE state transitions and achieve optimum network behaviour. How the RAN uses the CN assistance information is not defined in this specification.

Core Network assisted RAN parameters tuning may be derived by the AMF per UE in the AMF based on collection of UE behaviour statistics and/or other available information about the expected UE behaviour (such as subscribed DNN, SUPI ranges, or other information). If the Expected UE Behaviour parameters of the UE is available (as described in clause 4.15.6.3, TS 23.502 [3]), the AMF may use this information for selecting the CN assisted RAN parameter values. If the AMF is able to derive the Mobility Pattern of the UE (as described in clause 5.3.4.2), the AMF may take the Mobility Pattern information into account when selecting the CN assisted RAN parameter values.

The Expected UE Behaviour parameters can be provisioned by external party via the NEF, as described in clause 5.20.

The CN assistance information provides the RAN with a way to understand the UE behaviour for these aspects:

-	"Expected UE activity behavior", i.e. the expected pattern of the UE's changes between CM-CONNECTED and CM-IDLE states. This may be derived e.g. from the statistical information, or Expected UE Behaviour or from subscription information;

-	"Expected HO behavior", i.e. the expected interval between inter-RAN handovers. This may be derived by the AMF e.g. from the Mobility Pattern information;

-	"Expected UE mobility", i.e. whether the UE is expected to be stationary or mobile. This may be derived e.g. from the statistical information or Expected UE Behaviour parameters or from subscription information;

-	"Expected UE moving trajectory" which may be derived e.g. from the statistical information or Expected UE Behaviour parameters or from subscription information.

-	"QoS Differentiation Information" including the QoS profile selected by SMF to support Inter-UE QoS differentiation for NB-IoT UEs using Control Plane CIoT 5GS Optimisation. SMF selects one of the QoS profiles of the active QoS Flows and sends it to AMF in the SMF derived CN assisted RAN parameters tuning.

The AMF decides when to send this information to the RAN as "Expected UE Behaviour" carried in N2 request over the N2 interface (see TS 38.413 [34]).

NOTE:	The calculation of the CN assistance information, i.e. the algorithms used and related criteria, and the decision when it is considered suitable and stable to send to the RAN are vendor specific.

END OF CHANGES