3GPP TSG-SA2 Meeting #131	S2-1902680

# Santa Cruz, Tenerife, Spain	 25th February - 1st March, 2019	 	    (revsion of S2-1901378)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0736 | rev | 8 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of indirect communication between NF services, and implicit discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG, Tencent, ZTE, Ericsson, NTT DOCOMO, Sprint, Oracle Interdigital, AT&T, Verizon Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorporation of changes according to agreed conclusions of the Feasibility Study on enhanhed Service Based Architecture (FS_eSBA) regarding KI #3 Improvements to service framework related aspects on the aspects of indirect communication of NF/NF Services via an intermediary function and implicit (delegated) discovery by this intermediary function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification of indirect communication of NF/NF Services via an intermediary function (Service Communication Proxy - SCP) and delegated discovery by this SCP function. Changes in Rev8: Clause 6.2.x on SCP description Added: - Message forwarding and routing to destination NF/NF service. - More than one SCP may be present in the communication path between NF Services. -The SCP may be deployed in a distributed manner. - removed the EN on consumer authorization and clarified the bullet about "Communication Security" in NOTE 1: NOTE 1: Communication security, e.g.authorization of the NF Service Consumer to access the NF Service Producer's API are specified in 3GPP TS 33.501 [29]. Clause 6.3.1 changes: 1) Added: The SCP may interact with the NRF to get a discovery result. 2) Updated to: In case of Indirect Communication without Delegated Discovery the requester NF uses the discovery result to select a NF instance and NF Service type while the associated NF service instance selection may be done by the requester NF or an SCP on behalf of the requester NF. 3) changed: stored -> cached 4) The NRF to be used may be provided by the NF consumer as part of the discovery parameters as a result of a NSSF query. Annex X: 1) changed: query -> discovery 2) added: The SCP may interact with an NRF to get a discovery result. 3) updated figure for model C All the changes in rev8 are highlighted in turqoise or yellow color. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No incorporation of agreed conclusions on FS_eSBA regarding indirect communication and implicit discovery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.1, 4.2.1, 4.2.2, 4.2.3, 4.2.4, 6.2.x(new), 6.3.1, annex x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************** First Change ****************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Access Network: An access network comprising a NG-RAN and/or non-3GPP AN connecting to a 5G Core Network.

5G Core Network: The core network specified in the present document. It connects to a 5G Access Network.

5G QoS Flow: The finest granularity for QoS forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.). Providing different QoS forwarding treatment requires separate 5G QoS Flow.

5G QoS Identifier: A scalar that is used as a reference to a specific QoS forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a 5G QoS Flow. This may be implemented in the access network by the 5QI referencing node specific parameters that control the QoS forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Allowed NSSAI: NSSAI provided by the Serving PLMN during e.g. a Registration procedure, indicating the S-NSSAIs values the UE could use in the Serving PLMN for the current registration area.

Allowed Area: Area where the UE is allowed to initiate communication as specified in clause 5.3.2.3.

AMF Region: An AMF Region consists of one or multiple AMF Sets.

AMF Set: An AMF Set consists of some AMFs that serve a given area and Network Slice(s). AMF Set is unique within an AMF Region and it comprises of AMFs that support the same Network Slice(s). Multiple AMF Sets may be defined per AMF Region.

Application identifier: An identifier that can be mapped to a specific application traffic detection rule.

AUSF Group ID: This refers to one or more AUSF instances managing a specific set of SUPIs.

Configured NSSAI: NSSAI provisioned in the UE applicable to one or more PLMNs.

Delegated Discovery: This refers to delegating the discovery and associated selection of NF instances or NF service instances to an SCP.

Direct Communication: This refers to the communication between NFs or NF services without using an SCP.

DN Access Identifier (DNAI): Identifier of a user plane access to one or more DN(s) where applications are deployed.

Emergency Registered: A UE is considered Emergency Registered over an Access Type in a PLMN when in limited service state and registered for emergency services only over this Access Type in this PLMN.

Endpoint Address: An address used by a NF service consumer to access the NF service (i.e. to invoke service operations) provided by a NF service provider. An Endpoint Address is represented in the syntax of Uniform Resource Identifier (e.g. part of Resource URI of the NF service API).

Expected UE Behaviour: Set of parameters provisioned by an external party to 5G network functions on the foreseen or expected UE behaviour, see clause 5.20.

Forbidden Area: An area where the UE is not allowed to initiate communication as specified in clause 5.3.2.3.

GBR QoS Flow: A QoS Flow using the GBR resource type or the Delay-critical GBR resource type and requiring guaranteed flow bit rate.

Indirect Communication: This refers to the communication between NFs or NF services using an SCP.

Initial Registration: UE registration in RM-DEREGISTERED state as specified in clause 5.3.2.

Local Area Data Network: a DN that is accessible by the UE only in specific locations, that provides connectivity to a specific DNN, and whose availability is provided to the UE.

Local Break Out (LBO): Roaming scenario for a PDU Session where the PDU Session Anchor and its controlling SMF are located in the serving PLMN (VPLMN).

Mobility Pattern: Network concept of determining within the AMF the UE mobility parameters as specified in clause 5.3.2.4.

Mobility Registration Update: UE re-registration when entering new TA outside the TAI List as specified in clause 5.3.2.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NGAP UE association: The logical per UE association between a 5G-AN node and an AMF.

NGAP UE-TNLA-binding: The binding between a NGAP UE association and a specific TNL association for a given UE.

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 2:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Network Instance: Information identifying a domain. Used by the UPF for traffic detection and routing.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

Network Slice instance: A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice.

Non-GBR QoS Flow: A QoS Flow using the Non-GBR resource type and not requiring guaranteed flow bit rate.

NSI ID: an identifier for a Network Slice instance.

NF instance: an identifiable instance of the NF.

NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service instance: an identifiable instance of the NF service.

NF service operation: An elementary unit a NF service is composed of.

NG-RAN: A radio access network that supports one or more of the following options with the common characteristics that it connects to 5GC:

1)	Standalone New Radio.

2)	New Radio is the anchor with E-UTRA extensions.

3)	Standalone E-UTRA.

4)	E-UTRA is the anchor with New Radio extensions.

Non-Allowed area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3.

Non-Seamless Non-3GPP offload: The offload of user plane traffic via non-3GPP access without traversing either N3IWF or UPF.

PDU Connectivity Service: A service that provides exchange of PDUs between a UE and a Data Network.

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service.

PDU Session Type: The type of PDU Session which can be IPv4, IPv6, IPv4v6, Ethernet or Unstructured.

Periodic Registration Update: UE re-registration at expiry of periodic registration timer as specified in clause 5.3.2.

(Radio) Access Network: See 5G Access Network.

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Service based interface: It represents how a set of services is provided/exposed by a given NF.

Service Continuity: The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.

Service Data Flow Filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet (IP or Ethernet) flows constituting a Service Data Flow.

Service Data Flow Template: The set of Service Data Flow filters in a policy rule or an application identifier in a policy rule referring to an application detection filter, required for defining a Service Data Flow.

Session Continuity: The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.

Subscribed S-NSSAI: S-NSSAI based on subscriber information, which a UE is subscribed to use in a PLMN

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs.

UDR Group ID: This refers to one or more UDR instances managing a specific set of SUPIs.

UPF Service Area: The area within which PDU Session associated with the UPF can be served by (R)AN nodes via a N3 interface between the (R)AN and the UPF without need to add a new UPF in between or to remove/re-allocate the UPF.

Uplink Classifier: UPF functionality that aims at diverting Uplink traffic, based on filter rules provided by SMF, towards Data Network.

************** Next Change **************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

AUSF	Authentication Server Function

BSF	Binding Support Function

CAPIF	Common API Framework for 3GPP northbound APIs

CP	Control Plane

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

FAR	Forwarding Action Rule

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

N3IWF	Non-3GPP InterWorking Function

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDR	Packet Detection Rule

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

QFI	QoS Flow Identifier

QoE	Quality of Experience

(R)AN	(Radio) Access Network

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSP	Traffic Steering Policy

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

************** Next Change **************

## 4.1	General concepts

The 5G System architecture is defined to support data connectivity and services enabling deployments to use techniques such as e.g. Network Function Virtualization and Software Defined Networking. The 5G System architecture shall leverage service-based interactions between Control Plane (CP) Network Functions where identified. Some key principles and concept are to:

-	Separate the User Plane (UP) functions from the Control Plane (CP) functions, allowing independent scalability, evolution and flexible deployments e.g. centralized location or distributed (remote) location.

-	Modularize the function design, e.g. to enable flexible and efficient network slicing.

-	Wherever applicable, define procedures (i.e. the set of interactions between network functions) as services, so that their re-use is possible.

-	Enable each Network Function and its Network Function Services to interact with other NF and its Network Function Services directly or indirectly via aService Communication Proxy if required. The architecture does not preclude the use of another intermediate function to help route Control Plane messages (e.g. like a DRA).

-	Minimize dependencies between the Access Network (AN) and the Core Network (CN). The architecture is defined with a converged core network with a common AN - CN interface which integrates different Access Types e.g. 3GPP access and non-3GPP access.

-	Support a unified authentication framework.

-	Support "stateless" NFs, where the "compute" resource is decoupled from the "storage" resource.

-	Support capability exposure.

-	Support concurrent access to local and centralized services. To support low latency services and access to local data networks, UP functions can be deployed close to the Access Network.

-	Support roaming with both Home routed traffic as well as Local breakout traffic in the visited PLMN.

************** Next Change **************

### 4.2.1	General

This specification describes the architecture for the 5G System. The 5G architecture is defined as service-based and the interaction between network functions is represented in two ways.

-	A service-based representation, where network functions (e.g. AMF) within the Control Plane enables other authorized network functions to access their services. This representation also includes point-to-point reference points where necessary.

-	A reference point representation, shows the interaction exist between the NF services in the network functions described by point-to-point reference point (e.g. N11) between any two network functions (e.g. AMF and SMF).

Service-based interfaces are listed in clause 4.2.6. Reference points are listed in clause 4.2.7.

Network functions within the 5GC Control Plane shall only use service-based interfaces for their interactions.

NOTE 1:	The interactions between NF services within one NF are not specified in this Release of the specification.

NOTE 2:	UPF does not provide any services in this Release of the specification, but can consume services provided by 5GC Control Plane NFs.

NFs/NF services can communicate directly, referred to as Direct Communication, or indirectly via the SCP, referred to as Indirect Communication.  For more information on communication options, see Annex x and clauses under 6.3.1 and 7.1.2.

************** Next Change **************

### 4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF). The functional description of these network functions is specified in clause 6.

-	Authentication Server Function (AUSF)

-	Access and Mobility Management Function (AMF)

-	Data Network (DN), e.g. operator services, Internet access or 3rd party services

-	Unstructured Data Storage Function (UDSF)

-	Network Exposure Function (NEF)

-	Network Repository Function (NRF)

-	Network Slice Selection Function (NSSF)

-	Policy Control Function (PCF)

-	Session Management Function (SMF)

-	Unified Data Management (UDM)

-	Unified Data Repository (UDR)

-	User Plane Function (UPF)

-	Application Function (AF)

-	User Equipment (UE)

-	(Radio) Access Network ((R)AN)

-	5G-Equipment Identity Register (5G-EIR)

-	Security Edge Protection Proxy (SEPP)

-	Network Data Analytics Function (NWDAF)

-	Service Communication Proxy (SCP)

************** Next Change **************

### 4.2.3	Non-roaming reference architecture

Figure 4.2.3-1 depicts the non-roaming reference architecture. Service-based interfaces are used within the Control Plane.

Figure 4.2.3-1: 5G System architecture

Figure 4.2.3-2 depicts the 5G System architecture in the non-roaming case, using the reference point representation showing how various network functions interact with each other.

Figure 4.2.3-2: Non-Roaming 5G System Architecture in reference point representation

NOTE 1:	N9, N14 are not shown in all other figures however they may also be applicable for other scenarios.

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the UDSF, NEF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDSF, UDR, NEF and NRF as necessary.

NOTE 3:	The UDM uses subscription data and authentication data and the PCF uses policy data that may be stored in UDR (refer to clause 4.2.5).

NOTE 4:	For clarity, the UDR and its connections with other NFs, e.g. PCF, are not depicted in the point-to-point and service-based architecture diagrams. For more information on data storage architectures refer to clause 4.2.5.

NOTE 5:	For clarity, the NWDAF and its connections with other NFs, e.g. PCF, are not depicted in the point-to-point and service-based architecture diagrams. For more information on network data analytics architecture refer to clause 4.2.9.

Figure 4.2.3-3 depicts the non-roaming architecture for UEs concurrently accessing two (e.g. local and central) data networks using multiple PDU Sessions, using the reference point representation. This figure shows the architecture for multiple PDU Sessions where two SMFs are selected for the two different PDU Sessions. However, each SMF may also have the capability to control both a local and a central UPF within a PDU Session.

Figure 4.2.3-3: Applying non-roaming 5G System architecture for multiple PDU Session in reference point representation

Figure 4.2.3-4 depicts the non-roaming architecture in the case of concurrent access to two (e.g. local and central) data networks is provided within a single PDU Session, using the reference point representation.

Figure 4.2.3-4: Applying non-roaming 5G System architecture for concurrent access to two (e.g. local and central) data networks (single PDU Session option) in reference point representation

Figure 4.2.3-5 depicts the non-roaming architecture for Network Exposure Function, using reference point representation.

Figure 4.2.3-5: Non-roaming architecture for Network Exposure Function in reference point representation

NOTE 1:	In figure 4.2.3-5, Trust domain for NEF is same as Trust domain for SCEF as defined in TS 23.682 [36].

NOTE 2:	In figure 4.2.3-5, 3GPP Interface represents southbound interfaces between NEF and 5GC Network Functions e.g. N29 interface between NEF and SMF, N30 interface between NEF and PCF, etc. All southbound interfaces from NEF are not shown for the sake of simplicity.

************** Next Change **************

### 4.2.4	Roaming reference architectures

Figure 4.2.4-1 depicts the 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane.

Figure 4.2.4-1 Roaming 5G System architecture- local breakout scenario in service-based interface representation

NOTE 1:	In the LBO architecture. The PCF in the VPLMN may interact with the AF in order to generate PCC Rules for services delivered via the VPLMN. The PCF in the VPLMN uses locally configured policies according to the roaming agreement with the HPLMN operator as input for PCC Rule generation. The PCF in VPLMN has no access to subscriber policy information from the HPLMN.

NOTE x:	The SCP can be used for indirect communication between NF/NF service instances in theVPLMN, in the HPLMN, or in both VPLMN and HPLMN. Therefore, for simplicity the SCP is not shown in the roaming architecture.

Figure 4.2.4-3 depicts the 5G System roaming architecture in the case of home routed scenario with service-based interfaces within the Control Plane.

Figure 4.2.4-3 Roaming 5G System architecture - home routed scenario in service-based interface representation

NOTE x:	The SCP can be used for indirect communication between NF/NF service instances in theVPLMN, in the HPLMN, or in both VPLMN and HPLMN. Therefore, for simplicity the SCP is not shown in the roaming architecture.

Figure 4.2.4-4 depicts 5G System roaming architecture in the case of local break out scenario using the reference point representation.

Figure 4.2.4-4: Roaming 5G System architecture - local breakout scenario in reference point representation

NOTE 2:	The NRF is not depicted in reference point architecture figures. Refer to Figure 4.2.4-7 for details on NRF and NF interfaces.

NOTE 3:	For the sake of clarity, SEPPs are not depicted in the roaming reference point architecture figures.

The following figure 4.2.4-6 depicts the 5G System roaming architecture in the case of home routed scenario using the reference point representation.

Figure 4.2.4-6: Roaming 5G System architecture-Home routed scenario in reference point representation

For the roaming scenarios described above each PLMN implements proxy functionality to secure interconnection and hide topology on the inter-PLMN interfaces.

Figure 4.2.4-7: NRF Roaming architecture in reference point representation

NOTE 4:	For the sake of clarity, SEPPs on both sides of PLMN borders are not depicted in figure 4.2.4-7.

**************Next Change **************

### 6.2.X	SCP

The Service Communication Proxy (SCP) includes the following functionalitiesy. Some or all of the SCP functionalities may be supported in a single instance of an SCP:

-	Indirect Communication (see section X.Y.Z for details)

-	Delegated Discovery (see section X.Y.Z for details)

-	Message forwarding and routing to destination NF/NF service.

-	Communication security (e.g. authorization of the NF Service Consumer to access the NF Service Producer API), load balancing, monitoring, overload control, etc.

NOTE 1:	Communication security, e.g.authorization of the NF Service Consumer to access the NF Service Producer's API are specified in 3GPP TS 33.501 [29].

NOTE 2: Load balancing, monitoring, overload control functionality provided by the SCP is left up to implementation.

The SCP may be deployed in a distributed manner.

************** Next Change **************

### 6.3.1	General

The NF discovery and NF service discovery enables Core Network entities (NFs or Service Communication Proxy (SCP)) to discover a set of NF instance(s) and NF service instance(s) for a specific NF service or a target NF type. NF service discovery is enabled via the NF discovery procedure, as specified in TS 23.502 [3], clause 4.17.4,4.17.5, 4.17.x and 4.17.y.

Unless the expected NF and NF service information is locally configured on the requester NF, e.g. when the expected NF service or NF is in the same PLMN as the requester NF, the NF and NF service discovery is implemented via the NRF. The Network Repository Function (NRF) is the logical function that is used to support the functionality of NF and NF service discovery as specified in clause 6.2.6.

NOTE 1:	NRF can be implemented together with SCP e.g. for communication option D, depicted in Annex x.

In order for the requested NF type or NF service to be discovered via the NRF, the NF instance with its NF service instance(s) need to be registered in the NRF (e.g. when the producer NF instance and its NF service instance(s) become operative for the first time) as specified in TS 23.502 [3], clause 4.17.1.

In order for the requester NF or SCP to obtain information about the NF and/or NF service(s) registered or configured in a PLMN/slice, based on local configuration the requester NF or SCP may initiate a discovery procedure with the NRF by providing the type of the NF and optionally a list of the specific service(s) it is attempting to discover. The requester NF or SCP may also provide other service parameters e.g. slicing related information. For the detailed service parameter(s) used for specific NF and NF service discovery refer to TS 23.502 [3], clause 5.2.7.3.2.

In case of Indirect Communication a NF Service Consumer employs an SCP which routes the request to the intended target of the request.

If requester NF is configured to delegate discovery, the requester NF may omit the discovery procedure with the NRF and instead delegate the discovery, SCP will act on behalf of the requester NF. In this case, the requester NF adds any necessary discovery and selection parameters to the request in order for the SCP instance(s) to be able to do discovery and associated selection. The SCP may interact with the NRF to get a discovery result.

Depending on the chosen message routing model, the NRF may provide the IP address or the FQDN of NF instance(s) or the Endpoint Address (es) of relevant NF services instance(s) to the requester NF for target NF instance selection. The NRF provides a list of NF instances and NF service instances relevant for the discovery criteria. The result of the NF and NF service discovery procedure is applicable to any subscriber that fulfils the same discovery criteria. The entity that does the discovery (NF or SCP) may cache the result of the NRF discovery procedure received from the NRF.

In case of Direct Communication the requester NF uses the discovery result to select NF instance and a NF service instance that is able to provide a wanted NF Service (e.g., an instance of the PCF that can provide Policy Authorization).

In case of Indirect Communication without Delegated Discovery the requester NF uses the discovery result to select a NF instance and NF Service type while the associated NF service instance selection may be done by the requester NF or an SCP  on behalf of the requester NF.

In both the cases, the requester NF may use the information from a previously cached discovery result for subsequent selections (i.e. the requester NF does not need to trigger a new NF discovery procedure to perform the selection).

In case of Indirect Communication with Delegated Discovery, the SCP will select a suitable NF service instance based on selection parameters provided by the requester NF and optional interaction with the NRF. The NRF to be used may be provided by the NF consumer as part of the discovery parameters as a result of a NSSF query.

A requester NF or SCP having selected an NF service instance within a NF instance may re-select another service instance of the same NF service within that same NF instance and get the same results as if the original selected NF service instance was used. Re-selecting may be needed if the requester NF or SCP fails to contact the original NF service instance.

The requester NF or SCP may subscribe in the NRF to receive notifications of newly registered/updated/de-registered NF/NF service instances of target NF/NF services using Nnrf_NFManagement_NFStatusSubscribe/Notify service operations as defined in TS 23.502 [3].

For NF and NF service discovery across PLMNs, the requester NF or SCP provides the NRF the PLMN ID of the target NF. The NRF in the local PLMN reaches the NRF in the target PLMN by forming a target PLMN specific query using the PLMN ID provided by the requester NF or SCP.

NOTE:	See TS 29.510 [58] for details on using the target PLMN ID specific query to reach the NRF in the target PLMN.

For NF and NF service discovery across PLMNs in the context of Network Slicing, the NRF in the serving PLMN interacts with the NRF in the home PLMN identified as specified in clause 4.17.5 of TS 23.502 [3] and, for SMF in clause 4.3.2.2.3.3 of TS 23.502 [3].

The NRF in the local PLMN interacts with the NRF in the target PLMN to retrieve the FQDN or the identifier of relevant services of the target NF instance (s). For topology hiding, see clause 6.2.17.

**************** Next Change ****************

Annex X:
Communication options for NF/NF services interaction (informative)

## D.1	General

This section provides a high level description of the different communication options NF and NF services may use to interact which each other. Table D.1-1 summarizes the communication options, their usage and how they relate to the usage of an SCP.

Table D.1-1: Communication options for NF/NF services interaction summary

| Communication between consumer and producer | Service discovery and request routing | Communication option |
| --- | --- | --- |
| Direct communication | No NRF or SCP; direct routing | A |
|  | Discovery using NRF services; direct routing | B |
| Indirect communication | Discovery using NRF services; routing by SCP | C |
|  | Discovery and associated selection delegated to an SCP using discovery and selection parameters in consumer request; routing by SCP | D |

Option A - Direct communication without NRF interaction: Neither NRF nor SCP are required. Consumers are configured with producers’ “NF profiles” and directly communicate with a producer of their choice.

Option B - Direct communication with NRF interaction: Consumers do discovery by querying the NRF. Based on the discovery result, the consumer does selection. The consumer sends the request to the selected producer.

Option C - Indirect communication without delegated discovery: Consumers do discovery by querying the NRF. Based on discovery result, the consumer does the selection. The consumer sends the request to the SCP containing the address of the selected producer (e.g. an NF instance or NF Service instance set), the SCP may select an instance and it routes the request to the targeted producer instance.

Option D - Indirect communication with delegated discovery: Consumers do not do any discovery or selection. The consumer adds any necessary discovery and selection parameters required to find a suitable producer to the request. The SCP uses the request address and the discovery and selection parameters in the request message to route the request to a suitable producer instance. The SCP may interact with an NRF to get a discovery result.

Figure D.1-1 depicts the different communication options.

Figure D.1-1: Communication options for NF/NF services interaction