SA WG2 Meeting #S2-131	S2-1902936

25 February - 1 March, 2019, Santa Cruz - Tenerife	(e-mail revision 3 of S2-1902776)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0802 | rev | 3 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eSBA communication schemas related to PCF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO, Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-02-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) |  |
| Reason for change: |  | FS_eSBA |  |  |  |  |  |  |  |  |
| Summary of change: |  | eSBA communication schemas related to PCF discovery and selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No incorporation of agreed communication schemas related to FS_eSBA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 6.3.7	PCF discovery and selection

#### 6.3.7.0	General principles

Clause 6.3.7.0 describes the underlying principles for PCF selection and discovery:

-	There may be multiple and separately addressable PCFs in a PLMN.

-	The PCF must be able to correlate the AF service session established over N5 or Rx with the associated PDU Session (Session binding) handled over N7.

-	It shall be possible to deploy a network so that the PCF may serve only specific DN(s). For example, Policy Control may be enabled on a per DNN basis.

-	Unique identification of a PDU Session in the PCF shall be possible based on the (UE ID, DNN)-tuple, the (UE IP Address(es), DNN)-tuple and the (UE ID, UE IP Address(es), DNN).

#### 6.3.7.1	PCF discovery and selection for a UE or a PDU Session

PCF discovery and selection functionality is implemented in AMF, SMF and SCP, and follows the principles in clause 6.3.1. The AMF uses  the PCF services for a UE and the SMF uses the PCF services for a PDU Session.

When NF service consumer performs discovery and selection, the following applies:

-	The AMF may utilize the NRF to discover the candidate PCF instance(s) for a UE. In addition, PCF information may also be locally configured in the AMF. The AMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the AMF, depending on operator's policies.

-	The SMF may utilize the NRF to discover the candidate PCF instance(s) for a PDU Session. In addition, PCF information may also be locally configured in the SMF. The SMF selects a PCF instance based on the available PCF instances obtained from the NRF or locally configured information in the SMF, depending on operator's policies.

The following factors may be considered during the PCF selection for a PDU session:

a)	Local operator policies.

b)	Selected Data Network Name (DNN).

c)	S-NSSAI of the PDU Session.

d)	SUPI range.

e)	PCF selected by the AMF for the UE.

In the case of delegated discovery and selection in SCP, the SMF shall include the available factors in the first request.

Editor's note:	It still needs to be clarified which actual set of parameters the sentence above is referring to.

The selected PCF for serving the UE and the selected PCF for serving a PDU session of this UE may be the same or may be different.

In the following scenarios, information about the PCF instance that has been selected i.e. the PCF ID, may be forwarded to another NF. In case of NF service consumer performs discovery and selection, this NF may use this PCF instance. In case of delegated discovery and selection, this NF may include PCF ID in the request and the SCP may use this information to select the associated PCF instance.

When NF service consumer performs discovery and selection, the following applies:

-	During AMF relocation, the AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF by the AMF, and the AMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	The AMF may, based on operator policies, forward the selected PCF ID to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF for the AMF and the SMF. The SMF may decide based on operator policy either to use the same PCF or select a new PCF.

-	In the roaming case, the AMF may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above. The AMF sends the selected H-PCFID to the V-PCF during the policy association establishment procedure.

In the case of delegated discovery and selection in the SCP, the following applies:

-	During AMF relocation, the AMF may receive a PCF ID from the source AMF to enable the usage of the same PCF instance by the AMF. The AMF includes that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	The AMF may, based on operator policies, forward the selected PCF ID to the SMF during the PDU Session Establishment procedure to enable the usage of the same PCF instance for the AMF and the SMF. The SMF may include that information in the request. SCP may decide based on operator policy either to use the PCF instance or select another PCF instance that is not interchangeable to the used PCF instance.

-	In the roaming case, SCP may, based on operator policies, e.g. roaming agreement, select the H-PCF in addition to the V-PCF for a UE by performing the PCF discovery and selection as described above.

Editor's note:	If the sentence above applies, it is FFS how the AMF can send the selected H-PCF ID to the V-PCF during the policy association establishment procedure or it is FFS whether the sentence above means that the SCP needs to add an extra information H-PCF ID in the application part of the request; it is also FFS whether “SCP” means a SCP in VPLMN or SCP in HPLMN.

*** END CHANGES ***