3GPP TSG-SA WG2 Meeting #131 	S2-1901592

Santa Cruz - Tenerife, Spain, 25 Feb - 1 Mar

Source:	Ericsson

Title:	NF Set and NF Service Set way forward

Document for:	Agreement

Agenda Item: 	6.19

Work Item / Release:	5G_eSBA /Rel-16

Abstract of the contribution: This contribution proposes a way forward to the EN: “In distributed SCP deployments, which SCP instance does the NF Service Producer instance selection is FFS” in CR 0799 for 23.501

# 1	introduction

This DP discusses the EN in CR 23.501 CR 0799:
“In distributed SCP deployments, which SCP instance does the NF Service Producer instance selection is FFS”

And proposes to remove the EN.

# 2	Discussion

RFC 7230 defines 3 different types of intermediary nodes in a HTTP path: proxy, gateway, and tunnel.

A "tunnel" acts as a blind relay between two connections without changing the messages

A "proxy" is a message-forwarding agent that is selected by the client, usually via local configuration rules, to receive requests for some type(s) of absolute URI and attempt to satisfy those requests via translation through the HTTP interface. A proxy shall not change the URI.

A "gateway" (a.k.a. "reverse proxy") is an intermediary that acts as an origin server for the outbound connection but translates received requests and forwards them inbound to another server or servers.

RFC 7230 does not mandate any order of these intermediary nodes. RFC 7230 does not either mandate that only one of these types can be within a HTTP connection.

For eSBA the SCP can act as any of these, and in any order. It is only a gateway that can change the URI, and thus it is only a gateway that can do instance selection for eSBA.

Producer

consumer

For a connection where 2 SCPs are within the path between a consumer and producer we get the following possible roles for SCP1 and SCP2.

SCP2

SCP1

| Consumer | Tunnel | Tunnel | Producer |
| --- | --- | --- | --- |
| Consumer | Tunnel | Proxy | Producer |
| Consumer | Proxy | Tunnel | Producer |
| Consumer | Tunnel | Gateway | Producer |
| Consumer | Gateway | Tunnel | Producer |
| Consumer | Proxy | Proxy | Producer |
| Consumer | Proxy | Gateway | Producer |
| Consumer | Gateway | Proxy | Producer |
| Consumer | Gateway | Gateway | Producer |

Observation: For the case of 2 SCP in the path any of them can be a Gateway, and thus any of them can do instance selection.

Proposal: remove the editor’s note

The security solution for how these configurations may work needs to be addressed by SA3.

# 3	Proposal

Remove editor’s note:
“In distributed SCP deployments, which SCP instance does the NF Service Producer instance selection is FFS”,
in CR 0799 for 23.501