3GPP TSG-SA2 Meeting #131	S2-1902745

# 25 February – 1 March 2019, Santa Cruz - Tenerife, Spain

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0872 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of solution 7 in 23.725 as replication framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Impement the conclusion of the FS_URLLC for user plane replication |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the UE requests PDU Session for a certain DNN and S-NSSAI, appropriate UPF with access to RHF (out of scope) functionality is selected and RHF in the NG-RAN is also activated. The RHF replicates the packets over transport layer and there is no need for redundant GTP-U tunnel in this case. When will this be useful? - Allows 3GPP 5G system (deployed in MNO network) to inherently support reliability and redundancy framework without relying on the protocol supported in the DN. - It is beneficial for supporting Time sensitive communication natively within 5G System. - Ability to support redundancy framework for any application layer protocol thus allowing support for a flexible framework in the 3GPP system. Reference – solution #7, S2-1812284 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of FS_URLLC not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change ( all new text)* * * *

#### 5.x.2.3 Support for redundant transmission at transport layer

Redundant transmission can be supported within the 5G System can be supported within the 5GS without making any assumption on support for protocols such as IEEE FRER in the application layer (DN only) at the same time it can be supported without requiring redundant GTP-U tunnel over N3. The backhaul provides two transport paths between UPF and NG-RAN. The Redundant function (RHF, which is out of 3GPP scope) in NG-RAN and UPF make use of the independent paths at transport layer. For this approach, there is no need to enhance the “3GPP defined protocol stack” of UPF and NG-RAN to support data duplication as the transport layer protocol used between RFs in the NG-RAN and UPF is out of scope for 3GPP.

Figure 5.x.2.3-1: Support for Redundant Transmission with RF on the transport layer

This approach works as follows:

- 	UE establishes the PDU session for URLLC services. Based on DNN, S-NSSAI and other factors, SMF selects the UPF that supports RF for the PDU session. One N3 GTP-U tunnel is established between UPF and NG-RAN.

- 	For DL data transmission, UPF sends the DL packets on N3 GTP-U tunnel. RHF in UPF duplicate the DL data on the transport layer. RF in NG-RAN eliminates the received duplicated DL data and sends to NG-RAN.

- 	For UL data transmission, NG-RAN sends the received UL packets on N3 GTP-U tunnel, the RF in NG-RAN performs the redundant handling on the backhaul transport layer. The RF in UPF eliminates the received duplicated UL data and sends to UPF.

* * * * End of Change * * * *