3GPP TSG-SA WG2 Meeting #131	S2-1902773

# Santa Cruz - Tenerife, Spain,  25 Feb - 1 Mar

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0804 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eSBA communication schemas related to AMF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-02-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) |  |
| Reason for change: |  | FS_eSBA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new communication schemas related to AMF discovery and selection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No incorporation of agreed communication schemas related to FS_eSBA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 6.3.5	AMF discovery and selection

The AMF selection functionality is applicable to both 3GPP access and non-3GPP access. The AMF selection functionality can be supported by the 5G-AN (e.g. RAN, N3IWF) and is used to select an AMF for a given UE. For inter AMF - AMF communication, the AMF selection function selects an AMF for relocation or because the initially selected AMF was not an appropriate AMF to serve the UE (e.g. due to change of Allowed NSSAI). For inter communication between AMF and other CP NFs than AMF, e.g. SMF, the selection follows the principles in clause 6.3.1, with the addition that the AMF selection function will select an AMF from the AMF set when the original AMF serving a UE is unavailable.

5G-AN selects an AMF Set and an AMF from the AMF Set under the following circumstances:

1)	When the UE provides no 5G-S-TMSI nor the GUAMI to the 5G-AN.

2)	When the UE provides 5G-S-TMSI or GUAMI but the routing information (i.e. AMF identified based on AMF Set ID, AMF pointer) present in the 5G-S-TMSI or GUAMI is not sufficient and/or not usable (e.g. UE provides GUAMI with an AMF region ID from a different region).

3)	AMF has instructed AN that the AMF (identified by GUAMI(s)) is unavailable and no target AMF is identified and/or AN has detected that the AMF has failed.

In case ofNF Service Consumer based  discovery and selection, the CP NF selects an AMF from the AMF Set under the following circumstances:

-	When the AMF has instructed CP NF that a certain AMF identified by GUAMI(s) is unavailable and the CP NF was not notified of target AMF, and/or

-	CP NF has detected that the AMF has failed.

In case of delegated discovery and associated selection, the SCP selects an AMF from the AMF Set under the following circumstances:

-	The SCP gets an indication “select new AMF within SET” from the CP NF, and/or

-	SCP has detected that the AMF has failed.

The AMF selection functionality in the 5G-AN may consider the following factors for selecting the AMF Set:

-	AMF Region ID and AMF Set ID derived from GUAMI.

-	Requested NSSAI.

-	Local operator policies.

AMF selection functionality in the 5G-AN or CP NFs or SCP considers the following factors for selecting an AMF from AMF Set:

-	Availability of candidate AMFs.

-	Load balancing across candidate AMFs (e.g. considering weight factors of candidate AMFs in the AMF Set).

When the UE accesses the 5G-AN with a 5G-S-TMSI or GUAMI that identifies more than one AMF (as configured during N2 setup procedure), the 5G-AN selects the AMF considering the weight factors.

When 5G-S-TMSI or GUAMI provided by the UE to the 5G-AN contains an AMF Set ID that is usable, and the AMF identified by AMF pointer that is not usable (e.g. AN detects that the AMF has failed) or the corresponding AMF indicates it is unavailable (e.g. out of operation) then the 5G-AN uses the AMF Set ID for selecting another AMF from the AMF set considering the factors above.

The discovery and selection of AMF in the CP NFs or SCP follows the principle in clause 6.3.1

In case of NF Service Consumer based discovery and selection, the AMF or other CP NFs shall utilize the NRF to discover the AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AMF or other CP NFs. The NRF provides the IP address or the FQDN of AMF instance(s) or the Endpoint Address(es) of AMF service instance(s) to the AMF or other CP NFs. The AMF selection function in the AMF or other CP NFs selects an AMF instance based on the available AMF instances (obtained from the NRF or locally configured in the AMF or other CP NFs).

The Network Slicing aspects of the AMF selection are described in clause 5.15.5.2.1.

When NF Servcie Consumer performs discovery and selection  the following applies:

-	AMF selection functionality in AMF or other CP NFs use GUAMI, TAI to discover the AMF instance(s), the NRF provides the IP address, or the FQDN of the associated AMF instance(s) or the Endpoint Address(es) of the associated AMF service instance(s) if it is available. If the associated AMF is unavailable due to AMF planned removal, the backup AMF used for planned removal is provided by the NRF. If the associated AMF is unavailable due to AMF failure, the backup AMF used for failure is provided by the NRF. If no AMF instances related to the indicated GUAMI can be found or AMF Pointer value used by more than one AMF is found, a list of candidate AMF instances in the same AMF Set together with additional information (e.g. priority) is provided by the NRF. In addition, NRF may provide the TAI(s), if available. In this case, other CP NF can select any AMF instance from the list of candidate AMF instances.

-	AMF selection functionality in AMF use AMF Set ID to discover the AMF instance(s), the NRF provides a list of AMF/AMF service instances in the same AMF Set together with additional information (e.g. priority).

-	At intra-PLMN mobility, the AMF selection functionality in source AMF use source AMF Set ID, source AMF Region ID, and the target location information, S-NSSAI(s) of Allowed NSSAI to discover target AMF instance(s). The NRF provides the target AMF instance belonged to the target AMF set in target AMF Region which can be the mapping of the source AMF set in source AMF region.

-	At inter PLMN mobility, the source AMF selects an AMF in the target PLMN via the PLMN level NRF. After the Handover procedure the AMF may select a different AMF as specified in clause 4.2.2.2.3 in TS 23.502 [3].

When delegated discovery and associated selection is used, the following applies:

-	In case the CP NF includes GUAMI and TAI in the request the SCP selects will send the request to a GUAMI associated an AMF service instance that is associated with the GUAMI if it is available.

-	If a none of the AMF service instances associated with the GUAMI associated AMF are available due to AMF planned removal, an AMF service instance from the backup AMF used for planned removal is selected by the SCP.

-	If none of the AMF service instances associated with the GUAMI associated AMF are available due to AMF failure, an AMF service instance from the backup AMF used for failure is selected by the SCP. If no AMF service instances related to the indicated GUAMI can be found or AMF Pointer value used by more than one AMF is found, an AMF instance in the same AMF Set is selected

-	In case the CP NF includes AMF Set ID in the request, the SCP selects AMF/AMF service instances in the provided AMF Set.

-	At intra-PLMN mobility, if a target AMF instance needs to be selected, the AMF provides the source AMF Set ID, source AMF Region ID, and the target location information, S-NSSAI(s) of Allowed NSSAI in the request, optionally NRF to use. The SCP will select a target AMF instance belonged to the target AMF set in target AMF Region which can be the mapping of the source AMF set in source AMF region.

-	At inter PLMN mobility, the source AMF selects indicates “roaming” to the SCP. The SCP interacts with the NRF in source PLMN so that the NRF in source PLMN can discover an AMF in the target PLMN via a provided target PLMN NRF. After the Handover procedure the AMF may send a request with the indication “AMF relocation” to trigger SCP to select a different AMF as specified in clause 4.2.2.2.3 in TS 23.502 [3].

*** END CHANGES ***