SA WG2 Meeting #133	S2-1905003

May 13 – 14, 2019, Reno, Nevada, US	(revision of S2-19xxxx)

Source:	Ericsson

Title:	Deregistration procedure for FN-RG

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5WWC/Rel-16

Abstract of the contribution: This contribution proposes a FN-RG deregistration procedure.

# Introduction

This contribution proposes a FN-RG deregistration procedure to TS 23.316. It uses solution #23 in 23.716 as baseline as agreed during the study phase.

# Proposal

It is proposed to update TS 23.316 as follows:

**** All text is new ****

#### 7.2.1.X	FN-RG Deregistration via W-5GAN

The deregistration procedure for the FN-RG is similar to that of 5G-RG described in clause 7.2.1.2 but with the following differences:

- The 5G-RG is replaced with a FN-RG.

- In step 1a and 1b, the W-AGF sends and receives NAS deregistration request/accept messages on behalf of FN-RG.

- UE-initiated deregistration procedure can be initiated by the W-AGF, when it has lost connectivity to the FN-RG.

- For both UE/Network-initiated deregistration procedures, the W-AGF may initiate the release of the signalling connection between the FN-RG and W-AGF based on legacy protocols.

NOTE: The message exchanges between the FN-RG and W-AGF are based on legacy protocols in the wireline access network as described in clause 6.2.2.

**** End of new text ****