SA WG2 Meeting S2#133	S2-1904869

13 - 17 May, 2019, Reno, Nevada, USA	(revision of S2-1904789)

SA WG2 Meeting #S2-132	S2-1904789

08 - 12 April, 2019, Xi'an China

3GPP TSG RAN WG1 Meeting #96b	R1-1905904

Xi’an, China, 8th – 12th April, 2019

Title:	Response to LS on Combinations of Uu QoS characteristics values for V2X services

Response to:	S2-1813386

Release:	Rel-16

Work Item:	NR_L1enh_URLLC

Source:	RAN WG1

To:	SA WG2, RAN WG3

Cc:	RAN WG2

Contact Person:

Name:	                 Yongxia Lyu

E-mail Address:	  lvyongxia@huawei.com

1. Overall Description:

RAN1 has received the LS S2-1813386 from SA2. There are two new combinations of QoS characteristics values in the LS:

- Combination 1) PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes (e.g., for Collision Avoidance and Platooning with high LoA), and

- Combination 2) PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes (e.g., for Emergency Trajectory Alignment and Sensors information Sharing with high LoA).

SA2 asked the following question to RAN1.

SA WG2 would like to ask RAN WG2 and RAN WG1 whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values indicated above are feasible or not.

In order to get air interface latency budget for feasibility evaluation, RAN1 needs SA2 to provide information about the delay between UPF and 5G-AN for two new combinations of QoS characteristics values and needs RAN3 to provide information about the network interface latency for the two new combinations of QoS characteristics values, for the example services of eV2X.

Based on related evaluation assuming air interface latency as 3ms, RAN1 expects Combination 1) to be feasible since it is less challenging than remote driving requirements in Table 5.5-1 of TS 22.186 which were evaluated as feasible.

2. Actions:

To SA WG2

RAN1 respectfully asks SA2 to provide information about the delay between UPF and 5G-AN in order to get air interface latency budget for RAN1 feasibility evaluation for Combination 2).

RAN1 expects Combination 1) assuming air interface latency as 3ms to be feasible. RAN1 respectfully asks SA2 to take this into consideration.

To RAN WG3

RAN1 respectfully asks RAN3 to provide information about the network interface latency for Combination 2) of QoS characteristics values, considering the example eV2X services, in order to get air interface latency budget for feasibility evaluation.

4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #97		13 – 17 May 2019, Reno, US

TSG RAN WG1 Meeting #98		26 – 30 August 2019, Prague, CZ