3GPP TSG-SA2 Meeting #133	S2-1905313

# Reno, Nevada (USA), 13 - 17 May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1371 | rev | - | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for the related CAG identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The related CAG identifier is not clear in 5.30.3.4. During connected mode mobility procedures, the source NG-RAN can receive the Mobility Restrictions from the AMF include the UE’s allowed CAG list. Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and any CAG Identifier supporting by the CAG cell is not part of the UE's Allowed CAG list In Update of Mobility Restrictions, if UE is camping in a CAG cell, only when the UDM remove the CAG identifiers supporting both by the current CAG cell and UE’s allowed CAG list, the AMF shall release the NAS signalling connection for the UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the clear meaning of the related CAG identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear for the meaning of the related CAG identifier and hard for implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

#### 5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell;

NOTE:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and any CAG Identifier supporting by the CAG cell  is not part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the CAG Identifier(s) both supporting by the CAG cell and the Mobility Restrictions  have been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

<<< End of changes >>>