3GPP TSG-SA WG2 Meeting #133	S2-1906563

# RENO, NEVADA, USA, MAY 13 – 17, 2019	(revision of S2-1905686)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1430 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | implementation of 5GLAN related interfaces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | N19 interface is used as a shared User Plane tunnel connecting PSA UPFs to support N19-based forwarding of a 5G VN group. The SMF is response for handling the N19 tunnels. For the Source interface of PDR or the Destination interface of FAR, the value “5G LAN internal” and “5G LAN Nx” can be used to detect and forward the traffic of a 5G VN group. For “5G LAN internal” of a PDR, the UPF needs to recognize this interface for the traffic that is forwarded internally before matching the traffic with this kind of PDR. The following ways can be considered: - Use a method similar with traffic steering policy in FAR, avoiding recognizing the “5G LAN internal”. [A1] - Use the tunnel information to distinguish “core side” / “access side”, “5G LAN internal”. [A2] - “5G LAN internal” together with Network Instance set with 5G VN group is used to indicate the secondary filtering by the group relevant PDRs inside the UPF, no need for interface recognization. [A3] For “5G LAN N19” of a PDR, the UPF needs to recognize this interface for the traffic that is forwarded from the other UPFs before matching the traffic with this kind of PDR. The following ways can be considered: - Deploy physical links for N19 interface, independently with physical links for N9 interface. [B1] - N19 interface and N9 interface share the physical links, “core side” value is used for N19 interface and N9 interface and the tunnel information is used by the UPF to distinguish the N19 interface and N9 interface. [B2] It is proposed to go with the following: - Using GTP-U protocol for N19 interface to implement the tunnel. - Using [A3] to avoid the interface recognition for traffic forwarding within a UPF. - Use [B2] for traffic forwarding across UPFs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add user plane protocol stack for N19-based forwarding - update to avoid the internal interface recognition - clarification on N19 interface recognition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear implementation of 5G LAN N19 and Internal interface |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29.4, 5.8.2.5, 5.8.2.11.3, 5.8.2.11.6, 4.2.7, 8.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

NOTE 1:	The traffic forwarding details of N6 between a UPF acting as an uplink classifier and a local data network are not specified in this Release of the specification.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2: in some cases, a couple of NFs may need to be associated with each other to serve a UE.

In addition to the reference points above, there are interfaces/reference point(s) between SMF and the CHF. The reference point(s) are not depicted in the architecture illustrations in this specification.

NOTE 3:	The functionality of these interface/reference points are defined in TS 32.240 [41].

N32:	Reference point between SEPP in the visited network and the SEPP in the home network.

NOTE 4:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs.

N40:	Reference point between SMF and the CHF.

NOTE 5:	The reference points from N40 up to and including N49 are reserved for allocation and definition in TS 23.503 [45].

N50:	Reference point between AMF and the CBCF.

NOTE 6:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

**** Next Change *****

5.8.2.5	Control of User Plane Forwarding

The SMF controls user-plane packet forwarding for traffic detected by a PDR by providing a FAR with instructions to the UPF, including:

-	Forwarding operation information;

-	Forwarding target information.

The details of the forwarding target and operation will depend on the scenario and is described below. The following forwarding functionality is required by the UPF:

-	Apply N3 /N9 tunnel related handling, i.e. encapsulation.

-	Forward the traffic to/from the SMF, e.g. as described in Table 5.8.2.5-1.

-	Forward the SM PDU DN Request Container from SMF to DN-AAA server

-	Forward the traffic according to locally configured policy for traffic steering.

-	Forward the traffic according to N4 rules of a 5G VN group for 5G VN group communication.

Data forwarding between the SMF and UPF is transmitted on the user plane tunnel established on N4 interface, defined in TS 29.244 [65].

Scenarios for data forwarding between the SMF and UPF are defined as below:

Table 5.8.2.5-1: Scenarios for data forwarding between the SMF and UPF

|  | Scenario description | Data forwarding direction |
| --- | --- | --- |
| 1 | Forwarding of user-plane packets between the UE and the SMF e.g. DHCP signalling. | UPF to SMF SMF to UPF |
| 2 | Forwarding of packets between the SMF and the external DN e.g. with DN-AAA server | UPF to SMF SMF to UPF |
| 3 | Forwarding of packets subject to buffering in the SMF. | UPF to SMF SMF to UPF |
| 4 | Forwarding of End Marker Packets constructed by the SMF to a downstream node. | SMF to UPF |
| 5 | Forwarding of user data using Control Plane CIoT 5GS Optimisation | UPF to SMF SMF to UPF |

When configuring an UPF acting as PSA for an Ethernet PDU Session Type, the SMF may instruct the UPF to route the DL traffic based on the MAC address(es) used by the UE for the UL traffic.

In that case:

-	broadcast and/or multicast DL traffic on the N6 Network Instance targets every DL PDU Session (corresponding to any N4 Session) associated with this Network Instance.

-	for uplink broadcast and/or multicast traffic received by the UPF over a PDU session on a N3/N9 interface, the UPF should forward the traffic to the N6 interface and downlink to every PDU session (except the one of the incoming traffic) associated with the same N6 Network Instance.

For ARP/IPv6 Neighbour Solicitation traffic, a SMF's request to respond to ARP/IPv6 Neighbour Solicitation based on local cache information or to redirect such traffic from the UPF to the SMF overrules the traffic forwarding rules described above.

NOTE:	Local policies in UPF associated with the Network Instance can prevent local traffic switching in the UPF between PDU Sessions either for unicast traffic only or for any traffic. In the case where UPF policies prevent local traffic switching for any traffic (thus for broadcast/multicast traffic) some mechanism such as responding to ARP/ND based on local cache information or local multicast group handling is needed to ensure that upper layer protocol can run on the Ethernet PDU sessions.

If allowed by local UPF (acting as PSA) policies, for Ethernet traffic with unknown unicast destination MAC addresses, the UPF should forward the traffic in the same manner as described above for the broadcast and/or multicast traffic handling.

The SMF may ask to get notified with the source MAC addresses used by the UE and provide UPF with corresponding forwarding rules related with these MAC addresses.

**** Next Change ****

5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR |  |
| Rule ID |  | Unique identifier to identify this rule |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied |  |
| Packet | Source interface (NOTE 4) | Contains the values "access side", "core side", "SMF", "N6-LAN" , "5G LAN internal" | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3) | packet filter set, application ID, Ethernet PDU Session |
| information | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application ID |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN or the local DN), or from "5G LAN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14 | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: The “core side” value is used for N9 and N19 interfaces, the different GTP-U headers are used by SMF to instruct the UPF to detect the pactets from “N9” or “N19”. |  |  |  |

**** Next Change *****

5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. |  |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included. |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). |  |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G LAN internal" or "5G LAN Nx". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN or the local DN), to 5G LAN internal (i.e. local switch), or to 5G LAN Nx (i.e. Nx interface). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP+QFI, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI). Any extension header stored for this packet shall be added. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. |  |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and Nx forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 in TS 29.244 [65]. NOTE 7: The use of "5G LAN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. |  |  |

**** Next Change *****

5.29.4	User Plane handling

User Plane management as defined for 5GS in clause 5.8 is applicable to 5G-LAN-type services with the following clarifications:

-	There are three types of traffic forwarding methods for 5G-LAN communication:

-	N6-based, where the UL/DL traffic for the 5G-LAN communication is forwarded to/from the DN;

-	N19-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via Nx. N19 is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G LAN group.

-	The SMF handles the user plane paths of the 5GLAN group, including:

-	Establishing Nx tunnels between PSA UPFs to support Nx-based traffic forwarding.

-	For Ethernet PDU Session, the SMF may instruct the UPF(s) classify frames based on VLAN tags, and to add and remove VLAN tags, on frames received and sent on N6, as described in clause 5.6.10.2.

NOTE:	For handling VLAN tags to traffic on N6, TSP ID could also be used as described in clause 6.2.2.6 of TS 23.503 [45].

Further description on User Plane management for 5G LAN groups is available in clause 5.8.2.13.

Editor's note:	Whether PCF can indicate to SMF the allowed traffic forwarding methods for a 5G-LAN group is FFS.

**** Next Change *****

8.3	User Plane Protocol Stacks

8.3.X	User Plane for N19-based forwarding of a 5G VN group

Figure 8.3.X-1: User Plane for N19-based forwarding

Details about the PDU Layer, PDU Session User Plane Protocol Stack are included in clause 8.3.1 and clause 8.3.2. The N19 is based on a shared User Plane tunnel connecting two PSA UPFs of a single 5G VN group.

**** End of Changes *****