3GPP TSG-SA2 Meeting #134	S2-1908036

# Sapporo, Japan, 24th June 2019 - 28th June 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0015 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Analytics information provided by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 7.1-2 in TS 23.288 is not completed, now it only includes the analytics on slice load level information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.1-2 is filled up with other analytics, which have been proposed in TS 23.288 in clause 6. The related Editor Note below the table is deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS 23.288 is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF |

The following table shows the analytics information provided by NWDAF service:

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description | Operation semantics |
| --- | --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Requested Analytics data, including load level information of Network Slice instance(s). | Subscribe/Notify, Request/Response |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience may be provided in detail, individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice. | Subscribe/Notify, Request/Response |
| NF Load information | Analytics ID: NF load information | Load statistics information for specific NF(s). | Subscribe/Notify, Request/Response |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. | Subscribe/Notify, Request/Response |
| UE mobility information | Analytics ID: UE Mobility | Statistics or prediction on UE mobility | Subscribe/Notify, Request/Response |
| UE Communication information | Analytics ID: UE Communication | Statistics or prediction on UE communication | Subscribe/Notify, Request/Response |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. | Subscribe/Notify, Request/Response |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | The exception with an appropriate exception ID | Subscribe/Notify, Request/Response |
| User Data Congestion information | Analytics ID: User Data Congestion | The user data congestion analytics for transfer over the user plane, for transfer over the control plane, or for both | Subscribe/Notify, Request/Response |
| QoS Sustainability Analytics | Analytics ID: QoS Sustainability Analytics | The information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed | Subscribe/Notify, Request/Response |

**** Next Change ****