SA WG2 Meeting #S2-135	S2-1909969

14 - 18 October, 2019, Split Croatia

3GPP TSG-CT WG6 Meeting #96	C6-190351

# Portoroz, Slovenia; 08th – 11th October 2019

# Title:	Reply to LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update

# Response to:	LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update (C6-190341/C4-193790)

# Release:	Rel-16

# Work Item:

Source:	CT6

To:	CT4, SA3, SA2

Cc:	CT1

Contact Person:

Name:	Ly Thanh Phan

Tel. Number:

E-mail Address:	ly-thanh.phan@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT6 thanks CT4 for their LS on 5GS Enhanced support of OTA mechanism for UICC configuration parameter update and the attached work item description.

CT6 has discussed the provided WID on “5GS Enhanced support of OTA mechanism for UICC configuration parameter update” and would like to provide the following feedback to CT4:

- Until now, for security reasons only the OTA USIM management platform is allowed to provide updates to the USIM. Due to the key role performed by the OTA Platform in the security architecture of the MNO, it is important that considerations with regard to the security are taken into account in the design of the API so only legitimate and authorized network functions get access to the OTAF through the API.

- It is CT6 understanding that the work item is covering USIM parameter updates only via NAS transport protocol.

- Our understanding is that in the future any network functions would be able to access the API to the OTAF and therefore CT6 would like to be informed of new parameter updates as potential technical aspects would need be assessed and evaluated for any impact on the secure packet.

- In order to reduce the impact on existing OTA Platforms and USIMs, the secure packet defined in 3GPP TS 31.115 shall be used with no (or minor) changes to avoid backward compatibility issues. Our understanding is that the secure packet structure would not be affected.

2. Actions:

To CT4 group

ACTION: Take the above considerations into account

3. Date of Next CT6 Meetings:

3GPP TSG CT6#97	12th – 15th November 2019	Reno, US

3GPP TSG CT6#98	25th – 28th February 2020	Sophia Antipolis, FR