3GPP TSG SA2 Meeting #136AH	S2-2000373

Incheon, South Korea, January 13 – 17, 2020

Source:	Huawei, Hisilicon

Title:	UE DNS cache flush

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution discuss application discovery.

- Discussion

If UL-CL is used to access the Local network, the UE is not aware of the UL-CL insertion/removal/change. An application server may be deployed in multiple edge networks, and the UE may already have cached the related DNS record locally. When UE moves, the application server located at old edge network may be not optimized for UE access. For example if the same application server located at the new edge network, it is preferred that the application server in the new edge network is selected.

However, if the TTL of DNS record is not expired, the UE will still connect to the Application Server located in old edge network, which may be not optimization. This may cause unnecessary delay or service interruption. To solve the issue, the following solutions are possible:

Alternative 1: The DNS Server set the TTL of a DNS record to 0.

Since the TTL is 0, the UE will not cache any DNS query result. UE will query DNS every time when the UE needs to connect to application server.

This alternative causes frequent DNS query. In the most cases the UE stays within the same coverage of an edge computing network. Hence this cause too much unnecessary DNS query.

Alternative 2: CN flushes UE’s DNS cache

In this alternative, when DNS cache information need be cleaned, e.g. the UL-CL is changed or removed, the SMF sends a DNS flush indication to UE. The DNS flush indication tells the UE to remove all the cached DNS records. After the UE removes all the cached DNS records, the UE will trigger a new DNS query if required. Thus the application server in the new edge network can be selected and the path is optimized.

- Proposal

It is proposed to have the following text in TR 23.748:

* * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |  |
| 6.x UE DNS cache flush | x |  |  |  |  |

* * * * 2nd change * * * *

6.X	Solution #X: UE DNS cache flush

6.X.1	Description

If ULCL is used to access the edge network, the UE is unaware of the ULCL insertion/removal/change. An application may be deployed in multiple edge networks, and the UE may already have connected to an application server in one edge network, and cached the related DNS record locally. When UE moves, the application located at the old edge network may be not optimized for UE to visit. In this case, if the new edge network has the same application server, it is preferred that the new application server in the new edge network is selected by the UE.

However, since the UE has cached the DNS record, the UE will still connect to the application server located in old edge network. To solve the issue, the following solution is proposed:

When DNAI changes, i.e. the ULCL is changed or removed, the SMF sends a DNS flush indication to UE. This DNS flush indication tells UE to remove all the cached DNS records. After the UE removes all the cached DNS records, the UE will trigger a new DNS query, and the application server in the new edge network can then be selected and the path is optimized.

6.X.2	Procedures

- The UE connected to the application server located in an edge computing network accessed via ULCL1. So the original data path from UE to application server is via ULCL1. When the UE moves, and SMF decides to remove ULCL1 which corresponding to DNAI1, and inserts ULCL2 corresponding to DNAI2.

- The SMF sends DNS flush indication to UE. The indication may be associated with the IP segment or subnet info.

If the IP segment or subnet info is not included, it means all the DNS cache information is cleaned.

- The UE removes the DNS records stored locally. If the IP segment or subnet info is included in the DNS flush indication, the UE only removes the DNS records corresponding to the IP segment or subnet.

4~6. When the UE wants to connect to the application server that was belonged to the DNS information which is required to be cleaned, the UE send a DNS query.

Then based on the solution on how to find the edge application server, the best application server that can serve the UE, e.g. the application server located in the edge computing network corresponding to ULCL2, is selected.

The UE is connected to the new application server located in the edge computing network corresponding ULCL2.

6.X.3	Impacts on Existing Nodes and Functionality

- SMF: send DNS Flush indication to UE when DNAI changes.

- UE: remove corresponding cached DNS records based on DNS Flush indication received from network.

- * * * * End of changes * * * *