SA WG2 Meeting #136AH	S2-2000792

Incheon, Korean, January 12th, 2020	revision of S2-20xxxx

Source:	China Mobile, Huawei, AsiaInfo

Title:	key issue: NWDAF-assisted charging information generation

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA / Rel-17

Abstract of the contribution: This contribution proposes a new key issue on NWDAF-assisted charging information generation to support differentiated pricing option.

Discussion

This new key issue is to address the use case #x NWDAF-assisted charging information generation to support differentiated pricing option described in TR 23.700-91.

Proposal

Proposal: It is proposed to add a new key issue on NWDAF-assisted charging information generation to support differentiated pricing option.

The following changes are proposed for FS_eNA TR 23.700-91.

* * * * Start of Changes * * * *

5.2.X	Key issue #X: < NWDAF-assisted charging information generation >

5.2.X.1	Description

As described in SA5 TS 28.201, slice load level analytics could be consumed by CHF for the network slice charging purpose. The architectural between NWDAF and CHF/BSS system is depicted in the following figure.

Figure 1: The relationship between NWDAF and CHF

In addition to per slice charging based on analytics provided by NWDAF, 5G brings more values on for the diverse services. Correspondingly, 5G may also need to support per subscriber charging information based on analytics provided by NWDAF for VR/AR, gaming, autonomous cars, industry automation and e-health. e.g., charging based on service MOS, which is the combination of bandwidth, latency and packet loss, RSRP, etc.

Moreover, for the VIP users, NWDAF should be able to provide real-time or near real-time service MOS for 5G to generate the charging information and perform the corresponding actions (e.g. may provide timely apology information to the VIP user for poor Service Experience, which shall be discussed and decided by SA5 charging WG) in time to avoid or reduce complaint from the VIP users as much as possible.

Therefore, per subscriber real time/near real-time communication between NWDAF and CHF/BSS needs to be supported.

This key issue describes analytics information that may be provided by NWDAF to assisted charging information generation to support differentiated pricing option by CHF/BSS system.

Solutions to this key issue are expected to address the following aspect:

- Which kind of data analytics should the NWDAF provide to the service consumer, e.g., Bill system?

- What kind of information is made available to the NWDAF for generating data analytics and from which NFs?

* * * * End of Changes * * * *