3GPP TSG-WG SA2 Meeting #136 	S2-1912006

# November 14 – 18, 2019, Reno USA	(revision of S2-1910015,11542,11984)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1785 | rev | 6 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to handling of Alternative QoS Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Volkswagen AG, Fraunhofer HHI, Vodafone, Ericsson, Tencent, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2019-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#134 the concept of Alternative QoS Profile was introduced in TS 23.501. Some aspects were not handled due to the need of RAN2 and RAN3 feedback which was then provided in R2-1911788 and R3-194795, respectively. This CR addresses the issues left open in SA2#134. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections are applied: - Alternative QoS Profiles are provided as prioritized list to the NG-RAN to allow the NG-RAN to select the profile that is most beneficial for the application. - There is no change of QoS Profile triggered when the NG-RAN decides to select one of the Alternative QoS Profiles. - If none of the Alternative QoS Profiles can be guaranteed, the NG-RAN shall indicate the values for the QoS parameters that are currently fulfilled for this GBR QoS Flow. - Described the behavior in case of improvement of QoS conditions. - Introduced usage of existing NAS signaling to notify change of QoS conditions to the UE. This procedure is intended to not impact the UE’s 3GPP protocol layers. Rev 6: in 5.7.2.4, - Moved generic text to the end of clause -If there is no match, the NG-RAN shall indicate the release of the NG-RAN resources for this GBR QoS Flow. - Clarified the magnitued of the hysteresis applied in upgrade - if the currently fulfilled values of the GFBR, the PDB and the PER match the QoS profile or a different alternative QoS profile as indicated in the last notification, the NG-RAN shall send a further notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.2a, 5.7.1.5, 5.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7.1.2a	Alternative QoS Profile

The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN.

An Alternative QoS Profile represents a combination of QoS parameters to which the application traffic is able to adapt and has the same format as the QoS profile for that QoS Flow.

When the NG-RAN sends a notification to the SMF that the QoS profile is not fulfilled, the NG-RAN shall, if the currently fulfilled values match an Alternative QoS Profile, include also the reference to the Alternative QoS Profile to indicate the QoS that the NG-RAN  currently fulfils (see clause 5.7.2.4).

# * * * * Second change * * * *

#### 5.7.1.5	QoS Flow mapping

The SMF performs the binding of SDFs to QoS Flows based on the QoS and service requirements (as defined in TS 23.503 [45]). The SMF assigns the QFI for a new QoS Flow and derives its QoS profile, corresponding UPF instructions and QoS Rule(s) from the PCC rules and other information provided by the PCF.

When applicable, the SMF provides the following information to the (R)AN:

-	QFI;

-	QoS profile as described in clause 5.7.1.2.

-	optionally, Alternative QoS Profile(s) as described in clause 5.7.1.2a;

The SMF provides the following information to the UPF enabling classification, bandwidth enforcement and marking of User Plane traffic (the details are described in clause 5.8):

-	a DL PDR containing the DL part of the SDF template and an UL PDR containing the UL part of the SDF template;

-	the PDR precedence value (see clause 5.7.1.9) for both PDRs is set to the precedence value of the PCC rule;

-	QoS related information (e.g. MBR for an SDF, GFBR and MFBR for a GBR QoS Flow) as described in clause 5.8.2;

-	the corresponding packet marking information (e.g. the QFI, the transport level packet marking value (e.g. the DSCP value of the outer IP header);

-	for the DL PDR, optionally, the Reflective QoS Indication.

For each SDF, when applicable, the SMF generates an explicitly signalled QoS rule (see clause 5.7.1.4) according to the following principles and provides it to the UE together with an add operation:

-	A unique (for the PDU Session) QoS rule identifier is assigned;

-	The QFI in the QoS rule is set to the QFI of the QoS Flow to which the PCC rule is bound;

-	The Packet Filter Set of the QoS rule is generated from the UL SDF filters and optionally the DL SDF filters of the PCC rule (but only from those SDF filters that have an indication for being signaled to the UE, as defined in TS 23.503 [45]);

-	The QoS rule precedence value is set to the precedence value of the PCC rule for which the QoS rule is generated;

-	for a dynamically assigned QFI, the QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR, Averaging Window, see TS 24.501 [47]) are signalled to UE in addition to the QoS rule(s) associated to the QoS Flow. The QoS Flow level QoS parameters (i.e. GFBR and MFBR) of an existing QoS Flow may be updated based on the MBR and GBR information received in the PCC rule (MBR and GBR per SDF are not provided to UE over N1) or upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed (see clause 5.7.2.4).

Changes in the binding of SDFs to QoS Flows as well as changes in the PCC rules or other information provided by the PCF can require QoS Flow changes which the SMF has to provide to (R)AN, UPF and/or UE. In case of changes in the explicitly signalled QoS rules associated to a QoS Flow, the SMF provides the explicitly signalled QoS rules and their operation (i.e. add/modify/delete) to the UE.

NOTE 1:	The SMF cannot provide, update or remove pre-configured QoS rules or UE derived QoS rules.

The principle for classification and marking of User Plane traffic and mapping of QoS Flows to AN resources is illustrated in Figure 5.7.1.5-1.

Figure 5.7.1.5-1: The principle for classification and User Plane marking for QoS Flows and mapping to AN Resources

In DL, incoming data packets are classified by the UPF based on the Packet Filter Sets of the DL PDRs in the order of their precedence (without initiating additional N4 signalling). The UPF conveys the classification of the User Plane traffic belonging to a QoS Flow through an N3 (and N9) User Plane marking using a QFI. The AN binds QoS Flows to AN resources (i.e. Data Radio Bearers of in the case of 3GPP RAN). There is no strict 1:1 relation between QoS Flows and AN resources. It is up to the AN to establish the necessary AN resources that QoS Flows can be mapped to, and to release them. The AN shall indicate to the SMF when the AN resources onto which a QoS Flow is mapped are released.

If no matching DL PDR is found, the UPF shall discard the DL data packet.

In UL:

-	For a PDU Session of Type IP or Ethernet, the UE evaluates UL packets against the UL Packet Filters in the Packet Filter Set in the QoS rules based on the precedence value of QoS rules in increasing order until a matching QoS rule (i.e. whose Packet Filter matches the UL packet) is found.

-	If no matching QoS rule is found, the UE shall discard the UL data packet.

-	For a PDU Session of Type Unstructured, the default QoS rule does not contain a Packet Filter Set and allows all UL packets.

NOTE 2:	Only the default QoS rule exist for a PDU Session of Type Unstructured.

The UE uses the QFI in the corresponding matching QoS rule to bind the UL packet to a QoS Flow. The UE then binds QoS Flows to AN resources.

# * * * * Third change * * * *

#### 5.7.2.4	Notification control

The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled, NG-RAN shall send a notification towards SMF that the GFBR can no longer be guaranteed. Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not delivering the requested GFBR for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to fulfil the GFBR, the PDB and the PER of the QoS profile again.

NOTE 1:	NG-RAN can decide that the GFBR can no longer be guaranteed based on, e.g. measurements like queuing delay or system load.

If the NG-RAN has received the list of Alternative QoS Profile(s) for this QoS Flow and supports the Alternative QoS Profile handling, the NG-RAN shall apply the following:

-	Before sending a notification that the GFBR can no longer be guaranteed towards the SMF, the NG-RAN shall check whether the values of the GFBR, the PDB and the PER parameters that the NG-RAN  currently fulfils match any of the Alternative QoS Profile(s) in the indicated priority order. If there is a match, the NG-RAN shall indicate the reference to the matching Alternative QoS Profile with the highest priority together with the notification to the SMF.

-	If there is no match, the NG-RAN shall indicate the release of the NG-RAN resources for this GBR QoS Flow.

If the RAN has notified the SMF that the QoS Profile can no longer be guaranteed and the NG-RAN determines that the currently fulfilled values of the GFBR, the PDB or  the PER are different for the QoS Profile and  are different from the Alternative QoS Profile indicated in the last notification, the NG-RAN shall apply the principles described above.

NOTE 2:		In order to avoid a too frequent signalling to the SMF, it is assumed that NG-RAN implementation can apply hysteresis of a configurable time interval before notifying the SMF that the currently fulfilled values match a different Alternative QoS Profile of higher priority.

Upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed or can be fulfilled again, the SMF may forward the notification to the PCF, see TS 23.503 [45].

When the NG-RAN determines that the GFBR, the PDB and the PER of the QoS profile can be fulfilled again for a QoS Flow (for which a notification that the GFBR can no longer be guaranteed has been sent), the NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed whenever necessary.

NOTE 3:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the GFBR can be guaranteed again and therefore a frequent signalling of GFBR can be guaranteed again followed by GFBR can no longer be guaranteed is not expected.

NOTE 4:	If the QoS Flow is modified, the NG-RAN restarts the check whether the GFBR can no longer be guaranteed according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification that the GFBR for a QoS Flow can no longer be guaranteed. The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. The accepted QoS Flows are included in the N2 Path Switch Request or N2 Handover Request Acknowledge message from the NG-RAN to the AMF. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that GFBR can be guaranteed again for this QoS Flow. After the handover is successfully completed, the Target NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed for such a QoS Flow whenever necessary. If the SMF has previously notified the PCF that the GFBR can no longer be guaranteed and the SMF does not receive an explicit notification that the GFBR can no longer be guaranteed for that QoS Flow from the Target NG-RAN within a configured time, the SMF shall notify the PCF that the GFBR can be guaranteed again.

If the NG-RAN supports the Alternative QoS Profile handling and the list of Alternative QoS Profile(s) for the QoS Flow has been provided by the SMF, the SMF uses NAS signalling (that is sent transparently through the RAN) to inform the UE about changes in the QoS parameters (i.e., 5QI, GFBR, MFBR) that the NG-RAN is currently fulfilling for the QoS Flow after receiving a notification from the NG-RAN.

# * * * * End of changes * * * *