SA WG2 Meeting #136 	S2-1911468

18 – 22 Nov 2019, Reno, NV, USA	 (revision of S2-191xxxx)

Source:	CATT

Title:	Update to KI #1

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: update the key issue #1

Discussion

In the KI#1, the NWDAF functionalities may be split and placed in different NFs. Different NWDAF functionalities have different responsibility, e.g. data collection, analytic information provision, etc.. Thus it is important to study how to split such NWDAF functionalities and how such cooperate to provide data analytics. For example, data collection and analytic information provision functionalities may not be split for distributed NWDAFs in a PLMN, but may be split for a central NWDAF.

With different functionalities split, there may be different working logic among the splitting NWDAF functionalities, so this KI should also study how the splitting NWDAF functionalities work together.

Proposal

It is proposed to update the KI#1 in the TR 23.700-91.

FIRST CHANGE

5.2.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

5.2.1.1	Description

Rel-16 introduced possibility of networks implementing multiple NWDAF instances for different Analytics IDs. NWDAF currently is considered to be a monolithic entity and each NWDAF instance may need to reflect such monolithic architecture consisting of all of the logical functions. The NWDAF NF provides the following functionality according to TS 23.501 [2].

-	Support data collection from NFs and AFs;

-	Support data collection from OAM;

-	NWDAF service registration and metadata exposure to NFs/AFs;

-	Support analytics information provisioning to NFs, AF.

The proposal as per this Key Issue is to:

-	Study whether NWDAF functional split is required, and identify the NWDAF functionality in TS 23.288 [5] and TS 23.501 [2] that can be separated or placed in a different NF/NF Service.

-	Study how to split the NWDAF functionality is different network deployment scenarios (e.g. central/distributed NWDAF in same/different PLMN), and how the splitting NWDAF functionalities work together.

-	Study whether standard based services need to be defined as a result of splitting NWDAF functionality.

End of CHANGE